/*

Cwriter is a graphics list processor.

List format:

<forward pointer	>

< object number		>

< x position		>

< y position		>

< flag word		>

< object pointer	>

< x increment for movers>

< y increment for movers>

< x accumulator		>

< y accumulator		>

< list pointer 0 link	>

< count 0		>

< spare word #D		>

< spare word #E		>

< spare word #F		>

where -

all the above items are 16 bits.

Forward pointer is pointer to the next list block or

= NULL if the last block.

X position is from 0 to 255

Y position if from 0 to 192  both referenced to the top

left edge of the enclosing image box

Flag word maintains the state of an object (ie,blanked).

Objects in this list (as defined above) refer to two (or more)

lists which contain the same object information for a particular

display page. The display page lists have end at the entry

< Pointer to object table>

after the flag word which points to the actual object definition.

(see writeobj for that data structure). For a two page system,

it looks like

Master List

/     \

/	\

/         \

Page 0 list	Page 1 list

All interactions are with the Master list. Cwriter

takes the master list information and a start of page list

pointer and updates the page list to match the Master list.

Cwriter also outputs the updated information to the page screen.

It would be far simpler to merely erase the entire page

and rewrite it, but the time to do this function would be excessive.

Games and other graphic applications often have only a few

'movers'  with most objects 'static'. Only the 'movers' and

the objects they affect need be updated.

Cwriter takes 3 passes over the Master list and the

current page list.

Pass 0: Flag any objects in the display list which have

moved in the master list. Update the x and y. Also, if

an object has become unblanked, flag it as a write and update

its x and y position in the display list. If an object has

become erased, flag the object as blanked and erased in the

display list and erase the object. New objects are replacements of

the object definition pointer which require an erase of the old object

and a rewrite with the new.  Objects which do no fall

into the four catagories(new,moved,unblanked,blanked) are static

objects and are not considered in pass 0.

Pass 1: Movers - Test for an overlap (underlap) with any

other object in the display list. Flag such objects for write.

Blanked - Test for an overlap with any other object

in the display list and flag that object for a write.

Write   - Test for and overlap with objects which

are not already flagged for erase,write ,or blanked. If an overlap,

flag the affected object for write.

Pass 2:	Write any object flagged for write and

clear all flags except blanked.

If all goes well, overlapping objects will be properly

updated and free floating statics will not be touched. Note that

all list actions are the result of Pass 0 operations.

#ased in the

display list and erase the object. New obj