CQP - California QSO Party contest logging program version 2.5

by Les Delmarter, WB6YIK

What is CQP?

CQP is a contest logging program for the California QSO party.

Features include on-screen display of multipliers worked/needed,

identification of duplicate contacts during logging, fast entry

of qso information, and on-line display of contacts per band/mode,

number of multipliers worked and total score. The program will

also generate the necessary forms and files for submission of your

contest entry, including a dupe sheet by mode & band, a log file and

a summary sheet ready for your signature.

New in version 2 is CW keying support and a faster file system

using an ISAM engine.

Hardware requirements:

IBM compatible PC, DOS 3.1 or higher, 640K memory. A color VGA

monitor recommended but not required. A serial port is required

for CW output.

Setup:

CQP comes initially configured for a monochrome monitor.  Color

can be enabled thru the setup option (F3) on the main menu bar.

You will need to fill in all of the fields in the setup screen

before the contest begins, as most of the information is needed

for logging purposes.  If you plan on using the CW option, you

will need to set the call letters field so that your call can

be sent.  You will also need to set the serial port number (1-4)

you are using for keying.  Setting the port number to 0 turns off

the CW output feature.  However, CW will be generated thru the

computer speaker if you switch to CW mode during logging.

Be sure to use a switching transistor, such as a 2N2222, to

key your transmitter.

Logging:

To begin logging QSOs, press the F1 key at the main menu bar.

Enter "PH" or "CW" for mode.  Enter the band in Mhz (valid bands are

displayed in the score box).  To go back a field, press the left-arrow

key.  To skip a field and go to the next field, press the ENTER key

or the right-arrow key.  Pressing the ESC key at any time returns

you to the main menu bar.  If you miss a field during logging, you

may skip it for now and go on to the next field.  The program will

not update the log or allow you to log another contact until the

missing information has been entered.  For example, a burst of QRM

prevented you from getting the other stations contact number.  Just

skip that field for the moment and go on to the multiplier field.

After entry of the multiplier field, CQP will determine that the

QSO number field is missing, and will re-prompt you for the data.

Once all fields have been filled in, the QSO is logged and moved

to the line below, and you are ready for the next contact.

Enter the callsign of the station being worked.  If this is a dupe

for this band and mode, the PC will beep and a dupe message will be

displayed.  However, you may continue logging this QSO, since you are

allowed to work the same station on this band and mode as long as they

have changed multipliers (state for California stations, county for

non-California stations).  If the dupe message has been displayed,

you have the option of pressing the Page Up key to view all log

entries for this station.

You may enter the time manually or press the ENTER key to use the

current system time (make sure you have set the GMT offset on the

setup screen, or set your PC's clock to GMT and the offset to 0).

Enter the QSO number as given by the station you are working.  At

this field you may also press the PgUp key to display all QSOs for

this station on this band and mode, allowing you to verify that

a station is a dupe and when they were worked. (use the F6 key

at the main menu bar for a more complete search and display function).

Enter the multiplier of the station.  Muliplier must be one of the

multipliers displayed in the multiplier box.  If this is a new

multiplier, the multiplier display will change to a lower intensity

(bright white to dim white, for example), indicating this multiplier

has been worked. If you have worked this station previously on

another band or mode, the multiplier previously logged will be

displayed in lower intensity in the multiplier field.  This lets

you see what they gave you last time, and allows you to just press

ENTER to default to the previously logged multiplier.  Very helpful

when you are having trouble copying a station's state or county.

This QSO has now been logged.  The score box will be updated

immediately, as well as the multiplier box.

To get back to the main menu bar, press the ESC key.  Press the

left-arrow key at any log field to go back to the previous field

(for example, press left-arrow to go from the NBR field to the

CALLSIGN field).

At the main menu bar, pressing the F2 key will allow you to edit

previously logged information.  Pressing the F3 key allows changes

to the setup screen, including the station call, type of entry, GMT

offset amount, CW port/speed and screen colors.

Press the F5 key to re-initialize the log file, F6 to display selected

QSOs, F7 to print your log entry for submission to the contest sponsor.

You may edit the file CWKEYS.INI to program the F5 thru F12 function

keys with any special text you may want to send when operating CW.

Keys F1 thru F4 are pre-programmed and will be ignored if added to

CWKEYS.INI.  Make sure the function key label is the first thing on

each line (F5:, F6:, etc...) and that the colon follows the key number.

The rest of the line may contain any text you want.

Please note that this version of CQP stores contact information in

CQP.MDB and summary totals by band/mode/multiplier in CQP.DAT.  If

necessary, CQP.DAT can be erased and CQP will re-build the totals

and recreate CQP.DAT.

Thank you for using CQP.  Let me know of any suggestions or problems.

I'm especially interested in hearing from users outside of California,

since most of my testing and all of my on-air use has been from within

California.

Les Delmarter, WB6YIK

10545A Road 240

Terra Bella, CA  93270