Pacsat Protocol Upload Processor for the AEA DSP1232/DSP2232

____________________________________________________________

(C) Copyright by ABAX BVBA, Belgium, 1993. All Rights Reserved.

This software may be used and distributed free-of-charge by radio-amateurs or

SWLs in the course of amateur radio communications. It may not be sold nor

used for any non-amateur purpose without written permission from ABAX.

This software was written by Andre Marchandise, ON1AIG in cooperation with

AEA - Advanced Electronic Applications.

================================================================================

__Version_Info__ * %n - V1 R02 M%v - Date %d *

* PG_AEA.DOC - V1 R02 M16 - Date 1-Nov-93 *

================================================================================

1. Introduction

This software was especially written for the AEA DSP-1232/DSP-2232.

It uses the features of this magnificent machine as much as

possible. This results in a more reliable uplink then ever

before possible.

The program is developed using C and a commercially available

communications package.

2. Copyright

(C) Copyright by ABAX BVBA, Belgium, 1993. All Rights Reserved.

This software may be used and distributed free-of-charge by

radio-amateurs or SWLs in the course of amateur radio communications.

It may not be sold nor used for any non-amateur purpose

without written permission from ABAX.

This software was written by Andre Marchandise, ON1AIG.

3. Packing List

This package consists of the following :

1. PG_AEA.EXE    : The Pacsat Upload Processor.

2. PG_AEA.DOC    : This documentation

3. PG_AEA16.TNC  : The file that get's uploaded to the DSP-UNIT

PG_AEA22.TNC    at the beginning of the program.

PG_AEA23.TNC

This file comes in three versions,

AO-16, UO-22 and KO-23.

Pls copy them to different directories.

Then rename PG_AEAxx.TNC to PG_AEA.TNC for

each of the three files.

4. PG_AEA16.CFG  : Configuration file, whith COM-port, baud rate,

PG_AEA22.CFG    etc.

PG_AEA23.CFG

This file comes in three versions,

AO-16, UO-22 and KO-23.

Pls copy them to different directories, in

such a way that the PG_AEAxx.CFG is in the

same directory as the corresponding

PG_AEAxx.TNC file.

Then rename PG_AEAxx.CFG to PG_AEA.CFG for

each of the three files.

4. Installation

First, PKUNZIP the PG_AEA.ZIP file.

Install the program ( PG_AEA.EXE ) and the accompaning

DOC-file ( PG_AEA.DOC ) in the directory where your other

satellite programs reside.

Now make sure the PG_AEA16.CFG and PG_AEA16.TNC end up in a

separate directory for AO-16. The same applies for

PG_AEA.22.CFG and PG_AEA22.TNC ( UOSAT5 ) and PG_AEA23.TNC and

PG_AEA23.CFG ( KITSAT1 ).

Make sure you rename the PG_AEAxx.CFG and PG_AEAxx.TNC to

PG_AEA.CFG and PG_AEA.TNC respectively.

Check your .BAT files and replace any reference to the

original PG-program with PG_AEA.

Users of SAS or Satsked will probably have to rename

PG_AEA.EXE to PG.EXE, because the executable is called

directly by another program.

5. Running the program

Run the program by typing "PG_AEA" ( without the double quotes ! )

at the DOS-prompt. ( or if you've renamed PG_AEA.EXE to

PG.EXE, you must type "PG" ).

After being started, the program first looks in the current

directory to see if there are files to upload. It will look

for files with extension .OUT ( not uploaded yet )

or with extension .PUL ( partially uploaded ).

If there are no such files, the program will exit.

Next the program initializes the TNC, by sending the file

PG_AEA.TNC.  IT IS NOT RECOMMENDED TO ALTER THIS FILE IN ANY

WAY, unless you know exactly what you're doing !!

The program immediately tries to connect the satellite, as

specified in the PACSAT PROTOCOL Suite Guidelines.

There is a time-out function of 120 sec. during the procedure

set-up, that will only start after receiving the first

OPEN-packet from the satellite. During DATA-upload, the

time-out is set to 360 sec.

Upon receiving the ESC-key from a program like STUFFIT, the

program will exit ( SatSked ).

The program makes sure to reset the parameters that were

altered, upon leaving the program.

6. Support of DSP PORT 2

If you want PG_AEA to support DSP port 2, you should add the

following statement to the PG_AEA.CFG file :

DSPPORT 2

Do not forget to alter the "MODEM"-statement in the

PG_AEA.TNC-file to support a Port 2 modem ( modem 28 replacing

modem 18 ).

Also check the other statements in your PG_AEA.TNC-file to

make sure they support port 2.

Example :

Using "FULLDUP ON" will only set DSP PORT 1 to fulldup.

Instead use either :

FULLDUP /ON     ( only DSP PORT 2 in fulldup )

or

FULLDUP ON/ON   ( recommended ).

7. Compatibility with other programs ( SAS, Satsked, .. )

The best way to avoid any compatibility problems, is to rename

your original PG.EXE in PG_UOS.EXE and then rename PG_AEA.EXE to

PG.EXE. I guess it is best to leave the original PG.CFG, PG.TNC, ...

in place, because some programs test for their existence.

Remember that PG_AEA.EXE ( now renamed PG.EXE ) uses

PG_AEA.CFG and PG_AEA.TNC. So DON'T RENAME them to PG.CFG/PG.TNC.

8. If you like the program, please leave a message on KO-23 ( preferred )

or on AO-16 ( regularly checked ).

9. In case of problem

If you're having a problem with the program, documentation,

config files or whatever pls. feel free to contact the author :

Andre Marchandise

ON1AIG

AX.25 : @ ON7RC.BT.BEL.EU

SAT   : @ KO-23 & AO-16 (QRT UO-22)

CIS   : 100272,1644