��������������������������������������������������������������ͻ

�      Propagation Forecast Bulletin Graphic Plot Utility      �

�                        Version 2.5                           �

�                                                              �

�                  Program file GRAPH.DOC                      �

��������������������������������������������������������������ͼ

��������������������������������������������������������������Ŀ

�              Background and General Information              �

����������������������������������������������������������������

I don't claim to be an expert on solar activity; in fact, that was why I

wrote this program.  I really enjoy reading Tad Cook's PROPAGATION

FORECAST BULLETIN each week (transmitted on W1AW and available on many

Packet BBS's) but I needed some way to put his sunspot and solar flux

values into a format I could relate to my on-the-air experience.  This

little program lets me do that by plotting the sunspot and solar flux

values from the PROPAGATION FORECAST BULLETINS in a graphic format.  I

can look at the plot and look at my logbooks and see why the band was

dead on a particular day, or why propagation was so good on a particular

day.

This is not a real fancy program.  It doesn't do MUF plots or anything

like that.  It is just a utility that puts things into perspective for

you, and makes them easier to visualize.  It will hold up to about 2400

daily entries for sunspot and 10.7cm solar flux values.  The data is

displayed graphically on two screens; one is an overall view showing all

of the data from day one to day "x".  The second is a close-up view of

196 entries.  When the data file gets quite large the entries get so

close together that it is difficult to see anything on the "Overall"

screen.  Also, when you are viewing just a portion of the data it is

difficult to get the "Big Picture".  Therefore, I give you two screen

modes. The overall is generally used for getting an overall view of the

data and the close-up screen is used for looking at more precise

information.  The close-up screen may be "Panned" left or right to scroll

through the data file month-by-month.

The vertical divisions on the screen correspond to days and weeks.  The

days are drawn in gray dotted lines and the weeks are in blue dotted

lines.  Monthly divisions (every 28 days starting at the beginning of the

file) are drawn in solid lines.  On the overall screen, the "Day" lines

have been omitted since they are normally too close together to see.

The program was designed to use a Microsoft(tm) compatible mouse as a

pointing device.  It does not require one, but you really need one to get

the most out of the software.  When a mouse is in use, you can "Click" on

an area of the overall screen and the software will automatically switch

to the close-up screen with the selected area in the center of the

screen.  Also, when a Mouse is used, the current position of the Mouse

cursor is displayed on the screen.  This allows you to measure values

from the screen.

��������������������������������������������������������������Ŀ

�                     Hardware Requirements                    �

����������������������������������������������������������������

This program was designed for the IBM-PC(tm) and compatible computers

with the MS-DOS(tm) operating system, Version 2.1 or later with at least

128 Kbytes of available memory.  An EGA or VGA video adapter and monitor

are required, and a Microsoft(tm) compatible Mouse is recommended but not

required.  A hard disk drive with enough capacity to store the program

and data files is highly recommended.

Page 1

��������������������������������������������������������������Ŀ

�                        Program Use                           �

����������������������������������������������������������������

Once activated, the program will read the data from the specified data

file and generate a set of averages. The initial display will be on the

close-up screen with the LAST day in the file at the right side of the

screen.  Vertical bars denote the days (dotted lines), weeks (bright

dotted lines), and months (solid lines) within the file.  Months as used

here are defined as 4 weeks or 28 days.  The day, week, and month lines

are all blue.  The number of sun-spots for each day are plotted as a

green line with the sun spot average plotted as a dotted cyan (light

blue) line.  The 10.7cm solar flux values are plotted as a violet line

with the averages plotted as a dotted red line.

Those of you with slower machines (especially the old 8086 machines like

I have) should be aware that it may take around a minute to do the

average calculations.  When the program is started, a bar on the title

screen will show the amount of data read, and the amount of the averages

that have been calculated. When the average interval or "Lag" days (see

below) are changed the averages have to be recalculated, so the

percentage completed is shown again during the recalculation.  With

faster machines the average calculations only take a few seconds; my

40 mHz 386 machine only takes about 8 seconds as opposed to 55 seconds

on my 10 mHz 8086 machine.  A math coprocessor speeds it up to less than

2 seconds on a fast machine.

Both the sunspot and flux values have the same scale.  A sunspot value of

100 will be exactly the same height as a solar flux value of 100.  The

higher on the screen, the higher the value.

The overall screen uses a reduced scale.  I initially wrote the program

without any scaling of the overall data, but it was so difficult to see

that I decided to scale it.  The vertical lines were so high in

comparison to the horizontal divisions that it was impossible to see

anything.  With the reduced scale, the more entries you have in the data

file, the smaller the height of the data.  In other words, as the data is

compressed across the screen, it is also compressed vertically to make it

easier see.

At the top of the screen is displayed the data for the currently selected

day. The "Currently Selected Day" is shown by an arrow at the bottom if

the screen if a Mouse is not in use, or by the Mouse cursor if a Mouse is

used.  The data that is displayed is the date, number of sun spots, and

the solar flux from the data file.  If you are using a Mouse, the current

vertical position of the Mouse cursor is also shows so that you may

"Measure" a value from the screen.

The left side of the screen shows:

o  The number of entries in the data file

o  The number of days to be displayed on the close-up screen

o  The number of days in the averaging period

o  The number of days lag in the averaging period

There are only a few keyboard commands to remember.  The next page (3)

is a summary of them:

Page 2

��������������������������������������������������������������Ŀ

�                      Keyboard Commands                       �

����������������������������������������������������������������

[S] .... Toggles between the OVERALL screen and the CLOSE-UP screen.

If you are in one screen mode, the program will switch to the

other mode.

[A] .... Select a different averaging period.  By default, the program

uses a 28-day averaging period (since the solar rotation

period is 27.5 days, this works pretty well), however you may

set the period for any desired value between 1 and the number

of days in the file.  More about the averaging later!

[L] .... Changes the number of "Lag" days in the averaging period.

This is explained in the section below!

[C] .... Defines whether cumulative averages for the first and last

part of the data file are displayed or not.

[D] .... Changes the number of days displayed on the close-up screen.

By default the program uses 196 days (or 28 weeks).  This

gives a pretty good display with enough room between the days

to see them.  If for some reason, you wish to use a larger or

smaller number of days, you may set the value by pressing the

[D] key and then entering the desired number of days to be

displayed.

[PG UP]. This is used to "Pan" the display left (or toward the TOP of

the data file). It is only active in the Close-up screen.

[PG DN]. This is used to "Pan" the display right (or toward the BOTTOM

of the data file). It is only active in the Close-up screen.

[LT ARR] The left arrow key moves the "Current Day" marker to the left

one day.  The data for the current day (shown at the top of

the screen) is updated to reflect the new day.  This key is

active only when a Mouse is not used.

[RT ARR] The right arrow key moves the "Current Day" marker to the

right one day.  The data for the current day (shown at the

top of the screen) is updated to reflect the new day.  This

key is active only when a Mouse is not used.

[?] .... Displays this HELP file!

[ESC] .. Exits the program and returns to DOS command level.

Page 3

��������������������������������������������������������������Ŀ

�                        Mouse Commands                        �

����������������������������������������������������������������

If you are using a Microsoft(tm) Compatible Mouse, your Mouse buttons

can be used like the [PG UP] and [PG DN] keys in the close-up screen.

When you are in the overall screen, you can select an area to look at

on the close up screen by placing the Mouse cursor on the desired area

and pressing the LEFT Mouse button.  The program will then switch to

the close-up screen with the identified day at the center of the

screen.

��������������������������������������������������������������Ŀ

�                   User-Definable Parameters                  �

����������������������������������������������������������������

There are four "User-definable" parameters; the number of days on the

close up screen, the number of days in the averaging period, the number

of "Lag" days, and whether "Cumulative Averaging" is on or off.  As

described above, all of these values have defaults that are used until

you change one.  When you change one (or more) of the defaults, a

configuration file is created to retain the new default values so that

they will be in effect the next time you run the program. The file is

named GRAPH.CFG, and if you want to edit this file you can, it is just

an ASCII file.  The first line is the number of days on the close-up

screen, the second is the number of days in the averaging period, the

third is the number of lag days, and the fourth line is either a one or

a zero to define whether cumulative average display is on or off.  To

just reset everything to the default values, you can just delete the

file GRAPH.CFG.

��������������������������������������������������������������Ŀ

�                     More About Averaging                     �

����������������������������������������������������������������

Tad Cook (KT7H) has been a great help in getting me straightened out on

how averages should be done. The averaging in this program uses a

moving-window average where each day on the average line represents a

value that is the average of a number of days before and after the

current day.  The sketch below may help you visualize this:

<-------------- 28 Day Averaging Period -------------->

�����������������������������������������������������Ŀ

13 Days Before + Current #        14 Days After

�������������������������������������������������������������������������

<------------------------- Days in Data File --------------------------->

The average for the current day is calculated by adding the values for

a certain number of days before and including the desired day and a

certain number of days after, then dividing by the number of days in

the period.  This means that the "Window" from which the average is

calculated "Moves" as the days progress through the data file.  The

number of days in the averaging period may be defined using the [A]

key.

Now, as shown in the following sketch, the number of days "Lag" also

comes into play.  Another method of averaging uses the number of days

in the period BEFORE the desired day to calculate the average.  For a

28 day period, the average value for any particular day uses the values

for the previous 27 days, along with the current day's value to

calculate the average.

Page 4

��������������������������������������������������������������Ŀ

�                   More About Averaging (cont.)               �

����������������������������������������������������������������

<-------------- 28 Day Averaging Period -------------->

�����������������������������������������������������Ŀ

27 Days Before + Current Day

�������������������������������������������������������������������������

<------------------------- Days in Data File --------------------------->

The number of "Lag" days, or the number of days BEFORE the current day,

used to compute the average is variable.  Some people like it one way,

some like it another.  Therefore, it is also user-definable.  You may set

the number of "Lag" days to any value between 1 and the number of days in

the period by pressing the [L] key and entering the desired number of

days.  The program will use this value as it's lag value.  The number of

days in the period minus this value will be the number of days AFTER the

current day used to calculate the average.  In other words, if you want

the average to be based on the PREVIOUS 27 days plus the current day,

enter 28 as the lag value. If you want the average to be based on the

previous 13 days plus the current day, and the following 14 days

(assuming a 28 day averaging period) enter 14.

Now, since the averaging period has a certain number of LAG and LEAD

days, the first part of the file and the last part of the file will not

have an average since part of the data required to calculate the averages

is not there.  If you need the previous 14 days to calculate an average,

the first day in the file doesn't have 14 days previous.  Likewise, if

you need 14 days past the current day to calculate the average the last

day in the file doesn't have them.  So what do you do about it?  One

recommendation is to just not display this data since anything there

would not be a correct average.  Personally, I prefer to see a cumulative

average up to the point where the moving average can take over.  The

choice is yours.  If you want to see the cumulative average, it is there,

displayed in WHITE instead of the normal color.  If you don't want to see

the cumulative averages, they can be turned off with the [C] key.

��������������������������������������������������������������Ŀ

�                    Data File Structure                       �

����������������������������������������������������������������

The data that is displayed graphically is stored in a data file on disk.

By default this file is named GRAPH.DAT but you can name the file

anything that you want to.  You may want to have a file for each year,

such as GRAPH.92 or PFB.93 or whatever you decide.  To run the program

with a file different from the default, just supply the file name on the

command line when you start the program.

Examples ... C>GRAPH .......... Starts the program using GRAPH.DAT

C>GRAPH PFB.93 ... Starts the program using PFB.93

The structure of the data file is simple.  It is just an ASCII file that

may be edited with any text editor.  Don't use a word processor unless

you can do so in ASCII mode since most word processing software puts

special control characters in the file that would not be understood by

this software.

Page 5

��������������������������������������������������������������Ŀ

�                       Data File Syntax                       �

����������������������������������������������������������������

The data file contains three columns of data, all three of which are

taken from Tad Cook's Propagation Forecast Bulletin.  The first is the

date, second is the sunspot value, and third is the 10.7cm Solar Flux

value.  Here are some examples:

04/19/91    152   229.0

04/20/91    122   201.0

04/21/91     92   180.0

04/22/91     73   166.0

05/20/93     48    91.1

05/21/93     34    91.0

05/22/93     22    91.8

Notice that the values are right-justified with the columns, or when the

sunspot or solar flux values are less than 100, the right-most characters

line up.  The date starts in column 1.  The sunspot value starts in

column 13, and the solar flux value in column 19. If you use the supplied

data file as an example, you won't have any trouble.

��������������������������������������������������������������Ŀ

�                            Summary                           �

����������������������������������������������������������������

That's about it.  As I said, the program is easy to use, and not too

fancy.  I wrote it initially for my own use, but at the request of Tad

Cook, I decided to let it become available to anyone that wants a copy.

If you want to pass it on to someone else, please feel free to.  While I

retain the copyright on the software, I do release it for NOT-COMMERCIAL

and NON-PECUNIARY distribution as long as the all program, data and help

files are transferred together and in an unmodified and unaltered state.

If you have problems or comments about the program, I'd love to hear from

you.  My address is correct in the 1988 or later callbook or as follows:

Scott Craig - WA4TTK

409 Jessie Drive

Nashville, TN 37211

WA4TTK @ KC4PRD.TN.USA.NOAM  (All Packet Mail)

WA4TTK @ AA4DO.TN.USA.NOAM   (Southeast US DX PacketCluster Address)

Please include the version number from your copy of the program.  It can

be found on the first screen when the program is started.

We all owe Tad Cook (KT7H) a vote of thanks for the Propagation Forecast

Bulletins he writes each week.  Speaking for myself, I enjoy reading

them, and look forward to them every week.  I also owe him a big Thank

You for his help in the writing of this program.  We have communicated

several times via Packet Mail, and his input has helped make this program

much more useful.

Thanks also go out to Joel - AA5YA for his help in reformatting my HELP

file into this nice documentation file, and Syd - VE3EGO for compiling

the 1989 through 1991 Solar Flux data.  I really appreciate their help!

73's ... Scott Craig ... WA4TTK ...