by Dr. W. Curtiss Priest

Center for Information, Technology and Society

Lexington, MA  02173

Internet: bmslib@mitvma.mit.edu

Voice: 617-862-7763

_______________________________________________________________________

***********************************************************************

ANET     EXE     49033 05-06-88   1:00p

ATALK    EXE     36801 05-06-88   1:00p

MINSES   EXE      1583 12-17-87   8:00a

REDIR    EXE     23718 12-17-87   8:00a

DA       EXE    143264 05-06-88   1:00p

LW       EXE     59480 04-27-89   3:38p

LW123    EXE     19248 04-27-89   3:38p

LWASC    EXE     33880 04-27-89   3:38p

LWMM     EXE     44376 04-27-89   3:38p

LWPRINT  EXE     19566 04-27-89   3:38p

LWWS     EXE     41224 04-27-89   3:38p

11 file(s)     472173 bytes

_______________________________________________________________________

***********************************************************************

Installed on May 26, 1993

The following is automatically done by install:

atalk /mem=7k  -- Version 2.0 driver, requires no interrupts, DMA #1

\laser\ashare -- to support AppleShare workstations, wants DOS 3.3 (must fool)

\laser\aprint -- printer driver

\laser\minses -- for a "network session layer" (does not find correct card now)

\laser\redir -- PC Network 1.0 "redirector" -- incorrect DOS

\laser\anet auto -- establishes environment, reads DA.DTA written by DA

for workstation name and name of printers, and other devices

So ----------- for printing just have atalk, aprint, and run anet

_______________________________________________________________________

***********************************************************************

Mysteries:

Did not know how to establish Laserwriter in startup

It is written into DA.DTA by DA

It is read by ANET with AUTO switch

Could not set Laserwriter to LPT1:

LPT1: was already assigned by DOS because detected port

So disabled parallel printer port on IDE card with jumper

Then DA would write this to DA.DTA, and then ANET would find

ATALK.COM does not require an IRQ (only DMA)

AppleTalk is 230.4 kbsec (bits not bytes)

so at 10 bits/baud it is like 23K baud, or at 8 bits/baud it is 29K baud

_______________________________________________________________________

***********************************************************************

ANET.EXE -- may run as a network supervisor or in batch

LOGOFF  /ALL  |  DosName

DISCON  /ALL  |  LPTx: [LPTx:]

CONNECT [ LPTx: N(Print Name) [ P(Type)] [ X(Conversion) ] [ Z(Zone) ]]

ATTACH  [ DRV: \\DosName\VolName\PathName ]

DETACH  DRV: [ DRV: ]

ACCESS  PathName [ O(Priv[="Owner"]) ] [ G(Priv[="Group"]) ] [ E(Priv) ]

Priv = +/-/= CDF (C = make changes, D = see dirs, F = see files)

MAP     [ Ext  [ Type ] ]

TYPE    FileName [ Type ]

NAME    [ Workstation Name ]

AUTO    Do all automatic connections saved by DA

quit    End session

DA.EXE -- DeskTop Assessory -- just like CHOOSER on Mac

Use to establish DA.DTA file

LW.EXE -- use to spool files to printer

MAIN MENU

Print

Printer Utilities

Return to DOS  #��

-- Select choice main.hlp

This is the Main Menu.  Up and Down Arrow keys move the

selection.  To select, press return.

PRINT:  lists applications for printing on the

LaserWriter.  Select the application used to create

the file that you want to print.

PRINTER UTILITIES:  lists choices for selecting a

printer, renaming a printer, and sending a test page.

RETURN TO DOS:  or the ESC key returns you to DOS.

prtutil.hlp

SELECT PRINTER:  Select which LaserWriter on the AppleTalk

Personal Network that will print your files.

RENAME PRINTER:  Is used to change the name of a selected

LaserWriter on the Network.

PRINT TEST PAGE:  Sends a test page to the selected

LaserWriter. Print a test page to verify a correct

installation of the card and the PC LaserWriter Program.

PRINT   WordStar   MultiMate   ASCII   Diablo 630

PostScript   Lotus Spreadsheet   Lotus Graph  #��

-- Select choice print.hlp

Select the application used to create the file that you want

to print.  Press RETURN to see a list of print options.

For Microsoft WORD or other files already in PostScript

format, select the PostScript option.

For applications not listed here, either configure the file

to print on a Diablo 630 printer and select Diablo 630, or

create an ASCII version of the file, and select ASCII.

ascii.hlp

Create an ASCII version of the file before selecting from

this menu.  You can print files from any application, as

long as they're in ASCII format.

ASCII FILE NAME:  Enter name (full pathname if necessary) of

file to print.

PRINT OPTION:  PRINT prints the file; SAVE saves PostScript

version of the file; PRINT/SAVE does both.

SAVE AS:  Enter name for saving PostScript version.

ORIENTATION:  PORTRAIT prints vertically, LANDSCAPE prints

horizontally.

NUMBER OF COPIES:  Enter from 1 to 100. Default is 1.

PAPER FEED:  Default is AUTO.

TEXT POINT SIZE:  Enter a point size.  Default is 12.0.

TITLE:  Prints filename, date, current time, and page number

on each page.

MARGINS:  Enter margins in tenths of an inch. Defaults are 5.

LINE NUMBERS:  Adds line numbers in left margin on each page.

PAGE BREAK:  Enter total lines per page.  Default is 66.

CHARACTER SET:  Lets you print non-ASCII characters.

diablo.hlp

wstar.hlp

WORDSTAR FILE NAME:  enter name (full path name if necessary)

of file to print.

PRINT OPTION:  PRINT prints the file; SAVE saves PostScript

version of the file; PRINT/SAVE does both.

SAVE AS:  Enter name for saving PostScript version.

ORIENTATION:  PORTRAIT prints vertically, LANDSCAPE prints

horizontally.

NUMBER OF COPIES:  Enter from 1 to 100. Default is 1.

PAPER FEED:  Default is AUTO.

SCALE:  Enlarges or reduces printing. Default is 1.00.

CHARACTER SET:  Lets you print non-ASCII characters.

mmate.hlp

To prepare a file in MultiMate for LaserWriter printing:

1) print the document to a file and 2) fill out the

MultiMate Print Document Utility Screen.

MULTIMATE FILE NAME:  Enter name (full pathname if necessary)

of file to print. File MUST be a .DOC file not .PRN).

PRINT OPTION:  PRINT prints the file; SAVE saves PostScript

version of the file; PRINT/SAVE does both.

SAVE AS:  Enter name for saving PostScript version.

ORIENTATION:  PORTRAIT prints vertically, LANDSCAPE prints

horizontally.

NUMBER OF COPIES:  Enter from 1 to 100. Default is 1.

PAPER FEED:  Default is AUTO.

SCALE:  Enlarges or reduces printing. Default is 1.00.

CHARACTER SET:  Lets you print non-ASCII characters.

lotuss.hlp

Lotus spreadsheets must be printed to a file before you can

use this menu.  Use the Lotus /Print command to select

spreadsheet options, and the File option to print to a file.

LOTUS FILE NAME:  Enter name (full pathname if necessary) of

file to print.  File must be a .PRN file.

PRINT OPTION:  PRINT prints the file; SAVE saves PostScript

version of the file; PRINT/SAVE does both.

SAVE AS:  Enter name for saving PostScript version.

ORIENTATION:  PORTRAIT prints vertically, LANDSCAPE prints

horizontally.

NUMBER OF COPIES:  Enter from 1 to 100. Default is 1.

PAPER FEED:  Default is AUTO.

TEXT POINT SIZE:  Enter a point size.  Default is 12.0.

TITLE:  Prints filename, date, current time, and page number

on each page.

MARGINS:  Enter margins in tenths of an inch. Defaults are 5.

LINE NUMBERS:  Adds line numbers in left margin on each page.

PAGE BREAK:  Enter total lines per page.  Default is 66.

CHARACTER SET:  Lets you print non-ASCII characters.

lotusg.hlp

Save Lotus graphs normally as a .pic file before using this

menu.  Use the Lotus /Graph option for the graph's design.

LOTUS FILE NAME:  Enter name (full pathname if necessary) of

file to print.  File must be a .pic file.

PRINT OPTION:  PRINT prints the file; SAVE saves PostScript

version of the file; PRINT/SAVE does both.

SAVE AS:  Enter name for saving PostScript version.

ORIENTATION:  PORTRAIT prints vertically; LANDSCAPE prints

horizontally.

NUMBER OF COPIES:  Enter from 1 to 100. Default is 1.

PAPER FEED:  Default is AUTO.

TITLE FONT:  Select font for the graph's title.

LETTERING FONT:  Select font for all other graph lables.

post.hlp

Microsoft WORD and other applications create files already

in PostScript format.  LaserWriter print choices are

available from each application's own menus.

PostScript files must be printed to a file before you use

this menu.

Microsoft WORD requires that you send an initializing file

to the LaserWriter at least once before you can print.  For

WORD versions 2.0 and 2.01 the initializing file is named

mssetup.ps; for version 3.00 it is named applaser.ini.

POSTSCRIPT FILE NAME:  Enter name (full pathname if

necessary) of file to print. File must be the one you printed to

a file while in the application.

PAPER FEED:  Default is AUTO.

rename.hlp

Type a new name for the selected LaserWriter.

The same name cannot be used twice in the same zone, but

different LaserWriters can have the same name if they are in

different zones.

selzone.hlp

A zone is one section of the Network.  Select the zone that

contains the LaserWriter you want to select.  Press RETURN.

It may take a few moments to find all the LaserWriters on

the selected zone.

selpr.hlp

Select the name of the LaserWriter you want.  Press RETURN.

That LaserWriter will now be the one that prints your files.

Press ESC to get back to the print menu for your

application.

LWASC.EXE

Usage:  lwasc [-option] infile [outfile]

Available options:

-a   Append PostScript output to existing file.

-b   Print entire file in BOLD.

-c#  Number of copies to print.

-e#  Set tab spacing.

-f   Process list of files.

-i#  Use International character set.

-k   Keep intermediate PostScript file.

-l#  Set page-break at specified line #.

-mt# Set top margin in tenths of an inch.

-ml# Set left margin in tenths of an inch.

-mb# Set bottom margin in tenths of an inch.

-n   Print line numbers.

-oc  'C' program listing option.

-oa  Assembler program listing option.

-p   Don't send to printer.

-r   Rotate to Landscape orientation.

-s#  Set character size in points.

-t   Print title and date/time header on each page.

-z#  Set paper size.

LWMM.EXE

Usage:  lwmm [-option] infile [outfile]

Available options:

-c#  Number of copies to print.

-i#  Use International character set.

-k   Keep (don't delete) output file.

-r   Rotate to Landscape orientation.

-s#  Set document scale factor.

LWPRINT.EXE

Usage:  lwprint [-option] filename

Available options:

-a   Strip ASCII End-Of-Text marker from file.

-c#  Number of copies to print.

-m   Manual paper feed.

-q   Non-destructive console operation.

-w   Wait for LaserWriter to finish printing.

LW123.EXE

Usage:  lw123 [-option] infile [outfile]

Available options:

-c#  Number of copies to print.

-f0  Specify font for Title.

-f1  Specify font for other lettering.

-k   Keep intermediate PostScript file.

-m   Use the Mini-Menu.

-r   Rotate to Landscape orientation.

LWWS.EXE

Usage:  lwws [-option] infile [outfile]

Available options:

-c#  Number of copies to print.

-i#  Use International character set.

-k   Keep (don't delete) output file.

-r   Rotate to Landscape orientation.

-s#  Set document scale factor.