----====<   VBE/AI Driver Sample Source Code   >====----

----====<	Copyright (c) 1993,1994 VESA, Inc.  >====----

----====<		All Rights Reserved	    >====----

----====<	      Version 0.98   12/16/93	    >====----

INTRODUCTION:

This document describes the VESA VBE/AI Software Developers

Kit as found on the VBE/AI SDK release diskette. Included

are sample WAVE drivers for the Pro Audio Spectrum, Disney

Sound Source, and Sound Blaster cards.	Also included are

sample MIDI drivers for the OPL2, OPL3 FM chip, and Roland

MPU-401 MIDI transmitter/receiver.

Each body of code is as complete as possible for the 0.98

revision of the VBE/AI specification. The code may still

change to match any changes when the specification is

adopted, or to fix bugs.

EXAMPLE DRIVERS:

The VBE/AI example drivers were written as DOS TSRs. There

is one API per TSR. Each one must be loaded separately into

memory. The program, SETPREF.EXE lets the user assign a

preference level to each driver.

LOADING SAMPLE DRIVERS:

Each driver uses a common set of command line parameters. The

set looks like this:

/H      helps

/?	    helps

/B:dd   board # if multiple boards are present

/A:xx   board address

/D:xx   dma channel

/I:dd   irq channel

/MB:xx  memory base (segment)

/MR:xx  memory range

/P:dd   user preference setting

/U	    unload this driver from memory

dd=decimal #, xx=hex #

To load a driver, just run the program. The program decides

if the hardware is present, then will stay resident. The

VBE/AI architecture does allow resident TSR drivers to be

unloaded. Only the first driver loaded will be permenantly

resident, all others may unload. The available drivers are:

PASWAVE.COM - Pro Audio Spectrum WAVE driver

SSWAVE.COM	- Disney Sound Source driver

SBWAVE.COM	- Sound Blaster & Compatible driver

OPL2.COM	- OPL2 MIDI driver

MPU.COM	- MPU-401 driver

RUNNING SAMPLE APPLICATIONS:

There are five sample programs currently available in the

SDK. The source for each program is included on the

diskette. Two programs test the WAVE drivers, two test the

MIDI drivers, and the last is a utility for both setting

user preference levels and displaying the current installed

driver list.

PMIDI.EXE

This program will play any type 0 or type 1 MIDI file. The

file must be under 64k in size. Currently, the program

requires the driver's patch library to be resident in the

current directory, specfically, OPL2.BNK. This version can

only handle patch banks under 64K, but will soon support

unlimited sizes. The following is a dump of the banner and

command line parameter list:

VESA VBE/AI MIDI Output Test Program, V0.98

Copyright (c) 1993,1994  VESA, Inc. All Rights Reserved.

To Use:  DOS>pmidi [song] [H] [L] [Mxxx] [K{+|-xx}] [R{+|-xx}] [V{xx}]

Where: [song] is the .MID file to be played

[H]	      High end synth playing on channels 1-10.

[I]	      to receive MIDI input only.

[K{+|-}xx] shifts all key #s up|down by xx count (cheap transpose).

[L]	      Low end synth playing on channels 11-16.

[Mxxxx]    patch Map file name (256 bytes for 2 tables).

[Pxx]      selects a VBE device at this (xx) user preference level.

[R{+|-}xx] shifts all patch #s up|down by xx count.

[T+|-xx]   Tempo shift faster (+) or (-) slower, in percent.

[V{xx}]    verbose mode to dump events as they play. xx can be 1-5.

NOTE.EXE

This program allows the user to enter a channel and note

number to be loaded into the target MIDI device. Once the

patch has been loaded, a menu of functions will be provided

to allow the user to turn the note ON or OFF.

To Use:  DOS>note [Pxx]

Where:	 [Pxx] is the user preference #.

NOTE: all values are zero based, so the key number ranges

from 0 to 127. Channels range from 0 to 15. Channel

number 9 is the percussive channel.

PLAY.EXE

This program tests the WAVE output of a given driver. It

allows the user to specify a starting block size,

incremental length, number of times to play the block, and

more. The program will send the block to the driver, and

wait for the appropriate callback within a fixed amount of

time. This time depends upon the sample rate and block size.

The following is a dump of the banner and command line

parameter list:

VESA VBE/AI WAVE Output Test Program, V0.98

Copyright (c) 1993,1994  VESA, Inc. All Rights Reserved.

To Use:  DOS>play [16] [Axxx] [Fxxx] [Dxxx] [Ixxx] [Lxxx] [R] [S] [Txxx]

Where: [16] enables 16 bit audio (8 bit is default).

[Axxx]  uses PlayCont/RecordCont. xxx is an optional block size.

[Fxxx]  user selects an alternate .WAV/.VOC file by the name of xxx.

[Dxxx]  allows for delays between blocks of xxx clock ticks (18.2tps).

[Ixxx]  after each block, Increment the next length by xxx.

[Lxxx]  block Length of first pcm block.

[R]     Reload the sample rate after each block finishes.

[S]     selects Stereo mode operation.

[Txxx]  number of Times to play the block.

REC.EXE

This program tests the WAVE input of a given driver. It

allows the user to specify a starting block size,

incremental length, number of times to play the block, and

more. The program will send the block to the driver, and

wait for the appropriate callback within a fixed amount of

time. This time depends upon the sample rate and block size.

The following is a dump of the banner and command line

parameter list:

VESA VBE/AI WAVE Input Test Program, V0.98

Copyright (c) 1993,1994  VESA, Inc. All Rights Reserved.

To Use:  DOS>rec  [16] [Axxx] [Fxxx] [Dxxx] [Ixxx] [Lxxx] [O{+}] [R] [S] [Txxx]

Where: [16] enables 16 bit audio (8 bit is default).

[Axxx]  uses PlayCont/RecordCont. xxx is an optional block size.

[Fxxx]  user selects a new File by the name of xxx.

[Dxxx]  allows for delays between blocks of xxx clock ticks (18.2tps).

[Ixxx]  after each block, Increment the next length by xxx.

[Lxxx]  block Length of first pcm block.

[O{+}]  Save one recorded block. '+' saves all blocks.

[R]     Reload the sample rate after each block finishes.

[S]     selects Stereo mode operation.

[Txxx]  number of Times to play the block.

SETPREF.EXE

This program allows the user to set his/her preference level

to each driver. This allows the application to choose which

driver is the most desirable to use. Also, the program shows

a listing of the driver configuration. The following is a

dump of the banner and command line parameter list:

VESA VBE/AI Preference setting Program, V0.98

Copyright (c) 1993,1994  VESA, Inc. All Rights Reserved.

To Use: DOS>SETPREF [H|?] [L] [+] [+xx] [-] [-xxx] [Vxxx]

Where: [L]	  lists all devices.

[+]	  increments all preference levels by 1

[+xxx]	  increments a specific devices's preference.

[-]	  decrements all preference levels by 1

[-xxx]	  decrements a specific devices's preference.

[Vxx]	  give a verbose listing of this driver.

NOTE: The highest preference level is zero (0), so incrementing the

preference actually lowers the user preference, and visa versa.

COMPILING SAMPLE SOURCE CODE:

All the example code was developed using Microsoft 'C' and

MASM Assember.	Debugging of the TSR's was done using

Nu-Mega's Soft-ICE and applications using Codeview. To build

the code, use the included batch files and MAKE files.

Again, the MAKE files depend upon the above directory

structure to build the executables. The following list

describes the files needed to build this driver release:

1. The file 'MKSDK.BAT' executes several MAKE sequences

to build the VBE/AI SDK program set.

2. The file 'MAKEFILE' is the general make file to

compile the five sample drivers.

3. The file 'PLAY' is the make file to build the WAVE

playback (PLAY.EXE) and record (REC.EXE) test programs.

4. The file 'PMIDI' is the make file to build the MIDI

player program.

5. The file 'NOTE' is the make file to build the NOTE

player program.

6. The file 'SETPREF' is the make file to build the user

preference adjustment program.

7. The file 'VOLUME' is the make file to build the Volume

control test program.

DRIVER DISKETTE CONTENTS:

Disk Contents:

SDK.DOC         Intro documentation to the SDK

SDK.BAT         Batch file to build the release floppy

MKSDK.BAT	Batch file to build the example programs

PASWAVE.COM	Pro Audio Spectrum WAVE VBE/AI driver

SSWAVE.COM	Disney Sound source WAVE VBE/AI driver

SBWAVE.COM	Sound Blaster & Compatibles WAVE VBE/AI driver

OPL2.COM	OPL2 VBE/AI MIDI driver

MPU.COM 	MPU-401 VBE/AI MIDI driver

ALFRE.MID	Sample MIDI file

GUPPY.WAV	Sample WAVE file

OPL2.BNK	Instrument bank for the OPL2 MIDI driver

MT32		Patch map from GM to an MT32

PLAY		Make file for the WAVE PLAY.EXE program

PLAY.C		Source to PLAY.EXE

PLAY.EXE	PLAY.EXE executable for playback testing of WAVE audio

REC.EXE 	REC.EXE executable for record testing of WAVE audio

NOTE		Make file for the single MIDI note test program

NOTE.C		Source code for the single MIDI note test program

NOTE.EXE	Executable MIDI note test program

PMIDI.C         Source to PMIDI.EXE midi file player

PMIDI		Make file for the MIDI PMIDI.EXE program

PMIDI.EXE	PMIDI.EXE executable for MIDI playback

PDATA.C 	Static data for PMIDI.C

SETPREF 	Make file for setting user preferences & general query

SETPREF.C	Source to SETPREF.EXE executable

SETPREF.EXE	SETPREF.EXE executable to set user preference levels

VOLUME		Make file for the volume test program

VOLUME.C	Tests the Volume control of a given device

VOLUME.EXE	Executable volume test program

VESA.C          Mid level helper code for VBE/AI driver support

MIDI.H		Miscellaneous header files

VBEAI.H

VESA.H

MIDI.INC

VBEAI.INC

WAVE.INC

MIDI\OPL2\TOOLS A directory with s/w for building the OPL2.BNK and

MT32 patch map.