*********************

*   The WaveGazer   *

*********************

The WaveGazer v1.0

A Receiver Control Program

for the

JRC NRD-535D

Written during long years

and released in

January 1994

by

Andreas Meister

Route de la Gare 13

CH-1131 Tolochenaz

Switzerland

*********************

*   The Wavegazer   *

*********************

January 1994

User's Guide                The WaveGazer v1.0                 Page 2

----------------------------------------------------------------------

0. Table Of Contents                                 2

1. Generalities

1.1 Standard Disclaimer, Warranty                  4

1.2 Hardware Requirements                          4

1.3 Software Requirements                          5

1.4 Third Party Software                           5

1.5 Inspiration                                    5

1.6 Restrictions                                   6

1.7 Copyright                                      6

1.8 How To Tell Me If You Like The WaveGazer       6

2. How To Get The WaveGazer                          7

3. History                                           8

4. A Quick Overlook                                  9

5. Installation

5.1 Installation On Harddisk                      11

5.2 Run The WaveGazer                             11

5.3 Tell The WaveGazer About Your PC's Speed      13

6. General Usage

6.1 Windows                                       14

6.2 Menus                                         14

6.3 Menu Options                                  14

6.4 Textfields                                    16

6.5 Popupboxes                                    16

6.6 File Selection Boxes                          17

6.7 Restrictions                                  17

7. General Receiver Control                         18

8. Receiver Control Window

8.1 Controllable Features                         19

8.2 Attaching / Detaching The Receiver To PC      19

8.3 Operation                                     20

8.4 Keystrokes                                    21

9. The Memory Editor

9.1 Description                                   23

9.2 Operation                                     23

9.3 Keystrokes                                    24

10. The Memory Viewer

10.1 Operation                                    26

10.2 Keystrokes                                   26

11. The Bandscan

11.1 Description                                  27

11.2 Operation                                    28

11.3 Keystrokes                                   30

12. The Predefined Bands

12.1 Theory                                       31

12.2 Examples                                     32

User's Guide                The WaveGazer v1.0                 Page 3

----------------------------------------------------------------------

13. Automatic Recording

13.1 Features                                     33

13.2 Operation                                    33

13.3 Keystrokes                                   34

14. The Database

14.1 Concepts                                     35

14.2 Usage Of Databases                           35

14.3 Creating A New Database                      36

14.4 Tracking Mode                                36

14.5 Database Cache                               36

15. The Database Window

15.1 Description                                  37

15.2 Operation                                    37

15.3 Keystrokes                                   38

15.4 Loading Database Entries To Memory           39

15.5 Keystrokes In The Memory Loader Window       39

16. Tracking Mode

16.1 Purpose                                      40

16.2 Operation                                    40

16.3 Keystrokes                                   41

17. Search Window

17.1 Concepts                                     42

17.2 Operation                                    42

17.3 Keystrokes                                   43

18. Adding / Editing A Record

18.1 Concepts                                     44

18.2 Operation                                    44

18.3 Editing A Record                             45

18.4 Keystrokes                                   45

19. Customization

19.1 Customization Of The WaveGazer               46

19.2 Customization Of The Printer                 46

19.3 Customization Of The Colors                  47

20. Release Notes                                   48

21. Problems                                        51

User's Guide                The WaveGazer v1.0                 Page 4

----------------------------------------------------------------------

1. Generalities

1.1 Standard Disclaimer, Warranty (none ;-)

This version of the WaveGazer is the first 'production' version

of this product. I do not claim that this version is perfect, but

I tried to eliminate all known bugs so far. If ever you find a

bug, you are kindly invited to communicate it to me.

Since the WaveGazer is available free of charge, you install it

on your own risk and I can not be made responsible for any damage

made to your system hardware or software. Also I do not take any

commitment for fixing bugs, but I try to keep the WaveGazer on a

good quality level, so I am really interested in your suggestions

and bug reports.

Please:   If you submit a bug report, always include the version

number and a complete description of your PC hard- and

software.

1.2 Hardware Requirements

The hardware I tested the WaveGazer on is rather limited: My two

286 clones I use at home and on a Olivetti 486/25 MHz and a

386/25 MHz. Once again, I would like to know on what hardware you

are running it, to enlarge my so tiny list. I also tried it on PC

emulators on a Unix machine and a WindowsNT PC: SoftPC from

Insigna Solutions on a DECstation 5000-200 running Ultrix and the

DOS emulator on a DEC AXP PC 150 running Windows NT! This does

not mean at all that DOS emulators on machine X are supported!

The two above mentioned examples are professional products,

emulating the PC hardware rather than DOS itself, so consider

them as an exception!

The minimal hardware requirements are:

*    IBM AT compatible or better (286 up)

*    VGA display, 640 * 480 or better (load driver for 640 * 480)

*    Microsoft compatible mouse

*    1 unused serial port

*    1 MB Ram

*    Hard disk, 20 Mega or more (the WaveGazer itself does not

take a lot of disk space, but your databases maybe will?)

User's Guide                The WaveGazer v1.0                 Page 5

----------------------------------------------------------------------

1.3 Software Requirements

The program has only been tested on MS-DOS v5.0, but it also runs

on other DOS versions, I saw it on v6.0 (without stacker...!) and

I started development under DOS v4.0

*    MS-DOS v5.0 or later

*    'share' must be loaded for database use

1.4 Third Party Software

This program has been compiled using Borland C++ v2.0. The

graphic is based on the well known BGI (Borland Graphic

Interface)

While most of the software is written by me (very proud...), I

used some components out of the freeware/shareware pool:

*    The database is implemented using CBase from Citadel

Software

*    The communication routines are based on IBMCOM, written by a

certain RAC...

*    The logo was created using Microsoft Windows Paint, and then

converted to a BGI file using the public domain software

'BMP2BGI' from R. G. Rasualis, Jr.

*    The implementation of the mouse interrupt is heavily

inspired by Dave Kirsch

1.5 Inspiration

I did never see a receiver control software before I decided to

write my own. But sometime ago, I got a copy of Tom Roach's NRD

program and this lead me to some enhancement in my band scan

feature, and to the inclusion of what I call the "tracking mode".

I was impressed by the NRD Software.

There is also another Swiss, Rolf Hanggi from the company SHOC,

who releases a very complete receiver control program for a

variety of receivers, called the Radio Manager 3. I read a review

about this program and decided to copy one of its features. This

feature is not yet included in the current version of the

WaveGazer!

Thomas Sundstroem from TRS Consultants and Richard Crisp were

both a big help in testing and debugging the program. Thanks

Guys...!

User's Guide                The WaveGazer v1.0                 Page 6

----------------------------------------------------------------------

1.6 Restrictions

*    The program may crash or hang if you use the WaveGazer

without the receiver connected to the serial port. I should

include some more error checking if no correct answers are

read from the serial port, e.g. if another device than the

'535 is connected to the port the WaveGazer thinks the '535

is. (a RTTY decoder, for instance...)

*    If you stop the WaveGazer other than by using the foreseen

menu option, the WaveGazer may let files in the directory

pointed to by the environment variable 'WG_SWP'. So checking

this directory periodically for left files may free missed

disk space. Best workaround for this problem is to use a

ramdisk as swap directory: You do not have to worry about

eventually let files and you gain in performance. You also

may want to startup the WaveGazer from within a batch file

which scans the directory for such files and then deletes

them.

*    Do not have the variable WG_FIL point to directory on a

stacked disk, this will prevent the WaveGazer from startup.

*    For more restrictions, see chapter 6.7

1.7 Copyright

This software is distributed freely to all owners of the NRD-535.

I do not know how to copyright something, so it is not under any

but a moral copyright. Myself, I copied ideas from other

programs...

Anyway, I ask programmers to tell me if they felt inspired by

this program, and all other users if and how they use it. Just

for my personal pride.

1.8 How To Tell Me If You Like The WaveGazer

An important question. I wondered quite a long time if the final

version of this software should become Shareware, if I should

distribute a crippled version to force people to register, etc.

But all this would not match with the spirit of free information

exchange we are used to from the world of radio and from the

electronic networks. This is why I decided to make it Freeware.

Anyway, there is a big amount of time digged in over 22'000 lines

of C/C++ code, and so on... (You know these phrases from other

network distributed software packages) If ever you feel a need to

be a registered user, send me what you feel adequate and I will

keep you personally updated with new versions if ready.

One thing I would like to ask every user of the WaveGazer is to

announce itself as a such, by sending me a simple postcard from

the place he lives. My wife and I adore getting postcards ;-)

User's Guide                The WaveGazer v1.0                 Page 7

----------------------------------------------------------------------

2. How To Get The WaveGazer

Since the WaveGazer is free of charge, you are invited to share

this software with other owners of the NRD-535D. I would not like

sending out diskettes and printed manuals to non-paying users, so

let's limit the free distribution to the Internet. I can e-mail

uuencoded zipfiles, which you have to assemble, uudecode and

unzip.

Hopefully there are some servers or BBS's on the net that are

willing to store this software for downline loading.

I cannot make the software available for anonymous FTP, since I

work for digital and we are not allowed to have our machines

directly connected to the Internet.

But there is at least one anonymous ftp site where from you may

copy the software: Risto Kotolampis server called "nic.funet.fi"

which holds an important collection of radio related software and

textfiles. Look for the file WG010.ZIP in the directory

"msdos/software".

Since I would like anybody using the WaveGazer to communicate me

this, here my home address:

Andreas Meister

Rte. de la Gare 13

1131 Tolochenaz

SWITZERLAND

(If you have a map of Europe/Switzerland, look on the north shore

of the large lake called "Lac Leman" for a town called

"Lausanne", and there it is nearly...)

I can be reached on the Internet under the following addresses:

(But don't forget the postcards...! ;-)

andreas@solarx.enet.dec.com

meister@leman.enet.dec.com

meister@lsncsc.enet.dec.com

User's Guide                The WaveGazer v1.0                 Page 8

----------------------------------------------------------------------

3. History

Once upon a time, my mother brought home some forms to

participate at a publicity contest from Philips. The hole family

filled in some tens of these forms, and we sent them to Philips.

I forgot about them soon, but one nice day, the postman rang the

doorbell, holding a parcel for me: A radio I won in the contest.

It was a simple 4-wave radio, featuring a so called short-wave

magnifier.

Soon I spent my time fiddling around with this and other radios,

I also bought some old 40-ies home receivers for close to

nothing.

One of these was a very nice one, with all major SW broadcast

bands spread over a large display, a "magic eye" and a knob for

bandwidth switching! (I only found later that this was the

bandwidth, first I thought that it was just a second "tone"

knob...) I used this engine to listen to Radio Beijing, and I

sent a reception report to China. I was very proud as I got a QSL

card from Beijing! Then, I was some more involved in school, and

I let the SWLing more or less be.

Some years later, the nice little Sony ICF7600 was launched. I

had no sleep until I had it and even less after I had it... A lot

of SWLing filled my time during 4 years, then I began real life

(work) and moved in a very small apartment without any

possibility for an aerial, so the Sony was only used on some

holiday trips.

A few moves later, I found myself in a larger apartment with a

Yaesu FRG7700, which I bought used, but never really used. My

project was to build a fax decoder on a old Mac I owned these

days, but soon I found that this was not the engine I needed.

(The old Mac). I bought a PC clone and a fax decoder, but was not

really satisfied with it, and let the whole thing be.

Rather I began to write some programs, learning C. After I got a

C++ primer in my hands, I bought a C++ compiler and wrote a class

called "button" and some mouse handling routines. (the birth of

the WaveGazer). Some scrollbars, textfields, togglebuttons etc.

later, my (then still) girlfriend and I went to a trip to

Argentina. With the ICF7600. Lots of time for listening to Latin

American stations, and soon I felt an inner need for a nice

receiver. One with a computer control feature. Why not assemble

my programs to a receiver control program? (This decision was

taken during a shower in a hotel in a Argentine town called San

Martin de Los Andes...)

Took the decision, bought the receiver and had a lot of undone

programming. But one fine day, I thought that this program now

should be ready to be used, so here it is!

The WaveGazer v1.0!

User's Guide                The WaveGazer v1.0                 Page 9

----------------------------------------------------------------------

4. A Quick Overlook

This software has several features which (I hope) you may find

useful.

*    The whole thing is built on a graphic interface, so you can

activate all the stuff with mouse clicks, -motions and

double-clicks, whereas most functions also can be accessed

from the keyboard. The user interface is some 'Pseudo-

Windows' but very basic since home made...

*    One window is to control all possible receiver functions

from the computer, there are still some analogous functions

that cannot be controlled via RS232. This window is not very

useful, controlling the receiver manually is much more

convenient.

It may be useful for very fine tuning, since the values for

frequency, bandwidth and passband shift can be read out very

accurate.

*    Two more windows serve to manage the '535's 200 memory

channels. You may edit every channel individually (define,

delete, change, etc.) and you may overlook these definitions

in another window. Useful is the fact that you can save the

200 memory channels in a diskfile, so you always can keep as

much memory definitions as you want, in files of up to 200

channels.

Channels are directly tunable, and receiver settings may be

dumped directly in a channel.

*    Exciting is the Bandscan feature: You may draw a graph of

the occupation of a band and then you may click in the

graph, e.g. on a peek representing a transmitter, and the

receiver will be tuned directly to this transmission. Mouse

dragging is possible, to stroll around the dial.

These scan graphs may be saved in a file for later

reloading, a previously interrupted and saved scan may be

reloaded and resumed. The scanning can be most flexibly

customized, and you may customize the scan feature to be

able to select the most used and preferred band with just a

mouse click.

*    There is also window that allows you to do unattended

recording: the 535 has a timer relay on the back, which will

be switched on at up to eight predefined times while the

receiver will be tuned to a user defined memory channel.

During long waiting periods the receiver is switched off to

save its lifetime and your A/C current.

*    A database system exists to hold either a database with

records entered from a schedule or your 'would like to catch

it' stations. In a line oriented display, you can click,

scroll, etc. around these entries, tuning the receiver

directly to a selected entry. The database can be browsed

following user defined search criteria, and you may have a

unlimited number of databases, although only one open at a

given time.

User's Guide                The WaveGazer v1.0                 Page 10

----------------------------------------------------------------------

*    The logbook is almost the same as the database, except that

the fields in the database entry differ slightly to reflect

their use. You can have either the database or the logbook

open, but not both at a time.

*    Browsing in the databases (logbooks), you are able to print

the found records, load them directly into the receiver's

memory  (starting at a user defined channel number) and/or

to load them in a file for later reloading the receiver's

memory.

*    With an open database, the radio may be manually tuned while

the screen display highlights the entries in the database

which match the current receiver frequency. The threshold

for this matching is a user defined parameter.

*    Databases may be printed out on virtually every printer, at

least if your printer is able to print 132 characters / line

(limited usage also possible with 80 characters / line)

*    Colors may explicitly be customized, while the preferred

(last used) settings are saved for the next invocation of

the program.

User's Guide                The WaveGazer v1.0                 Page 11

----------------------------------------------------------------------

5. Installation

5.1    Installation on Harddisk

*    Installation is straightforward. Create a directory where

you want to put all files concerning the WaveGazer. Do not

install on a stacked partition under DOS v6.0!

C:\>mkdir WG

C:\>cd WG

*    Unzip the zipfile using pkunzip.exe or any other compatible

unzipper

C:\WG>pkunzip A:\WG010.ZIP

*    Add the directory containing WG.EXE to your path or move

WG.EXE to a directory in your path.

*    Create a swap directory: The WaveGazer saves the graphics

hidden by the windows in files to save precious memory. So

we need a dedicated directory for these temporary files. (I

use a ramdisk, for better performance, but my disks are old

and slow...)

C:\WG>mkdir SWP

*    To be able to invoke the WaveGazer from everywhere, we need

to define 2 environment variables in your autoexec.bat:

set WG_FIL=C:\WG         #this is where all data-

#bases etc. will reside

set WG_SWP=C:\WG\SWP     #where the windows save what

#they hide for later restore

*    The database the WaveGazer relys on requires share to be

loaded, so add a line for share to your autoexec.bat.

*    Reboot your PC

5.2 Run The WaveGazer

Once your PC booted, type

C:>WG [com] [printer] [PCtime - UTC] [RXtime - UTC]

where the optional parameters com and printer define the ports

where the receiver and the printer are attached. Default values

are 1 (com1) for the receiver and 'lpt1' for the printer. If you

have your printer on com1 and the receiver on com2, invoke the

WaveGazer with

C:\>WG 2 com1

User's Guide                The WaveGazer v1.0                 Page 12

----------------------------------------------------------------------

If the printer is on lpt1 but the receiver on com2, type

C:\>WG 2

Only '1' or '2' are supported values for the receiver port, so do

not connect your receiver to an eventual com3/4 port.

The parameter for the printer can have any value, as long as it

represents a valid device or file name. That means you may also

print to a file

The parameters [PCtime - UTC] and [RXtime - UTC] specify the time

differential your device's clocks have respective to UTC. It is

always difficult for me to say if we are ahead or behind UTC, so

lets talk about 'East' or 'West'. If you are west of UTC

(Greenwich) the time differential is positive, else it is

negative.

Imagine you live in the U.S.A., 4 hours difference. Your

receiver's clock is set to UTC (this is what I recommend) and

your PC's clock shows the localtime. Start the WaveGazer with the

following line:

C:> WG 1 lpt1 4 0

Users in the U.K. normally will have both, the PC and the

receiver's clock set to UTC. (at least during wintertime...) They

start the WaveGazer with

C:>WG 1 lpt1 0 0

One case that should not be very frequent but possible: User in

Russia, 5 hours east of Greenwich, PC to localtime, receiver to

localtime +1 (positive = west). We start the WaveGazer with:

C:>WG 1 lpt1 -5 -4

If you want to change one of these parameters, you need to supply

it again on the command line of a subsequent invocation of the

WaveGazer. If you want to change the last parameter, all leading

parameters need to be supplied. In general: leading parameters

are mandatory, trailing parameters are optional.

Once a parameter specified, its value is permanently stored in

the file WG.CNF. All parameters OK, you may invoke the WaveGazer

with only the command name.

HELP:     The WaveGazer starts halfway up and then locks up with

black or other strange colored menu bars!

This may occur if your configuration file WG.CNF is corrupted or

if you start a new release of the WaveGazer, where the format of

this file has changed. Delete the file WG.CNF and restart the

WaveGazer again. Upon exit, a new file WG.CNF will be created and

everything will be OK, after you followed the steps described in

the next chapter again.

User's Guide                The WaveGazer v1.0                 Page 13

----------------------------------------------------------------------

5.3 Tell The WaveGazer about your PC's speed

When you start the WaveGazer successfully for the first time,

there are chances that it won't work OK...

Proceed as follows to have the WaveGazer adapt itself to your

computer:

*    Hit F8 to have the receiver attached / detached several

times. You should see the 'remote' led on the receiver going

on and off.

*    Look at the status line, probably the values read back from

the receiver are completely wrong.

*    Leave the receiver in the 'attached' state and start up the

Bandscan feature. Load a scan from disk (HELLO.SCN comes

with the distribution). Click in the Bandscan window and

drag the marker around. The receiver should now reflect the

state indicated on the computer display. If no proceed the

preceding steps again.

*    If communication from PC to receiver is OK, select the menu

option 'Control -> Calibrate' and wait some seconds for the

popupbox to come up. Acknowledge and exit the WaveGazer.

This stores a time-out value in the file WG.CNF and so it

will be read on the next invocation of the software.

*    Here, everything should be OK., test it by re-invoking the

WaveGazer. The status line should now display the actual

receiver state.

*    Eventually, if the clocks on the RX and the PC differ more

than 3 minutes (after substraction of the defined time

differential), you may choose to synchronize either's clock

to the other's.

If you fail to execute the above steps, the WaveGazer will either

be unable to read back values from the receiver, or it will run

very slowly if no communication is established between the PC and

the receiver.

User's Guide                The WaveGazer v1.0                 Page 14

----------------------------------------------------------------------

6. General Usage

6.1 Windows

The window system is very coarse (see chap 6.7 Restrictions). If

a window is opened, it saves the graphic covered by itself in a

file, which will be read for redrawing the screen if the window

will be closed. That is why the environment variable 'WG_SWP'

needs to be defined. (see chap. 5. Installation)

To close a window, press the button on top of the frame, the one

that displays the name of the window. To close the topmost window

using the keyboard, hit the spacebar.

6.2 Menus

The menus are implemented as you are used to from other

applications, clicking is very intuitive. To access the menu

using keystrokes, press F10 to have the last pulled down menu

pulled down, and then use the arrow keys to stroll through the

menu items. You also may access the menus directly using ALT-

something keystrokes:

*    ALT-C pulls down the "Control" menu

*    ALT-R pulls down the "Receiver" menu

*    ALT-S pulls down the "Scanning" menu

*    ALT-D pulls down the "Database" menu

*    ALT-L pulls down the "Logbook" menu

*    ALT-U pulls down the "Customize" menu

*    ALT-A pulls down the "About..." menu

6.3 Menu Options:

The "Control" menu has 4 options:

*    "Exit"              To quit the WaveGazer

*    "Get Memory"        To see how much unallocated memory

remains. Don't be afraid when the button

tells you that it needs more memory:

This is hardcoded to express my personal

frustration about DOS's 20 bit

addressing mode ;-)

User's Guide                The WaveGazer v1.0                 Page 15

----------------------------------------------------------------------

*    "Calibrate"         To calibrate the communication

parameters

*    "Get Time Diff."    To display the time differences the PC's

and receiver's clocks have respective to

UTC (in minutes)

The "Receiver" menu has 4 options:

*    "Control"           To start the "Receiver Control" window

*    "Memory Editor"     To start the "Memory Editor"

*    "Memory Viewer"     To start the "Memory Viewer"

*    "Attach"/"Detach"   To establish or loose connection to the

receiver

The "Scanning" menu has 2 options:

*    "Bandscan"          To start the "Bandscan" window

*    "Recording"         To start the unattended recording

feature

The "Database" menu has 5 options:

*    "Open Database"     To open a database

*    "Close Database"    To close the currently open database

*    "Open DB Window"    To open the "Database" window

*    "Search"            To open the "Search" window and the

"Database" window if not already opened.

*    "Add Item"          To open the "Add A Record" window and a

database, if none opened at the moment.

The "Logbook" menu has 5 options:

*    "Open Logbook"      To open a logbook

*    "Close Logbook"     To close the currently open logbook

*    "Open LOG Window"   To open the "Database" window

*    "Search"            To open the "Search" window and the

"Database" window if not already opened.

*    "Logging..."        To open the "Logging..." window and a

logbook, if none opened at the moment.

User's Guide                The WaveGazer v1.0                 Page 16

----------------------------------------------------------------------

The "Customize" menu has 4 options:

*    "Colors"            To open the "Customize Color" dialogbox

*    "Printer"           To open a dialogbox used to enter the

escape sequences respective to your

printer.

*    "Save Settings"     To save the current settings to disk

file

*    "Read Settings"     To update the current settings with data

from disk

The "About..." menu has 2 options

*    "WaveGazer"         To get information about the current

version of the WaveGazer

*    "The Author"        To get information about me, my address,

e.g...

6.4 Textfields:

The textfields may be input/output or output only. If textfields

are input/output, they may be edited by means of usual editor

commands (arrow keys, <HOME>, <END>, <DELETE> etc.). Overstrike

mode is not available and you cannot enter a string longer than

the length of the textfield.

Hitting <RETURN> terminates the input, while hitting <TAB>

transfers the input focus to the next input field in the chain.

Clicking in a field transfers input focus in this field, at the

position in the string where clicked.

Doubleclicking in a field erases the field entirely.

6.5 Popupboxes

All popupboxes can be accessed using either the mouse or the

keyboard. While mouse operation is straight forward, keyboard

operation needs some explanation.

*    To get a popupbox disappear, refusing the suggested option,

the equivalent of pressing the "Cancel" button, hit <ESCAPE>

key

*    To acknowledge the suggested option in a one button box, hit

the <RETURN> key

*    If more than one option to choose from, hit the first letter

of the button label to accept this option.

*    Positive options are generally mapped to <RETURN> while the

negation generally is mapped to <ESCAPE>

User's Guide                The WaveGazer v1.0                 Page 17

----------------------------------------------------------------------

6.6 File Selection Boxes

File selection boxes may display up to 6 filenames to choose

from. If more than 6 files match the search criteria, you may

scroll the tiny display using either the keyboard (see later) or

the triangular buttons righthand of the filename display.

Doubleclicking a filename selects the filename directly, while

you would need to hit the second button explicitly if you clicked

the name only once.

Use the following keystrokes to manipulate file selection boxes

from the keyboard:

*    <SPACE>:       To rescan the directory, equivalent to "find"

button

*    <RETURN>:      To accept the selected filename or the one

put in the textline holding the template

*    <ESCAPE>:      To close the file selection box, equivalent

to "cancel" button

*    <Up/DnArrow>:  To select a file

*    i:             To have input focus on the textfield

*    <PgUp/PgDn>:   To page up or down

6.7 Restrictions

The WaveGazer has been built on a user interface toolkit I

developed on my own, as a programming exercise. Whereas most of

the 'widgets' (X11?) are rather well implemented, there are some

drawbacks you need to know if you use the WaveGazer.

*    The windows cannot change position and size!

*    Hidden windows cannot be made visible (put on top of the

stack) without closing the windows hiding them entirely or

partially.

*    In the file selection box, the files are not sorted

alphabetically, rather they appear in sequence as they are

on the disk drive. I should correct this in a future release

*    The accelerator keys (for keyboard operation) are case

sensitive. So they won't work (or work in unexpected manner)

if you have the caps-lock key activated!

User's Guide                The WaveGazer v1.0                 Page 18

----------------------------------------------------------------------

7. General Receiver Control

At any time, you may control all computer controllable features

from the keyboard of the PC, and you may monitor the current

receiver status in the status line at the bottom of the display.

This control is implemented using some keystrokes on the function

key row and on the numerical keypad.

Controllable features:

*    Reception mode (modulation)        key: F1

*    IF filter                          key: F2

*    Automatic gain control AGC         key: F3

*    20 dB attenuator                   key: F4

*    Front-end circuitry bypass         key: F5

*    Noiseblankers                      key: F6

*    Receiver attach/detach             key: F8

*    Freq. step for tuning up/down      key: 7/1

*    Frequency up/down                  key: 6/4

*    Bandwidth control up/down          key: '+'/'-'

*    Passband shift left/right          key: '/'/'*'

These keys should be easy to retain (look at the order of the

items in the status line) and are accessible all the time the

computer has the connection to the receiver established.

The status line is meant to always reflect the actual state of

the receiver, except if the receiver is disconnected and tuned

manually.

User's Guide                The WaveGazer v1.0                 Page 19

----------------------------------------------------------------------

8. Receiver Control Window

The receiver control window allows you to control the receiver

nearly completely from the computer. All functions may be

accessed, except the analogous tuning:

*    AF gain

*    RF gain

*    Noiseblanker depth

*    Notch filter

*    The squelch level

This window is maybe not very useful, since tuning the receiver

directly may be more convenient.

8.1 Controllable Features

*    The -20 dB attenuator

*    The front-end circuit bypass

*    The noiseblankers (on/off of either one)

*    The AGC time constant

*    The IF-filter

*    The frequency, via keyboard and via the VFO knob

*    The tuning steps (not the ones of the receiver itself!)

*    The reception mode (modulation)

*    The passband shift

*    The variable bandwidth

8.2 Attaching / Detaching The Receiver To The PC

In the middle of the receiver control window, there is a toggle

button with associated textfield. When the window is called, the

textfield displays "Computer", what means that the receiver is

attached to the PC. Clicking in the toggle button makes the

textfield display "Manual" and unlocks the receiver, enabling you

to manipulate it manually!

User's Guide                The WaveGazer v1.0                 Page 20

----------------------------------------------------------------------

Keep in mind, that every time you attach the receiver to the PC,

the PC's display will be updated with the values found on the

NRD. The WaveGazer records all possible changes on the receiver

internally; so, even if the receiver control window is not

displayed, the program knows about the receiver's state.

In the menu called "Receiver" you find an option called either

"attach" or "detach". Use this option to attach or detach the

receiver without passing through the receiver control window.

8.3 Operation

The toggle button labeled "-20 dB" switches the attenuator on the

receiver.

The toggle button called "Bypass" switches the front end tuning

circuit on the NRD. The 535's user manual tells you, that this

feature should only be used under difficult reception conditions.

There is no way to inquire the state of the bypass from the

computer, so don't be puzzled if the PC display does not match

the receiver's state in that matter!

The toggles labeled "Noiseblanker" switch the two noiseblankers

on and off. The depth of the blanking has to be tuned manually on

the radio.

The AGC time constant can be switched with the radiobutton

labeled AGC (you might have found this one by your own, ...)

The filter used is switched with the buttons labeled "Filter".

Every time you switch filters, the textfield to the right of

the scrollbar called "Bandwidth" will display the name of the

selected filter, e.g. "narr" for the narrow filter. If this

textfield displays such text instead of a numeric value, then the

bandwidth control feature is off!

You may set the reception frequency in two different ways: With

the VFO knob or directly via keyboard by editing the frequency

display. Using the VFO knob may be not so handy, it requires some

get used to. All you have to do is clicking in the red button on

the knob and drag it (mouse still pressed) to the position you

want. That means you need to execute some circular movements with

the mouse. Try it, and you will agree that manual tuning is more

comfortable yet.

If you need to do some very fine tuning, this feature could be

useful, since it is more precise than the NRD's well balanced

tuning knob. In addition, the frequency display shows the very

exact frequency, the 1 Hz position is displayed.

This display is in fact an editable textfield. Click in it, the

cursor will blink and you may enter the desired frequency. Enter

the frequency in kHz, using a dot or a comma as the decimal

point. Once the frequency entered, click on the red button on the

VFO (or double <return> on the keyboard), and the radio will be

tuned to this frequency.

User's Guide                The WaveGazer v1.0                 Page 21

----------------------------------------------------------------------

In the VFO knob, there is a feature that allows you to select the

tuning steps used for VFO tuning. (these are steps employed

internally by the WaveGazer, these are not the tuning steps of

the receiver...!) Clicking in the up or down buttons alters the

step to the next value from the sequence of 1, 10, 100, 1000,

5000 and 10000 Hz. When the receiver control window is called,

this value is set to the last selected value.

Be aware that if you switch this value up, the display and the

receiver will align their current frequency to the next lower

value on this boundary. (e.g., switching from 10 Hz to 100 Hz

will switch the radio to 7145.500 kHz if before it was tuned to

7145.560 kHz).

The reception mode may be switched with the radiobutton called

"Mode", as you would do it on the NRD itself.

A scrollbar labeled "Bandwidth" allows you to vary the bandwidth

of the used filter from 500 to 5500 Hz, provided you own a 'new'

NRD 535, which is equipped with the new BWC feature.

In the textfield to the right of the scrollbar, the actual

bandwidth is displayed. From 500 up to 2100 Hz, the 'int' filter

is used, from 2100 to 5500 Hz, the 'wide' one. (This might

present some difficulties if the factory installed filters have

been changed on your receiver)

Filter switching is done automatically, but you will hear a

difference in the audio.

If a filter is directly chosen via the "Filter" buttons, the

variable bandwidth control is switched off, as you may verify on

the receiver's display.

The other scrollbar, labeled "Passband" is used to control the

passband shift, the associated textfield shows its actual value.

8.4 Keystrokes

*    a:        Toggles the attenuator

*    B/b:      Varies filter bandwidth (larger/smaller)

*    c:        Toggles the computer or manual interface

*    f:        Selects the filter (cyclic)

*    g:        Selects the AGC setting (cyclic)

*    h:        Input of frequency ('hertz'), double <RETURN> to

set receiver to this freq

*    I/i:      Rises/lowers the VFO's increment

*    m:        Selects the mode (cyclic)

User's Guide                The WaveGazer v1.0                 Page 22

----------------------------------------------------------------------

*    N/n:      Toggles either noiseblanker 1 or 2

*    p:        Toggles front end bypass

*    S/s:      Varies passband shift (up/down)

*    V/v:      Rises/lowers the frequency of RX

User's Guide                The WaveGazer v1.0                 Page 23

----------------------------------------------------------------------

9. The Memory Editor

The "Memory Editor" is used to manage the 535's 200 memory

channels. On startup of the WaveGazer, the memory channels are

read from the receiver to the computer's memory, which may take

some seconds, but it is done in the background, that means

invisible for you. (You may notice it only if you start the

Memory Editor or the Memory Viewer shortly after program

startup.)

9.1 Description

*    Manage all 200 memory channels, one by one or all together

*    Store user defined settings in a memory channel

*    Dump the current receiver settings in a memory channel

*    Erase memory channels

*    Tune receiver to a selected memory channel

*    Save all 200 memory channels to a disk file

*    Load 200 memory channels from a disk file

9.2 Operation

The Memory Editor consists mainly of two zones: One displaying

the numbers of all 200 memory channels and one showing the

contents of the selected channel.

The large area displaying the numbers is 'clickable' with the

mouse. It displays the numbers of all non-vacant channels, that

means all channels containing a valid definition.

*    The characteristics of the currently selected channel are

displayed in the smaller area to the right of the main

display.

*    Doubleclicking a channel tunes the receiver to this channel.

*    Clicking in one channel and dragging the mouse with MB1 hold

down in another, means moving the contents of the first

channel into the other (The original channel will be

erased). If the target channel is not empty, a warning box

will be displayed.

The buttons allow you to further manage the memory channels:

The one labeled "Define" loads the currently selected channel

with the characteristics in the editor box. This allows you to

change a channel manually.

User's Guide                The WaveGazer v1.0                 Page 24

----------------------------------------------------------------------

To do this, select a channel in in the large grid, then fill in

the settings in the smaller box and click the "Define" button

Clicking "Tune" tunes the NRD to the selected memory channel (the

same as doubleclicking a channel number.)

"Load" and "Save" are used to load all channels from a file or to

dump them in a file for later reloading. This allows you to have

a virtually unlimited number of memories.

The "Clear" button erases the currently selected memory channel

"Print" prints out all channels, their number and settings. This

is not yet implemented, (sorry)

"SetRX" is used to load the selected channel with the current

receiver settings.

"Update" allows you to re-read the 200 channels from the

receiver, in case something went wrong at any time (e.g. receiver

not attached at program startup).

The smaller box is to edit one channel at a time. Of course, you

cannot change the channel number, this fields displays the number

of the selected channel.

9.3 Keystrokes

*    arrow-keys:    To move in the grid containing the memory

numbers

*    a:             To toggle attenuator toggle in editor

*    f:             To cycle filter settings in editor

*    g:             To cycle AGC settings in editor

*    i:             Start input in frequency field in editor

*    l:             To load a memory file from disk

*    m:             To cycle mode setting in editor

*    p:             Print (not yet implemented)

*    r:             To dump the current receiver settings in the

selected memory channel

*    s:             Save receiver memory to disk file

User's Guide                The WaveGazer v1.0                 Page 25

----------------------------------------------------------------------

*    t:             Tune receiver to selected channel

*    u:             Update display from receiver

*    <INSERT>:      Set selected channel with settings from

editor

*    <DELETE>:      Clear selected memory channel

User's Guide                The WaveGazer v1.0                 Page 26

----------------------------------------------------------------------

10. The Memory Viewer

10.1 Operation

The Memory Viewer is most likely used in conjunction with the

Memory Editor. As its name implies, it only serves to view the

memory channels.

It allows you to view all the memory channels with their

contents, 33 at a time. The scrollbar permits you to adjust the

desired range of channel numbers.

If used with the Memory Editor, you may prefer selecting a chan-

nel using the Memory Viewer, there is a certain amount of

interaction between these two windows.

*    Channels selected in the Memory Viewer are also selected in

the Memory Editor

*    Upon invocation, the Memory Viewer displays the contents of

the channel numbers around the one selected in the Memory

Editor, or channel numbers from zero if the Memory Editor is

not present.

Doubleclicking on a line displaying a channel tunes the radio to

this channel, even if the channel is vacant.

10.2 Keystrokes

*    <PgUP/PgDN>:        page up/down

*    8/2:                select channel, (scroll)

User's Guide                The WaveGazer v1.0                 Page 27

----------------------------------------------------------------------

11. The Bandscan

This exciting feature allows you to scan bands in 555 steps

(don't ask why 555 steps, it was because the window, once

designed had 555 steps to go...!). Of course, you also can define

less than these 555 steps but this odd number is the maximum

number of steps available!

11.1 Description

*    Scan from a fixed start frequency to a fixed stop frequency,

the computer calculates the step.

*    Scan from a fixed start frequency using a fixed step, the

computer calculates the end frequency.

*    Scan from a fixed start frequency, using a fixed step, to a

fixed end frequency, provided the step is small enough to

allow the end frequency to be on the display! (if start and

stop frequency are defined as well as the step, the step

always has priority over the end frequency, in case the step

would be too big to allow the end frequency being on the

display!)

*    Monitor one single frequency over time (for observing

fading)

*    Select bands to be scanned from list of user defined bands

*    Scans may consist of only one sweep (a sweep being one pass

from start to the end) or of a undefined number of sweeps,

if the toggle "Endless" is activated.

*    A running scan may always be stopped with the "Stop" button

and later resumed with the "Resume" button.

*    The "Endless" toggle may be toggled every time, its state is

checked at the end of the current sweep. (that means, a

previously endless scan will end at the end of its current

sweep, if the state of the toggle has been changed.)

*    In the case of an endless scan, the toggles labeled

"Current" and "Average" allow you to select which values the

display should show. The average value is the one taken over

all previous runs!

*    A whole scan may be saved in a file ("Save"), storing also

the current scanposition. That means, a stored scan may be

resumed after being reloaded from disk!

*    Use the "Load" button to relaod a saved scan

*    Define the number of samples of a signal at a given

frequency taken with the scrollbar labeled "Samples".

User's Guide                The WaveGazer v1.0                 Page 28

----------------------------------------------------------------------

*    Allow the AGC to settle during a time period defined with

the other scrollbar, the one labeled "AGC Settle Time". This

does not affect the AGC decay time of the receiver, it only

is a value in microseconds, the WaveGazer waits before it

samples the S-meter on the next frequency.

*    Finally, the whole display may be cleared by means of the

"Clear" button!

11.2 Operation

The textfields displaying "Bandclass" and "Band" are used to

select a predefined band for scanning. The first is used to

select a certain class of bands, e.g.. 'Domestic Broadcast',

'International Broadcast', 'Marine Coast Stations' and so on;

while the second is used to select a particular band in this

bandclass. (e.g. the 49 meter band).

As soon as you selected a band, its limits and eventual a

scanstep will be displayed in the two textfields to the left and

right of the buttons. (The step in the field labeled "Step"). For

further information about how to define these predefined bands,

see chap. 12, "The Predefined Bands."

Next you see a scrollbar labeled "nr. Samples". Use this

scrollbar is to fix the number of times the S-meter should be

sampled for one scanstep. For fast scans select the minimum (1),

but for precise scans select a higher number.

The scrollbar labeled "AGC settle" serves to adjust the time in

microseconds the scan will wait before stepping to the next

frequency. Every AGC unit in every receiver has some decay time,

often just called 'fast' and 'slow'. So you may choose how much

time you will allow the AGC circuitry to settle before sampling

the next frequency. This is also useful for precise scans, allow

more time than for fast scans.

You will see that low values in this field will result in

asymmetric graphs. Do as you feel!

The following textfield indicates the step in Hz the scanner will

step further. You may fill in this field interactively, provided

you supply a valid value.

The field labeled "Freq" is output only, it indicates the

frequency the receiver is tuned to.

The togglebutton called "Endless" is to indicate if you want to

stop at the end of the running scan or if the scanner should run

endlessly.

Toggles called "Average" and "Current determine which value is

displayed, the average value for a frequency and/or the value

taken in the current sweep. Of course, for '1 sweep scans', both

values are the same. The value represented as a solid line is the

average value over all sweeps, the dots show the last value read.

User's Guide                The WaveGazer v1.0                 Page 29

----------------------------------------------------------------------

In the lower row of controls, we have the two textfields, one

left and one right. The right one holds the start frequency while

the other holds the end frequency of the scan. These fields can

be edited, they will be read at the beginning or on resuming of a

scan.

The first button ("Start") is to start a scan at the start

frequency. After clicking the button, the scanner remains for 1

second on the startfrequency to allow the AGC to settle. After

the start of a scan, you may click in the graph, the screen will

display a marker and the receiver will be set to the pointed

frequency.

The "Stop" button is (of course) to stop the running scan. A

stopped scan can be resumed with the "Resume" button or restarted

with "Start".

The buttons called "Save" and "Load" serve to store a scan in a

file or to reload one on the display. A reloaded scan may be

resumed where it has been stopped.

The "Clear" button is to erase the display. Sorry for the

slowness.

Once a scan graph begun or displayed, you can click and drag with

the mouse in the display and the receiver will be tuned to the

pointed frequency. I think this can be very handy to monitor

activity on a certain band, you do not need to remember

frequencies but the shape of the peaks drawn on the screen...

Most control features are active during a scan, so you can

dynamically change the number of samples, the AGC settle time and

toggle the toggle buttons. However, you cannot change the start

and end frequencies and the step.

Also, activating some control functions will stop the running

scan: selecting a new band, restarting a scan and

saving/redisplaying or clearing a scan.

The Receiver Control window may be active during a scan, so you

can change the current receiver setting easily.

This scan feature can be used also to visualize the characteris-

tics of the IF-filter used: just scan a small portion of a the

spectrum, lets say around a stable carrier somewhere, and choose

a precise scan. The graphs drawn on the screen show approximately

the Q-factor and the width of the filters. How to interpret this,

is up to you ...;-)

In general, best results are obtained with a small filter and a

fast AGC decay time. Unfortunately, the 'fast' position on the

NRD is still not fast enough to have the fast scan graphs look

nicely...

User's Guide                The WaveGazer v1.0                 Page 30

----------------------------------------------------------------------

If you want to visualize fading of an only one transmission, just

fill in the field for the start frequency with the frequency you

want to monitor and leave the fields for endfrequency and step

blank. Hit the "Start" button and the graph will show the fading.

I'm sure you will soon find your own scanning preferences.

11.3 Keystrokes

*    <ALT>a:   Toggles "Average" toggle

*    <ALT>c:   Toggles "Current" toggle

*    <ALT>e:   Toggles "Endless" toggle

*    <ALT>i:   Starts textfield input in "Step" field

*    <ALT>g:   Starts scanning (button start, 'go')

*    <ALT>l:   Loads a file from disk

*    <ALT>r:   Resumes stopped scanning

*    <ALT>s:   Saves a file to disk

*    L/l:      Rises/lowers the sample number

*    T/t:      Rises/lowers the AGC settle time

*    <DELETE>: Clears the display

*    <END>:    Stops scanning

User's Guide                The WaveGazer v1.0                 Page 31

----------------------------------------------------------------------

12. The Predefined Bands

*** in this chapter, a feature called Channelscan is mentioned.

** this is something I'm currently working on, but that is not

* yet ready so don't worry about it so far.

The Bandscan and Channelscan windows contain both a feature to

recall predefined bands with their limits and scansteps. In the

directory pointed by the environment variable 'WG_FIL' is a file

called BANDS.DEF which is a formatted text file. This chapter

describes the format of this text file.

12.1 Theory

You can have two kinds of machine readable lines in it: One to

define a band class and one to define a band in the previously

defined band class. The order of the lines is the same in which

they will be displayed in the selection widgets, so it can be

considered as important.

To define a new class, e.g. Tropical Broadcast, a line beginning

with "cl" and a classname must be inserted. e.g.:

cl Broadcasting          or

cl Tropical_Bands

Notice that the name must be one word (e.g. no spaces), so '_'

signs may be used to replace spaces. (I know this is not nice,

but it is easier to program...)

All following lines beginning with "bd" are to define a band in

this class.

The format is more complicated than the one of the classes:

bd start end chstep chnr scstep name

where 'start' stands for the start frequency in kHz, 'end' for

the end frequency in kHz and 'chstep' for the channel spacing in

kHz.

'chnr' holds an eventual channel number (can be used e.g. on CB

or marine bands, where channels are numbered and evenly spaced),

'scstep' is the step the bandscanner will employ (in Hz), it is

not used in the channel scanner.

Of these fields, only "bd",'start' and 'name' are really

mandatory. others can be replaced by an asterisk (*) and so they

will be ignored.

For bands used in the bandscanner, 'chstep' and 'chnr' are not

used, and either 'scstep' or 'end' are optional.

User's Guide                The WaveGazer v1.0                 Page 32

----------------------------------------------------------------------

Bands used in the channel scanner do not need the 'scstep' field

and either 'end' or 'chstep' are optional.

The 'chnr' field can be used in the channelscanner, but is not

required.

12.2 Examples

Some examples of lines defining bands:

bd 5950 6200 5 * * 49_meter

This line defines the 49-meter band for use in both the band- and

the channel scanner. In the bandscanner, the scanstep ('scstep')

will be calculated at scanstart and will certainly be something

weird and odd. ((6200-5950) / 555 = 450 Hz).

To have the scanstep look better, you may force it to 500 Hz in

the line:

bd 5950 6200 5 * 500 49_meter

For marine bands, a line may look like this:

bd 8704 8815 3 800 200 8_MHz

This line defined the 8_MHz coast stations band, which spreads

from 8707 to 8812 kHz, with channels numbered from 801 onwards.

'Start' has the value of 8704, which is the real start freq. of

this band minus the channel spacing (8707 -3). This is to prevent

that the first channel never will be scanned. So the channel

number ('chnr') field must contain the lowest channel number - 1

= 800.

The step used by the bandscanner is 200 Hz. It is important to

use scansteps that allow you to point the exact frequency of a

transmission, especially if SSB transmission will be tuned! The

step used in the channelscanner is 3 kHz, which is the real

spacing in the marine bands.

"bd" lines following a "cl" line define all members of the class

defined by this "cl" line.

Lines not beginning with either "cl" or "bd" are ignored, so feel

free to add comments to the file.

You may have a maximum of 16 classes containing each a maximum of

16 bands. (I think this is enough...)

User's Guide                The WaveGazer v1.0                 Page 33

----------------------------------------------------------------------

13. Automatic Recording

13.1 Features

This window allows you to schedule up to eight transmissions you

don't want to miss or want to record. The timer relay on the '535

will be switched at a given times, with the receiver tuned to the

wanted frequencies.

*    Define from one to eight transmissions for recording

*    Switches receiver off, during long waiting periods

*    Allows user defined period for receiver preheating

*    User may switch on/off receiver from computer (is this

really useful?)

*    User may switch on/off tape (time relay) from computer

*    Upon start of recording, eventual user errors are flagged

and signaled.

*    User definitions are saved for later re-use. (in file

RECORD.DAT)

13.2 Operation

The input fields are organized in lines and columns. One line

represents one scheduled transmission recording. The column

labeled "Mem." holds the memory channel which will be selected

during recording; the two following hold the start and the end

time of the recording.

You do not need to respect a particular order of these entries;

upon start, the software arranges the display for you, in the way

that all lines are sorted, with the starttime as sortkey.

Before, the WaveGazer checks if a line is incomplete, if two or

more lines do have overlapping times or if more than one line has

an endtime that is earlier than the starttime.

It is perfectly legal to have one line holding a starttime which

is later than the endtime: The WaveGazer assumes that this is a

transmission spanning midnight, so this line will be the last

listed in the sorted columns.

A small arrow pointing to the line number shows which line

currently is processed, while the status field displays the

action the recorder is doing:

*    inactive       "Start" button has not yet been pressed to go

in either waiting, tuning, preheating or

recording

User's Guide                The WaveGazer v1.0                 Page 34

----------------------------------------------------------------------

*    waiting        Wait for the receiver to switch on for

preheating

*    tuning         Tune the switched on receiver to the desired

channel (displayed during a very short time)

*    preheating     Wait for recorder to switch on, while

receiver is already switched on.

*    recording      Receiver and recorder are switched on, the

latter recording if you set it up well...

Pressing the "Start" button activates the format checking of the

input fields; and then, if all fields OK, the software goes in a

loop comparing regularly the current time to the times in the

different fields, to eventually switch on or off the connected

devices.

"Stop" may be pressed anytime. This terminates the previously

entered loop and switches the tape off (if not already done) and

the receiver back on if it was switched off.

The scrollbar labeled "Preheat" is used to define the period in

minutes used to warm up the receiver to achieve maximum

stability.

13.3 Keystrokes

*    P/p:      Rise/lower the preheat time

*    r:        Switches receiver on/off

*    s:        Starts recording feature (not recording itself,

but the timer function...)

*    t:        Switches timer relay on/off

*    <END>:    Stops the recording feature

User's Guide                The WaveGazer v1.0                 Page 35

----------------------------------------------------------------------

14. The Database

14.1 Concepts

At any given time you may have no more than one database open,

but the number of databases present on the hard disk is limited

only by the disk space. The number of entries in a database is

also unlimited, whenever the performance will decrease using

larger databases. The limiting factors for the size are:

*    Disk space

*    Memory

*    CPU performance

A database may either hold a database or a a logbook. The

internal representation of both is the same, but in either, some

fields are not used. A database/logbook consists of 4 files:

*    dbname.db or dbname.lb:  The database or logbook itself

*    dbname.bci:              The index for broadcaster name

*    dbname.fri:              The index for frequency

*    dbmame.tmi:              The index for time

Opening a database or a logbook is done using the respective menu

option:

*    'Database->Open Database'     To open a database

*    'Logbook->Open Logbook'       To open a logbook

Once a database or a logbook open you also may use the "Open"

button in the database window, but you are restricted to stay

with the kind of database you already opened: database if

database is open, logbook if logbook is open.

14.2 Usage Of Databases

*    A database may hold a database or a logbook.

*    A database entry consists of: Broadcaster name, frequency,

start time, endtime, country, language, relay/transmitter

site, a comment and all storeable receiver settings.

*    A logbook entry consists of: Broadcaster name, frequency,

time, date, country, language, SINPO, relay/transmitter

site, a comment and all storeable receiver settings.

*    Sort records after sortkeys: Broadcaster, Time, Frequency

User's Guide                The WaveGazer v1.0                 Page 36

----------------------------------------------------------------------

*    Define search criteria based on broadcaster name (case

insensitive, wildcarding), time, country and language.

*    Frequency search criteria may be 'equal given frequency',

'smaller than given frequency', 'greater than given

frequency' or 'between two given frequencies'.

*    If a record is added, all possible receiver settings may be

fetched in the record immediately, to speed up logging.

All the above features are general features, interfaced through

the windows called 'Database' and 'Search'. So please refer to

the documentation for these windows for further information.

14.3 Creating a new Database

To create a new database or logbook (one that does not yet

exist), click in menu "Database->Open Database" or "Logbook->Open

Logbook", to have the file selection box displayed. In this box,

fill in the name of the database you want to open and click

"Open". The new database or logbook with the desired name will be

created and opened for you. (Please respect DOS's file naming

convention...)

14.4 Tracking Mode

An important feature is called 'Tracking Mode' which allows the

user to tune the receiver manually while the database is looked

up for records matching the current receiver frequency. For more

information about this feature, please refer to chapter 15.

14.5 Database Cache

The WaveGazer keeps up to 27*20 database records in a local cache

and always tries to keep the currently needed portion of the

database in the cache. Of course, this will not always be the

case, so  you sometimes will feel the application slow down while

bringing the cache up-to-date.

The 'Database' window shows in its status line the state of the

cache; if it is displaying strings different from "all records

cached", "cache 100% filled" or "End of database reached", the

cache is being updated.

Remember:  Share must be installed for database use

User's Guide                The WaveGazer v1.0                 Page 37

----------------------------------------------------------------------

15. The Database Window

15.1 Description

The database window displays the database entries on the screen.

You may view the entries, tune the receiver to a selected entry,

print them out and delete or edit entries. A function also allows

you to add entries interactively.

You may:

*    View 20 entries at a time and scroll up or down, page per

page

*    Select an entry for editing, tuning, printing or deletion

*    Print out all entries or on-screen entries only, in either

short or complete format

*    Tune the receiver immediately to a selected entry

*    Open/close databases or logbooks

*    Load all records which correspond to a user defined search

criteria to the receivers memory, and/or print out a list of

these entries.

*    Load your receiver's memory and have an up-to-date list of

the memory occupation!

*    Load all records corresponding to the search criteria to a

file which can be loaded to the receiver's memories later.

*    Activate the 'Tracking Mode'. (see chap. 16.)

*    Read out the actual state of the cache

*    See the name of the open database.

15.2 Operation

The 'window in the window' is a 'mouse clickable' area and will

display the database entries. The 'Database' window does not work

without the 'Search' window and vice versa, so some reference

will be made to the search window.

To have the records of an open database displayed, click the

"Search" button in the 'Search' window. All records matching the

search criteria (see 'Search' window...) will be displayed, top

down.

If more than one screen full should be displayed, the buttons

labeled "prev Pg" and "next Pg" can be used to page up and down.

User's Guide                The WaveGazer v1.0                 Page 38

----------------------------------------------------------------------

Records are selected by clicking. Selected records can be:

*    Deleted, by clicking on the "delete" button

*    Edited, by pressing the "edit" button (see chap. 17., Adding

/ Editing a record)

*    Tuned, which means the settings found in the selected record

will be dumped to the receiver.

Doubleclicking a record also tunes the receiver to the selected

record.

Pressing the "Print" button displays a popup box asking whether

you want to print all records corresponding to the search

criteria or just the ones displayed on the screen. Further, you

may choose the format of the print: If short or complete format.

Short format requires a carriage width of 80 characters, while

the complete format requires 132 characters. (see the chapter

about printer customization and the users manual of your printer

on how to get it spit out these formats...)

The toggle button labeled "Tracking Mode" activates the tracking

mode. The receiver will be unlocked and manually tunable, while

still linked to the PC. The software will follow all changes

applied to the receiver and constantly update the RX status line.

See the next chapter for more information about tracking mode

operation.

The textline situated just over the button row should always show

the status of the database cache.

15.3 Keystrokes

*    a:             Adds a record (opens "add a record" or

"Logging..." window)

*    c:             Closes the open database or logbook

*    e:             Edits the selected record

*    l:             Pops up the memory loader dialogbox. (see

chapter 15.4)

*    m:             Activates tracking mode

*    o:             Opens a new database (closes the previously

opened one)

*    p:             Print

*    t:             Tunes receiver to the selected entry

User's Guide                The WaveGazer v1.0                 Page 39

----------------------------------------------------------------------

*    <DELETE>:      Deletes the currently selected entry

*    <PgUp>:        Backwards one page

*    <PgDn>:        Forward one page

*    <Up/DnArrow>:  To select the entries

15.4 Loading Database Entries To Memory

The button labeled "Load RX" calls a little dialog box, which

allows you to load database records matching the search criteria

in the receivers memory. You may choose to:

*    Load the records matching the search criteria in receiver

memory, starting at a defined channel number and ending at a

defined channel number. If less records found than channels

foreseen for writing, the remaining channels will be left

unchanged. If more records found than channels available,

not all records will be loaded.

*    Write the found records to a file, which can be reloaded

later into receiver's memory

*    Print out a list of all memory channels, their number and

description. Might be useful for a DXpedidion or so...

This dialogbox disappears automatically if all the work is done

15.5 Keystrokes For Memory Loader Window

*    i:        Get input focus on first textfield in window, use

<TAB> to move further

*    l:        Toggles 'Load To Memory" toggle

*    p:        Toggles 'Print' toggle

*    s:        Toggles 'Save To File' toggle

*    <RETURN>:   Equivalent to hit 'Load' button

*    <ESCAPE>:   Equivalent to hit 'Cancel' button

User's Guide                The WaveGazer v1.0                 Page 40

----------------------------------------------------------------------

16. Tracking Mode

16.1 Purpose

The so called tracking mode lets the user tune the receiver as if

it were not connected to the computer, whereas the PC continually

reads the receiver's status and tries to find a match in the

currently open database.

Since you rarely will be tuned to the exact frequency of a

database entry, the WaveGazer highlights all records containing a

frequency in between two values, which are calculated as

receiverfreq. +/- threshold.

The value of the threshold value is user configurable between 0

and 6000 Hz.

It is perfectly possible to have more than one entry highlighted

at once, depending on how many records are registered for the

same frequency and on how big the threshold value is set.

16.2 Operation

Activate the tracking mode by toggling the foreseen toggle in the

'Database' window. A tiny window will pop up (If not already

here) and show you a scrollbar which serves to adjust the

threshold value. You may alter this value at any time!

To really get the WaveGazer in tracking mode, some conditions

need to be met:

*    A database needs to be opened

*    The database needs to contain valid records

*    The database needs to be sorted for frequency

*    The database needs to be cached entirely or the cache needs

to be entirely filled.

If one of these criteria is not met, a popupbox will appear,

trying to explain why no go...

To leave tracking mode, several methods may be successful:

*    Toggle the toggle button back (you guessed it!)

*    Close the tiny window labeled 'Tracking Mode'

*    Hit the "Search" button in the 'Search' window (re-display

the database)

*    Page up or down in the display, using the PgUp / PgDn

buttons or keys.

User's Guide                The WaveGazer v1.0                 Page 41

----------------------------------------------------------------------

*    Open any Window that partially or entirely hides the

'Database' window.

*    Close the 'Database' window

While in tracking mode, you still may doubleclick a displayed

record to dump its settings to the receiver's VFO.

The function of the toggles "Match Mode" and "Match Time" are not

yet implemented, which means that all entries matching a certain

frequency are highlighted, regardless of other data in the

records.

16.3 Keystrokes

<ALT>m:        Toggles "Match Mode"

<ALT>s:        Toggles "Match Time"

<Up/DnArrow>:  Rises / lowers the tolerance value

User's Guide                The WaveGazer v1.0                 Page 42

----------------------------------------------------------------------

17. Search Window

17.1 Concepts

The "Search" window is used in conjunction with the "Database"

window, it cannot be used without it.

This window is mainly to define the search criteria and to

initiate database searching.

The search criteria may contain:

*    The broadcaster's name, with wildcards at the beginning and

the end of the string

*    Frequency to be matched exactly

*    Frequency to be smaller or greater than

*    Two frequencies to be in between

*    Two times (to be in between)

*    A known language

*    A country code (ITU abbreviation)

All these criteria may be combined, if they are not exclusive as

the different modes for the frequency are.

The search may be sorted based on three search keys:

*    Broadcasters name

*    Frequency

*    (Start) Time

17.2 Operation

To start searching, the database window must be active, that

means on the screen and not partially hidden by another window.

Upon pressing the "search" button, the WaveGazer begins

displaying the found records in the database window. In case you

only want to select some records matching a certain criteria, you

will need to fill in the textfields in the search window:

*    The broadcaster name may begin and end with an asterisk,

interpreted as zero-to-n character wildcards. Asterisks in

the middle of the word are interpreted as such. The

broadcaster name is compared case insensitive to the one in

the record, so case is not an issue.

User's Guide                The WaveGazer v1.0                 Page 43

----------------------------------------------------------------------

*    The frequency may be used in different ways as criteria.

If the selected button in the button line below the fields

for the frequency is "=f1", only records matching exactly

the frequency labeled "Freq 1" are selected. Since this will

not be useful very often, you rather will use the selection

">f1" which selects all records with a frequency higher than

the one in the "Freq 1" field. For all with frequency lower

than the one in this field, use the "<f1" option. If a

frequency is entered in the Field called "Freq 2", it will

be ignored.

It will be used if you select the fourth option for

frequency interpretation, "f1>f2". This mode selects all

records whose frequency is between these two values. Be sure

to define "Freq 2" and to define it larger than "Freq 1".

*    Searching for records corresponding to a given time is done

different depending on if you are searching a database or in

a logbook:

For databases, define one time: the start time only. The

search algorithm will select all records whose start time

falls before the given time and whose end time falls after

it. For logbooks, you need to specify the start and the end

time, the WaveGazer will select all logs logged in between

these two times.

*    If you use the language or the country as a search criteria,

you must supply the exact string (except for case) in the

corresponding fields. If no field is filled in, all records

will be selected, as the search criteria is null.

*    The 'Relay' field is not used...

Pressing the "Reset" button will erase all fields.

Pressing the "Search" button initiates searching through the

database.

17.3 Keystrokes

*    f:        Selects frequency search mode

*    i:        Starts textfield input in broadcaster field

*    k:        Selects the sort key

*    r:        Reset

*    s:        Search

User's Guide                The WaveGazer v1.0                 Page 44

----------------------------------------------------------------------

18. Adding / Editing A Record

18.1 Concepts

This window is 'fill in' form to define a record. It is also used

to edit an existing record. The 'add' functionality is called

from either a menu option or the database window, while the

'edit' function is called only from the database window. The

fields you need to fill in are:

*    The broadcasters name

*    The start and end time for database entries or just one time

for logbook entries

*    The language

*    The country code (ITU)

*    The SINPO code in case of a logbook entry

*    The date in case of a logbook entry

*    All storeable receiver settings

Optional fields comprise:

*    The relay / transmitter site

*    A comment

18.2 Operation

Pressing the button labeled "From RX" tells the WaveGazer to

fetch the current receiver settings and update the display with

it:

*    Time

*    Date (if logbook; from PC)

*    Frequency

*    All receiver settings

The button called "Reset" erases all fields for new fill in.

There is a small toggle button called "Cnv Brc" at the bottom of

the window. You may see that when you add a record, the format of

the fields change when you hit the "add" button. This is because

some internal format checking and conversion is done, to check if

a value is out of range and to provide a more or less uniform

look of the records, once added. If the toggle mentioned above is

active, the broadcaster's name will be converted, capitalized.

User's Guide                The WaveGazer v1.0                 Page 45

----------------------------------------------------------------------

This is normally adequate, e.g. if you write 'swiSS radio

INTernational' this will be converted to 'Swiss Radio

International'. But if you are lazy, and SRI is enough for you,

'SRI' will be converted to 'Sri', which is as badly readable as

'Bbc'. In these and similar cases, be sure the toggle is

deactivated before adding the record.

To finally add the record to the database, hit the "add" button.

(who guessed?).

Be careful:    Hitting the "add" button more than once on the

same entry will result in multiple entries of the

same type. No checking is performed on that!

18.3 Editing A Record

Similar thoughts if you edit a record: Editing a record opens the

'Add A Record' window with already filled in fields. These fields

may be modified (or not), and the modified entry will be added,

after the original entry has been removed from the database.

So this 'add' did not actually add the record but exchanged one

for another. If now you hit "add" again, the record in the window

will be added again, but no record will be removed anymore. In

this case you added one more record, probably one that already

existed.

If you want to edit a record (exchange one for another) you

explicitly need to call the "Add A Record" window through the

edit button in the database window, otherwise the new record will

be added but no record will be deleted.

18.4 Keystrokes

*    <ALT>a:        Toggles the toggle for the -20dB attenuator

*    <ALT>g:        Cycles AGC buttons

*    <ALT>f:        Cycles filter buttons

*    <ALT>i:        Starts input in topmost field

*    <ALT>m:        Cycles mode buttons

*    <ALT>r:        Fetches all possible receiver data to fill in

form (same as "fromRX" button)

*    <DELETE>:      Reset

*    <INSERT>:      Add the record

User's Guide                The WaveGazer v1.0                 Page 46

----------------------------------------------------------------------

19. Customization

19.1 Customization Of The WaveGazer

There is no explicit feature to customize the application itself.

But every user preference you set, e.g.. receiver preheating time

in "Recording" window, will be saved to be re-applied on the next

invocation of the feature.

If you quit the WaveGazer with some windows left open, these

windows will be closed before exiting and re-opened the next time

you start up the WaveGazer.

All possible customization (and also the colors) are kept in a

large structure, which will be written to disk (in file "WG.CNF")

at program exit and read at program startup.

The menu options "Customize -> Save Settings" and "Customize ->

Write Settings" may be used explicitly read or write this file.

19.2 Customization Of The Printer

You may customize the printer by means of the dialogbox shown if

you select "Customize -> Printer" menu.

There are four fields labeled

*    Init      The string sent to the printer at startup of the

WaveGazer. You may use this feature to initialize

the printer to a certain mode to be used while in

the WaveGazer. Normally, you may wish to leave

this field blank.

*    Condensed String to send to the printer to put it in

condensed mode. Condensed mode means that the

printer should be able to print 132 characters /

line.

*    Normal    String to send to the printer to select normal

printing, 80 characters / line.

*    Reset     String sent to the printer upon closing of the

WaveGazer. Use this to eventually reset settings

initialized by the 'Init' string.

You need to respect a defined syntax when filling in these

fields: All printers use unprintable characters as control

sequences, called escape sequences. If you need to send a escape

character to the printer, fill in the field the sequence '*E',

which will be translated to the character 'escape', decimal 27,

octal 033, hex 1B.

Often, you will need enable condensed mode with the code 15 hex.

This is a non-printable character as well, a ctrl-P. You will

need to enter this code as a '^P', as Unix users may know it...

Ctrl-A corresponds to 00H, ctrl-B to 01H, ctrl-C to 03H and so

on.

User's Guide                The WaveGazer v1.0                 Page 47

----------------------------------------------------------------------

These codes, you only need to store them once, for all subsequent

invocations of the WaveGazer, they will be read from the file

WG.CNF, where they are saved.

To test the codes you typed in, hit the "test" button, your

printer prints out some test which shows if the codes are

understood properly!

19.3 Customization Of The Colors

The menu option "Customize -> Colors" will pop up a dialog box

with several colors to choose from. (Change them by clicking in

the color fields directly)

All color selection boxes define two colors at once:

*    Backgr/Text:   These are the colors of the window background

and the text written on it

*    Frames:        The two colors used for drawing the frames

around the windows. If both colors are the

same, no 3-d effect will be visible...

*    Buttons:       These colors apply to the pushbuttons, the

knobs on the scrollbars and the VFO button in

the receiver control window. Buttons with 3-d

appearance always use the same color

(darkgray) as the lower shadow color and the

text always will be written in black.

*    I/O Fields:    The colors used for the background and the

text in the textfields

*    Toggles:       The color of the outline (the first color)

and the one of the 'light' in it

*    Menus:         Colors of the menus, same rules as for push

buttons apply

*    Textlines:     The background and text color of the

textlines in e.g. the file selection box or

the data base window. Also used in the main

window of the memory editor

*    Graphs         Colors used to draw the scan graphs

Changed colors are only read on construction of a widget, so

existing widgets do not change colors!

User's Guide                The WaveGazer v1.0                 Page 48

----------------------------------------------------------------------

20. Release Notes

Thanks to some user input, I was able to correct the following

bugs up to now:

In Release Beta.1:

*    Release Beta.0 had problems on fast machines. To be honest,

it only ran on rather slow machines. The problem was a

communication time-out that was too short for faster

machines, since I implemented the time-out in a simple loop.

Now, the 'Control' menu has one more option, called

'Calibrate'. You need to calibrate your sample of Yakta just

after the first time you started it up. The time-out value

will be stored in the file YAKTA.CNF, so you need to to this

only once. This communication problem showed up in different

ways:

- The Memory Editor and the Memory Viewer showed channels

containing 8888.888 kHz even when the channel was either

vacant or occupied.

- The bandscan feature worked badly or not at all.

- Startup reported 'receiver not responding', although it

was on.

- Sometimes, the receiver went in testmode, with all leds

lit on.

*    When Yakta established a connection to the receiver, it used

to set the variable bandwidth to 2000 Hz. This is no more

the case.

In Release Beta.2:

*    The recording feature was so far unusable, in this release,

I reviewed the whole stuff and I think this should be better

now.

In Release Beta.2.1 (never distributed):

*    The bandscan feature allows you to monitor a single

frequency over time. A bug where it did not stop at the end

of the display has been corrected

In Release Beta.3:

*    The timer sometimes displayed strange times (30:08 UTC).

This was due to bad programming in the timer module. This

release now records how many hours (minutes) the receiver is

ahead of UTC and how many hours (minutes) the PC is ahead of

UTC. This was needed by the new implementation of the timer.

*    There is one new menu option: 'Control -> Get Time Diff.'

This allows you to verify, how many minutes the PC's and the

receiver's clock differ from UTC. (at least by definition,

these are user supplied values.)

User's Guide                The WaveGazer v1.0                 Page 49

----------------------------------------------------------------------

*    The time differences UTC - localtime for the PC and the

receiver may be supplied at program startup if they are

different from 0. Take the case where your PC is set to

localtime and the receiver to UTC (that's what I suggest).

You live in the USA, 4 hours west of UTC, start Yakta with

the line "yakta 1 prn 4 0", which means "use serial port 1,

printer prn, assume PC is 4 hours ahead (west) of UTC, and

receiver is on UTC.

After startup, a popupbox is eventually displayed, telling

that the times of PC and receiver differ. You now may

synchronize either the receiver to the PC or the PC to the

receiver's clock. Once in sync, the popupbox won't be

displayed anymore on further invocations of the program.

Some more examples of startup:

Switzerland, one hour east of UTC, PC locatltime, RX UTC:

"yakta 1 lpt1 -1 0" in winter; "yakta 1 lpt1 -2 0"

during daylight saving time (summertime).

Somewhere in Russia, 4 hours east, PC and RX to localtime:

"yakta 1 lpt1 -4 -4"

*    Users using a serial port other than com1 always needed to

type "yakta 2 [printer]". With this release, all command

line arguments are stored in Yakta.cnf, so you need to

supply them only once! Eventual changes may be made by

supplying them again.

In Release Beta.4:

*    The 'window manager' handled some overlapping of windows

wrong, e.g. the 'Receiver Control' window and the

'Recording' window. If then you deleted the partially hidden

window, a lot of "runtime error == 1: mouse2:enabl(#)"

covered the screen. I do not claim that there will be no

more such errors, but I did not see any of them in a later

release!

*    I found a name for the product: "The WaveGazer". I will call

this software still Yakta while in test phase, but the day

it will be 'adult' it will be the WaveGazer (...sounds much

more professional ;-) The background already looks like it

will, because I'm so proud on the logo!

*    This release also exists for Hercules graphic adapter!

In Release 5.* (never distributed):

*    I tried to implement two big improvements to the software,

namely a database cache and what I call the 'tracking mode'.

The 'tracking mode' allows the user to tune the receiver

manually while the computer tries to find a record in the

database which matches the receiver's frequency. This was

quite a tough thing to realize, and took me over all

Beta.5.* versions (about 4565) to create something that

works more or less.

User's Guide                The WaveGazer v1.0                 Page 50

----------------------------------------------------------------------

*    The 'Database' window has gotten a status line, which

displays the actual status of the database cache.

*    The introduction of the database cache resolved a old

problem where upon hitting "Search" in the 'Search' window,

not all desired records were displayed, unless the user hit

"Reset" button first. Was not very handy, indeed...

*    Lots of small bugs were also corrected, bugs I don't

remember anymore, but they existed and they certainly still

have some colleagues in Yakta somewhere!

In Release 6:

*    Printing of databases is finally possible! Although possible

already in earlier versions, it is now really working and

somewhat configurable! A popupbox has been added, asking the

user for the desired format of the lists, simple (as before)

or complete.

*    Printers are configurable, a dialogbox for printer

customization has been added.

*    Startup has some bugs removed, especially concerning users

not using standard ports (com1 and lpt1)

*    Error messages have been added to advert the user in case he

forgot to specify the environment variables YAKFIL and

YAKSWP.

*    Unimplemented menu option "QSL" has been removed and has

been replaced with an option "About..." which is to pop up

two dialogboxes telling some info (junk...)

*    There is a documentation which is up-to-date and which has

an index. Hooray!

In v1.0 of Wavegazer:

*    Changed the name from Yakta to WaveGazer.

*    Changed the name of files and environment variables to

reflect the new name: Yakta.cnf got WG.CNF, Yakta.rec got

RECORD.DAT, YAKFIL is now WG_FIL and YAKSWP id WG_SWP.

*    Fixed a bug where the WaveGazer will hang if you close the

'Bandscan' window while a sweep is running.

*    Documentation has been reviewed. (Still not error free, but

I rather speak German and French than English...)

*    Hercules version is brought to this level as well.

User's Guide                The WaveGazer v1.0                 Page 51

----------------------------------------------------------------------

21. Problems

*    In the bandscan feature, if you select the fastest possible

scan, the values read back may be wrong, e.g. always the

same. If this occurs, you need to slow down the scan by

means of the scrollbars for number of samples taken or

(better) the one for the AGC settle time.

*    If in tracking mode, you should not use the keys to control

the receiver from the keyboard frequently (at keyboard

repetition rate): This may (and will) hang the program. It

seems to me a problem with interrupts in the communication

PC - receiver.

*    The tracking may show some problems, but none of these

crashed the application or even the PC. (lately, I did no

more experience these problems, so maybe they are fixed?)

*    If you install this version of the WaveGazer in a directory

where you already had a previous version of this software,

you need to remove the file called "yakta.cnf", the

configuration file. If you fail to remove it, Yakta will

probably refuse to start or display the menus and other

stuff in black. This is because the format of the config

file has changed and consequently cannot be read correctly.