BRAINEX WEATHER BRAIN

=====================

Now you can provide your users with local and state weather information,

updated every 3 to 4 hours... for the price of a phone call!

This is possible because nearly every airport in the country runs a

system that provides information to pilots (anyone may access this

information).  Weather Brain will call your local airport's number

and create ANSI and ASCII news files with your local and state weather.

GETTING STARTED

================

This program requires the installtion of the Brainex Door manager.

If you have not done so, please install BRAINEX.ZIP (available from

the Brainex System)

Check the included file STATION.TXT to determine your local

airport's 3 letter identification.

Example: LAX is Los Angeles International Airport.

Weather Brain supports two serivces:

Weathermation (tm)

------------------

Ask if your airport provides "Plain English Weather Information"

(also known as METRO information).  Many smaller airports don't.

Try to find the nearest airport that can provide this information.

Obtain your local WeatherMation (tm) number.

Pan Am systems, the makers of WeatherMation (tm) will provide this

information to you if you call them at 612-727-1084.

GTE Contel DUAT

---------------

Use 1-800-767-9989 to access this service. Information will not have

as much detail as Weathermation (tm), and only a local news bulletin

will be created.

Run the included SETUP batch file to install Weather Brain.

MODEM SETUP

===========

* Set time to wait for carrier to 45 seconds (S7=45)

* Use Verbal Result Codes (V=1)

* Use Extended results BUSY,RING,RINGING, etc. (check your modem manual)

* If you use a fossil, add FOSSIL to the command line: WBRAIN FOSSIL

* GTE Contel DUAT can use 9600 baud.

* You do not need to disable MNP for GTE DUAT

* IMPORTANT! If you use Weathermation (tm) you must disable

MNP error correction (check your modem manual)

* Use 1200 or 2400 baud with Weathermation (tm) for best results.

Sample Init Strings to Disable MNP:

ZYXEL:

AT&K0

HST:

AT&M0

HAYES:

AT\N0

---------------------------------------------------------------------

#                 #                #

This version of Weather Brain is a DEMO. When registered, the program

run in unattended mode. It will become full featured upon registration.

This door is released "As Is" and there is *NO* warranty or guarantee

that it will work for you and not cause any problems. You have chosen

to use this door, YOU are responsible for any unfortunate problems that

may develop from its use or misuse, including any long distance phone

charges that may be incurred by the operation of the program.

I will gladly listen to any suggestions or problems you may have,

and I will do my best to fix whatever needs to be fixed. I can not be

held responsible for any problems you may have, whether directly or

indirectly due to the use of the door or its utilities or associated

files. Please be sure the program works for you before you register!

Please read REGISTER.DOC for information on how you can register

your copy of Weather Brain.

For complete support, contact Stanley Sheff on The Brainex System.

Your questions will be quickly and cheerfully answered!

Stanley Sheff

The Brainex System

310-275-2344 BBS

310-273-5234 BBS

310-275-5833 Voice (10am-6pm PST)