<ME>  BONNEVILLE QUOTE EQUIPMENT

If difficulty is encountered in detecting the presence of a

live data feed, then check each piece of hardware provided

by Bonneville Market Information.  Make sure all red lights

are blinking or on solid.  Three lights that are particularly

important are listed below:

BS - Signal Strength.

RD - Receiving Data.  Blinks as data is received.

DO - Data Out.  Blinks as data is sent out to the computer.

DATA CONVERTER

If you have a data converter, the lights marked 'ON' and

'BS' should be on solid.  The 'DO' and 'RD' lights should be

blinking.

DATA RECEIVER

The data receiver box has 6 lights.  All should be on solid

except the 'DO' and the 'RD' lights which will blink.  The

'DO' light on the data receiver blinks in conjunction with

the 'RD' light on the data converter.

You can test your signal strength by pressing the  RED  test

switch on the front panel of the data receiver.  While this

switch is on, a blinking 'BS' light indicates a weak signal.

Do not be concerned if the light goes out mementarily upon

pressing the test switch.

VALLEY SATELLITE RECEIVER

This box has 6 lights and a signal level meter.  Signal loss

could be due to a temporary interference in the atmosphere

and will clear itself, or the satellite dish alignment may

need adjustment.  Keep the dish free of ice and snow.

If the 'DO' or 'RD' lights are not blinking then you are either

not receiving any data or not sending any data to the computer.

Unplugging the data receiving and data converter for 15 seconds

will sometimes reset the equipment.  If that doesn't work you

will need to call Bonneville Market Information and have the

receiver equipment re-profiled.  You can call 1-800-255-7374 for

help in trouble shooting.  Have your Data Receiver serial number

ready to give to the Customer Service Operator.  This number is

located on either the front of the Data Receiver, or on the back

on a silver label.