The RoundTable server for the G8BPQ switch.

Requirements:

Runs in a 30K DESQview window.

Uses it's own set of BPQ switch ports.

Has it's own node command.

Getting started:

First, configure the switch for RT.

In the file BPQCFG.TXT, add the new application RT.

Note that it MUST be the fourth or later application.

Example: APPLICATIONS=BBS,,,RT

Add some ports to the TNCPORTLIST.

RT ports cannot be shared with other applications.

Run BPQCFG and re-boot your system.

Startup RT in a 20K DESQview window.

RT APPL TimeOut Start_Port N_Ports

APPL is the bpq application number.

TimeOut is the inactivity timeout in seconds.

Start_Port is the first COM port RT will use.

N_Ports is the number of COM ports RT will use.

Example:

RT 4 240 9 8

Now anyone that wishes to use it can just connect to your node

and then issue the RT command.

You can log in to RT locally with control-E.

Control-F shuts RT down.

RT keeps track of user names and qth in file RT.USR