MON is a monitoring utility.

To date, this is just a test version.

It requires the g8bpq switch.

It requires a 34k - 37k DESQview window.

You need to supply it with a bpq stream number  (suggest 29).

Note that this stream number is NOT a COM port number, but

uses the 'extra' channels provided by bpq host mode.  You may

use any stream number you aren't using for something else.  For

example, if you use YTNC 25 1 1, then YTNC will be using host

ports up to 25.  Just don't use a number that you will be using

for something else, like PAC4, CHAT4.

Use:

MON -m stream  (insert -m # in the parameter field of DESQview window,

ie, Parameter -m 29.  Note that "m" is not a variable.

It has several commands.  Run it, hit the enter key, it will

tell you what it can do.  The commands and capabilities are

still under development ... and subject to change.

a - monitors all ports

1 - monitors port 1, etc.

n - monitors no ports

r - toggles to monitor rx, tx, or rx and tx frames

i - toggles to recieve text information or not.

l - shows the number of calls heard recently.

q - quit monitor

o and c allow you to open a log file, log activity for awhile,

then close it for later examination.

The stats command (S), will display statistics on the frames

MON has seen on each channel, and the percentage of those bytes

that carry data with respect to the total bytes seen.

FR    = Total frames heard of all types

%data = % of bytes that carry data

L3    = Layer 3 bytes total

L2    = Layer 2 bytes total

L1    = Layer 1 bytes total

REJ   = Number of FR frames that were rejects.

MON cannot count retries, since it didn't hear them.  BPQ stats are

more accurate.

I expect MON to become a much more useful network statistics collector

in the future.  Your ideas about what it should do are most welcome !