The other file in this ZIP, PSLWGS.PSF, contains version 1.2 of

some PostScript code that allows control over gray scale and minimum

line width on a color-by-color basis for monochrome PostScript output

using the AutoCAD Release 12 PSOUT command.  It's been tested on an

NEC Silentwriter 90 and an Agfa 2400 dpi PostScript printer.

PSLWGS.PSF requires AutoCAD Release 12 (or higher).

PSLWGS defines a minimum line width in inches for each AutoCAD

color. Entities with explicit widths (for example, plines) that are

greater than the minimum for that color will be printed with their

explicit width (this assumes interpreting AutoCAD drawing units as

inches).  In some cases some lines will be wider than desired (for

example, blocks inserted with X and Y scale factors that are not

equal in magnitude).

Due to limitations in the way AutoCAD handles the PSOUT command

and the PSPROLOG system variable, it's not particulary convenient to

change line widths and gray scales.  I hope this will improve in the

future.  For now, you must edit either ACAD.PSF or each individual

.EPS file produced by PSOUT to change the line widths or gray scales.

Changes in ACAD.PSF affect all files produced by PSOUT after the

change; changes in an .EPS file produced by PSOUT affect only that

file.

Currently, PSLWGS only supports portrait output.  This means

that lines that are vertical on the AutoCAD screen will be parallel

to the long edge of the paper when printed.  If you're doing PSOUT to

import to another program and need another orientation, the program

into which you're importing the .EPS file amy be able to rotate it.

To use this file:

I.  Initial setup:

A.  Locate the file ACAD.PSF.  It's probably in the SUPPORT

subdirectory of your AutoCAD directory.

B.  Use an ASCII text editor to insert PSWLGS.PSF at the end

of ACAD.PSF.  (ACAD.PSF is covered in the Release 12

"Customization Manual" on pages 228-232).

C.  If you will normally use line widths other than 0.001" (note

that a 300 dpi printer does line widths in units of 0.0033"),

change the values in LGP_colortowidth (see step C, below) to

be your most common linewidth settings.

II.  Using PSLWGS.PSF

A.  In the Autocad drawing editor, set the PSPROLOG system

variable to point to lw_gs_prolog.  NOTE THAT THIS IS

CASE SENSITIVE!  You can do this by typing:

psprolog lw_gs_prolog<Enter>

(Note that PSPROLOG is saved in ACAD.CFG and not with the

drawing.  This means that this setting will apply to all

your drawings until you change it).

B.  Perform a PSOUT command.

C.  IF you want to change the line widths from the defaults

(0.001" if you haven't changed them), use an ASCII text

editor to edit the resulting .EPS file. Near the beginning

of the file you'll find:

/LGP_colortowidth

[

0.001          % Color 1 (red)

0.001          % Color 2 (yellow)

0.001          % Color 3 (green)

0.001          % Color 4 (cyan)

0.001          % Color 5 (blue)

0.001          % Color 6 (magenta)

0.001          % Color 7 (white)

0.001          % Color 8

0.001          % Color 9

0.001          % Color 10

0.001          % Color 11

0.001          % Color 12

0.001          % Color 13

0.001          % Color 14

0.001          % Color 15

] def

Change the numbers as desired.  The units are inches. Note

that on a 300 dpi printer, the minimum line width is 0.003333

inches; any specified line width that's less than that will

come out as 0.003333 inches.

Just below this section you'll find:

/LGP_colortogray

[

0            % Color 1 (red)

0            % Color 2 (yellow)

0            % Color 3 (green)

0            % Color 4 (cyan)

0            % Color 5 (blue)

0            % Color 6 (magenta)

0            % Color 7 (white)

0            % Color 8

0            % Color 9

0            % Color 10

0            % Color 11

0            % Color 12

0            % Color 13

0            % Color 14

0            % Color 15

] def

Again change thise numbers as appropriate.  0 or 0.0 is

black; 1 or 1.0 is white (invisible, but it may overwrite

black or gray stuff depending on the order of drawing); 0.5

is 50% gray, and so forth.

D.  Save the file and copy it to your output device.

Jon Fleming

CIS:     70334,2443

REVISION HISTORY:

1.0 Initial release

1.1 Fixed bug of leaving an object on the stack after the entire

procedure finished

1.2 (13 Jul 93) Fixed bug of desired line width not saved and

restored with graphics state.  This manifested as "persistent

polyline widths"; when a polyline with explicit width greater

than the minimum was printed, all subsequent strokes were at the

same width as the polyline until a color change or another

polyline came along.