6/1/93

-- INSTALL.DOC--xall.sys V1.4.5

6400 Flying Cloud Drive Eden Prairie, MN 55344 (612) 943-9020

DigiBoard, GmbH Schmittgasse 64 5000 Cologne 90 Germany 49-2203-81083

Installation Guide Reference Manual for Universal OS/2 Device Driver for

DigiCHANNEL Intelligent Communications Boards

92000100C

DigiBoard, DigiCHANNEL, PC/Xi, PC/16i, PC/8i, PC/Xe, PC/16e, PC/8e,

PC/4e, PC/2e, Xem, PC/Xem, MC/Xem, EISA/Xem, COM/Xi, COM/8i, COM/4i,

MC/Xi, MC/16i, MC/8i, MC/4i and C/X are trademarks of DigiBoard.  All

other brand and product names are the trademarks of their respective

holders.

Digi International, Inc.  All Rights Reserved

Information in this document is subject to change without notice and

does not represent a commitment on the part of DigiBoard.  DigiBoard

provides this document as is, without warranty of any kind, either

expressed or implied, including, but not limited to, the particular

purpose.  DigiBoard may make improvements and/or changes in this manual

or in the product(s) and/or the program(s) described in this manual at

any time.  This product could include technical inaccuracies or

typographical errors.  Changes are periodically made to the information

herein; these changes may be incorporated in new editions of the

publication.

RESTRICTED RIGHTS:

For non-U.  S.  Government use: These programs are supplied under a

license.  They may be used, disclosed, and/or copied only as permitted

under such license agreement.  Any copy must contain the above copyright

notice and this restricted rights notice.  Use, copying, and/or

disclosure of the programs is strictly prohibited unless otherwise

provided in the license agreement.

For U.S.  Government use: Use, duplication, or disclosure by the

Government is subject to restrictions as set forth in sub-paragraph

(c)(1)(ii) of the Rights in Technical Data and Computer Software clause

of DFARS 52.227-7013.

Table of Contents				3

Introduction					4

The Distribution Diskette			6

Installing the Device Driver			7

Global Parameters				7

Per-Card Parameters				10

Examples					13

Testing the Ports				15

Troubleshooting					16

Error Messages					18

DMODE I/O Port Configuration Utility		19

Examples					23

Appendix Synchronous Communications Modes	25

Introduction

This release of the DigiWARE device driver for OS/2 supports the

entire line of DigiCHANNEL intelligent asynchronous serial communications

controllers:

Micro Channel (MCA) bus:

DigiCHANNEL MC/Xi 4, 8 and 16 port boards

DigiCHANNEL MC C/X cluster controller system

DigiCHANNEL MC/Xem PC/AT

(ISA) bus:

DigiCHANNEL PC/Xe 2, 4, 8 and 16 port boards

DigiCHANNEL PC/Xi 8 and 16 port boards

DigiCHANNEL COM/Xi 4 and 8 port boards

DigiCHANNEL ISA C/X cluster controller system

DigiCHANNEL PC/Xem

EISA bus:

DigiCHANNEL EISA C/X cluster controller system

DigiCHANNEL EISA/Xem

The device driver software supports multiple boards and multiple board

types in a single machine, and can handle up to 256 ports from any

combination of the boards listed above.  As new products are introduced,

the driver will be upgraded to include those devices as well.  The

driver takes all its configuration information from the device= command

line in CONFIG.SYS, and all DigiCHANNEL intelligent boards in a system

are configured from a single command line.

In addition to the device driver, the configuration utility DMODE.EXE is

provided to change communication parameters for individual ports without

rebooting.  DMODE.EXE is similar to the OS/2 MODE command, but has some

added features, and supports communications devices beyond OS/2s range

of COM1 to COM8.

Before installing this device driver software, complete the installation

of the DigiCHANNEL intelligent communications board(s) in your system,

according to the hardware installation manual provided with the

board(s), and write down your configuration!  The driver supports all

legal I/O and memory address settings, but must be informed of these

settings in the device driver command line.

The Distribution Diskette The diskette accompanying this manual contains

all of the files and documentation needed to install the DigiWARE OS/2

Universal Intelligent Device Driver in your system.  A list of these

files follows:

XALL.SYS	The installable device driver.

DMODE.EXE	A MODE work-alike program for configuring I/O ports beyond COM8

INSTALL.DOC	Installation instructions for the current release of the driver.

RELNOTES.DOC	Release notes.

DMODE.DOC	Instructions for the DMODE program.

XALL.DOC 	Programmers information about the driver and applicable

system calls.

*.bin		Miscellaneous binary files used by the driver.

Installing the Device Driver Copy all of the files from the distribution

diskette to a single directory on your hard drive.  By default, driver

and data files are expected to be in \DIGI.  The device driver is

invoked by placing its name and configuration arguments into the

systems CONFIG.SYS file.  All DigiCHANNEL intelligent asynchronous

serial communications boards must be installed from a single command

line, which contains complete set-up information for each board.  The

format of the command line is:

device=\digi\xall.sys [/n:names] [/b:baud] [/d:dtype] [/f:digipath] [/v:messagepath] /t:card /p:port /m:mem [, /t:card /p:port /m:mem] [, ...]

Note that the path for XALL.SYS is \DIGI.  This is the default path.  If an

alternative path is specified with the /f:digipath option, substitute that

path for \DIGI.

The above command line must appear on a single command line in the

CONFIG.SYS file.  This line may be up to 255 characters long.

Global Parameters The first six parameters in the command line above are

global parameters, and affect all boards installed.  These parameters

are optional, and may be omitted.  If omitted, default values will be

used.

/n:names 	Defines the naming convention to be use for the

DigiCHANNEL ports.  names consists of an optional

stem followed by the starting device number.  The new

stem must be enclosed in double-quotes ("").  If

the stem is omitted, COM will be used.  If the

number is omitted, 3 will be assumed.  If the /n

parameter is not specified, the default name COM3 will

be assigned to the first port.

/n:"digi"0 	DIGI0, DIGI1, DIGI2, ...  DIGIn

/n:4		COM4, COM5, COM6, ...  COMn

/n:"qdev"	QDEV3, QDEV4, QDEV5, ...  QDEVn

/b:baud 	Sets the default baud rate to be used for all ports.

If this parameter is not specified, all ports will be

set to the OS/2 default of 1200 baud.  Baud rates of

up to 38,400 are supported.  Use the DMODE.EXE

command to set baud rates of individual ports.

/b:9600		Initializes all ports to 9600 baud.

/d:dtype	Sets the initial data type (parity, character

length and number of stop bits).  If this parameter

is omitted, the OS/2 default data type of even

parity, 7 data bits and 1 stop bit is used.  The

syntax is [pds], where p is E, O or N; d is 8, 7, 6

or 5; and s is 1 or 2.  All three values must be

supplied, in the correct order.

/d:N81		No parity, 8 data bits, 1 stop bit.

/f:digipath	Defines an alternative directory path for the driver

and data files.  If this option is present, all files

from the distribution diskette must be present in this

directory.

/f:d:\drvr\digi		All data files are in D:\DRVR\DIGI.

/v:messagefile	Forces the driver to direct all verbose messages

to the file named.  If /v: is given without a

filename, messages will be placed in \XALL.MSG.

This flag prevents any information from being

displayed on the console; useful in some situations

as when the machine is a headless server, where

calls to Standard Output will cause problems.  Without

this flag, all messages will appear on the

system console.  The message file is overwritten

each time the system is started.

/v:errmsg	Writes banner and other info to file \ERRMSG.

/v: 		Writes banner and other info to file \XALL.MSG.

/a:a		Enable the alternate pin assignment of Data Carrier

Detect (DCD) for all ports (see discussion of Alt-Pin

processing.

Per-Card Parameters The remaining three parameters are per-card

parameters, and are used to inform the driver of the type of board and

where it is to be addressed in the hosts I/O and memory buses.  These

parameters are mandatory, and must be given for each board!  The

per-card parameters must be given in complete sets, and the sets must be

separated by commas.

/t:card		Defines the card type to the driver.  Legal values

for card are:	i for the ISA C/X,

e for the EISA C/X,

m for the MC C/X,

p for Xem (PC/Xem, MC/Xem or EISA/Xem)

x for COM/Xi boards.

This parameter must be omitted for PC/Xe, PC/Xi and

MC/Xi boards.  The designators i, e, and m must be

followed by an additional two digits defining the

number and arrangement of DigiCHANNEL C/CON-16 or

C/CON-16e concentrators on each host adapter channel.

If the concentrator(s) are connected to the host

adapter via a dial-up or leased line (remote

concentrators), each of the two digits must be

followed by a parameter, enclosed in parentheses,

specifying the synchronous communications mode to

be used for that line.  See the Appendix for a

list of available synchronous communications modes.

/t:i10		ISA C/X, one local concentrator on line 1 only.

/t:e11		EISA C/X, one local concentrator on each of

lines 1 and 2.

/t:m21		MC C/X, two local concentrators on line 1 and

1 on line 2.

/t:e12		EISA C/X, one local concentrator on line 1,

and two on line 2.

/t:i22		ISA C/X, two local concentrators on each of

lines 1 and 2.

/t:i2(21)3(27)	ISA C/X, two remote concentrators on line 1,

and two remote concentrators on line 2.

The concentrators on line 1 will use mode

#21 (76,800 baud, 8-wire, external clock) to

communicate with the host adapter, and the

concentrators on line 2 will use mode #27

(614,400 baud, 8-wire, internal clock).  See

the Appendix for a list of synchronous

communications modes.

/t:x		COM/Xi (4 or 8 channels)

/t:p           PC/Xem, MC/Xem or EISA Xem

/p:port		Defines the I/O port address for the card.

/p:300 		Board is addressed at I/O port 300h.

/p:2005		EISA board (EISA C/X or EISA/Xem host adapter)

is in slot 2 (EISA I/O addresses are 4

digits : the slot number, followed by 005).

When plugging an ISA board (PC/Xe, ISA

C/X host adapter, etc.) into an EISA

machine, the /p parameter must contain the

3-digit I/O address as set by the DIP switches

on the board, and not the 4-digit address

based on the EISA slot number.

/m:mem		Dual-ported memory starting address.  This is the full

32-bit hexadecimal address of the beginning of the

boards dual-ported memory.

/m:d0000	Sets dual-ported memory start address to

0D0000h (in the BIOS expansion area between

640K and 1 megabyte).

/m:e80000	Sets start address of E80000h (in the

fifteenth megabyte).  EISA and ISA boards

(except the COM/Xi), may all share the same

memory starting address.  COM/Xi and all

Micro Channel boards must each have

unique memory addresses.

Examples The following examples start out with just the minimum to

install a DigiCHANNEL board.  After locating an example for your type of

board, read the rest of the examples for optional situations that may be

appropriate to your application.  Default options will be altered by

your application program(s) when you configure the application.

--------

Example 1:      COM/Xi One COM/Xi at I/O port 300h, memory start address

D8000h; devices to be named COM5 COM6, etc.  All data files are in the

directory \DIGI:

device=c:\digi\xall.sys /n:5 /p:300 /m:d8000 /t:x

(Note that the above must all appear on one line in CONFIG.SYS)

--------

Example 2:      PC/Xe, PC/Xi One PC/Xe (or PC/Xi) at I/O port 220h,

memory address D0000h   Ports will be named COM3, COM4, etc.

(default names).  All data files are in the directory \MISC\DIGI:

device=c:\misc\digi\xall.sys /f:\misc\digi /p:220 /m:d0000

(Note that the above must all appear on one line in CONFIG.SYS)

--------

Example 3:      MC/Xi One MC/Xi at I/O port F1F0h, memory address

FC0000h.  Devices will be named DIGI1, DIGI2, etc.  and will have an

initial baud rate of 38,400.  All data files are in the default

directory \DIGI:

device=c:\digi\xall.sys /n:"digi"1 /b:38400 /p:F1F0 /m:FC0000

(Note that the above must all appear on one line in CONFIG.SYS)

--------

Example 4:      Two board installation with optional global parameters.

Two PC/Xi (or PC/Xe) boards at I/O ports 320h and 300h, with dual-ported

memory of both starting at D0000h.  Ports will be named COM0, COM1,

etc., and will be initialized to 38,400 baud, no parity, 8 data bits and

1 stop bit; messages will be contained in the file MESSAGES.MSG located

in the \DIGI directory.  All data files are in the default directory

\DIGI:

device=c:\digi\xall.sys /n:0 /b:38400 /v:\digi\messages.msg /d:N81 /p:320 /m:D0000, /p:300 /m:D0000

(Note that the above must all appear on one line in CONFIG.SYS)

---------

Example 5:      PC/Xem One PC/Xem at I/O port 224h, memory start address

C8000h; devices to be named COM3 COM4, etc.  All data files are in the

default directory \DIGI:

device=c:\digi\xall.sys /p:224 /m:C8000 /t:p

(Note that the above must all appear on one line in CONFIG.SYS)

---------

Example 6:      ISA C/X One ISA C/X host adapter at I/O port 228h,

memory address D0000h, with one concentrator on line 1 and two on line

2.  Devices to be named DCX1, DCX2, etc.  and will have an initial baud

rate of 38,400.  All data files are in the default directory \DIGI:

device=c:\digi\xall.sys /n:"dcx"1 /b:38400 /p:228 /m:d0000 /t:i12

(Note that the above must all appear on one line in CONFIG.SYS)

-----------

Example 7:      EISA C/X with a remote concentrator via an RS-232

synchronous modem.  One EISA C/X host adapter in slot 3, memory address

D8000h, with two local concentrators on line 1 and one remote

concentrator at 9600 baud and external clocking on line 2.  Devices to

be named COM5, COM6, etc.  and will have an initial baud rate of 19,200.

All data files are in the directory \OS2\DIGI:

device=c:\os2\digi\xall.sys /n:5 /b:19200 /f:c:\os2\digi /p:3005 /m:d8000 /t:e21(17)

(Note that the above must all appear on one line in CONFIG.SYS)

----------

Example 8:      MC C/X and MC/Xem One MC C/X host adapter at I/O port

228h, memory address D8000h, with one concentrator on line 1 and one

concentrator  on line 2; one MC/Xem at I/O Port 304h, memory address

D0000h.  Devices will be named DIGI1, DIGI2, etc.  and will have an

initial baud rate of 38,400, Alt-Pin enabled, no parity, 8 data bits,

and 1 stop bit.  All data files are in the default directory \DIGI:

device=c:\digi\xall.sys /n:"digi"1 /b:38400 /a:a /d:N81 /p:228 /m:d8000 /t:m11, /p:304 /m:d0000 /t:p

(Note that the above must all appear on one line in CONFIG.SYS)

(Note that for MC C/X and MC/Xem, the addresses 104 and 108 should not be

used to avoid possible conflicts)

----------

----------

Testing the Ports The simplest (and probably the best) way to check the

driver and I/O connections is to connect an ASCII terminal to one of the

ports and redirect some output to that port.  If the terminal is

connected to COM3, try something like the following:

dmode com3

(this will verify that the driver can find the portthe ports parameters will

be displayed.)

dmode com3 octs=off odsr=off

(this insures that the hardware handshaking signals CTS and DSR will

not prevent outputwe are not concerned with flow control at this time.)

dir > com3

(this redirects the current directory to COM3.) The console should simply

respond by repeating the C:\> prompt, and the current directory

(in this case, C:\) will appear on the terminal.

Troubleshooting If the C:\> prompt doesn't reappear, and the system

seems to be hung, you probably used a memory start address that is

being used by some other device, or the board is not set for the address

specified in CONFIG.SYS.  Try a different address.  If it still hangs,

try a different I/O port.  If your computer has a memory cache and the

boards memory start address is above 1 megabyte, try either disabling

the cache or moving the board to a low-memory address (D0000 is usually

a good choice).  It is also possible that a conflict exists with another

device driver which has been loaded via CONFIG.SYS.  The XALL.SYS line

should be placed near the end of the CONFIG.SYS, and must not be placed

before the DEVICE=TESTCFG.SYS line.  Due to potential device naming

conflicts, the pair of lines

DEVICE=\OS2\COM.SYS and

DEVICE=\OS2\VCOM.SYS

may cause problems.  Be sure that the XALL.SYS line is placed before

these lines in CONFIG.SYS.  If the XALL.SYS driver generates an error

message, try temporarily putting REM in front of the above two lines.

If this works, there is probably a naming conflict.  You may need to

add a /n: command line flag to the XALL.SYS line to force the driver to use

a different base name or start numbering the COM devices at 5 or above.

If the C:\> prompt does reappear, but there is no output on the terminal

screen, there is probably a cabling problem.  Make sure that all cable plugs

are securely seated in receptacles, and that they are configured correctly

(see the appropriate section of the hardware installation guide for your

board).  If the terminal displays output, but the data on the screen is

garbled, the terminal is probably not set with the same baud rate and data

type as specified in CONFIG.SYS (or the defaults, if not specified).

Verify these settings.  If everything checks out and you still have

problems, there could be a problem with your board.  Contact your distributor

or DigiBoard Technical Support for further assistance.

Error Messages If, when the driver is loaded, it cannot find the

board(s), or if they fail to respond properly to commands, certain error

messages may be sent to the console.  These error messages are defined

in the following table:

Could not find BIOS file

Could not find FEP file

Could not find data file 	One or more files were missing from the

digipath directory.  The default digipath

directory is /digi.  If you have loaded the

driver and data files into a different

directory, that directory must be

specified in the xall command line with

the /f:digipath parameter.

IO Port Not Responding 		I/O port address set incorrectly, or

conflicting with some other device.  Verify

setting, or try a different address.

DigiBoard BIOS Will Not Boot

DigiBoard BIOS Will Not Respond

DigiBoard FEPOS Will Not 	DigiCHANNEL board will not respond to

commands from the driver.  Typically due to a

memory conflict, or a memory cache conflict.

Driver Failed to Hook Timer

System Cannot Allocate Memory	 A system call to OS/2 has failed.

Invalid Command Line Arguments	Syntax error in the command line in

CONFIG.SYS.  Double-check your entries.

========

DMODE I/O Port Configuration Utility DMODE is DigiBoards I/O channel

configuration utility.  DMODE functions like the OS/2 MODE command, but

is more flexible, and provides some added features.

-Supports communications device names other than COM1-COM8.

-Any single parameter may be changed without causing all others

to be reset to default values.

-Can be used to enable or disable the Alt-Pin feature on

individual ports.

-Maintains the familiar MODE syntax.

DMODE arguments may be entered in any order, and in many cases may be

abbreviated.  For example, a baud rate of 19,200 may be entered simply

as 19, since within the context of the DMODE command, 19 cannot be

interpreted as anything else.  Arguments may be separated by spaces or

commas, and unspecified parameters are left unchanged.  DMODE returns

the settings of all parameters after any requested changes have been

made.  If no parameters are given, DMODE simply returns the current

settings.  The syntax of DMODE is: dmode port_name [parameters]

Port names and parameters are not case sensitive, so upper and lower

case letters may be freely mixed without effect on the operation of the

DMODE command.  The following parameters are accepted:

Baud Rate       Baud rates can be specified with 2 to 5 digit designations,

according to the following table:

Accepted Values 	Baud Rate

11 110 			110

15 150 			150

30 300 			300

60 600 			600

12 120 1200 		1200

24 240 2400 		2400

48 480 4800 		4800

96 960 9600 		9600

19 192 1920 19200	19200

38 384 3840 38400 	38400

56 567 5670 56700 	56700

Character Length Characters may be 5, 6, 7 or 8 bits long.  Enter

the appropriate number in the DMODE arguments.

Parity:  Enter the desired parity checking method with a single

letter E, O, N, M or S for Even, Odd, None, Mark or Space, respectively.

Stop Bits: Enter the desired number of stop bits.  Recognized

values are 1, 2 and 1.5.

Flags : DMODE supports the following flags in the same way as

the OS/2 MODE command.  Refer to your OS/2 documentation for

explanations of the flags.  To set a flag, enter the mnemonic for the

flag, followed by an equals sign (=), and the desired value (ON, OFF,

etc.).

TO=ON		Write timeout in effect.

TO=OFF		No timeout (infinite write timeout)

XON=ON		XON/XOFF handshaking enabled.

XON=OFF		XON/XOFF handshaking disabled.

IDSR=ON		Enables Input DSR Sensitivity.

IDSR=OFF	Disables Input DSR Sensitivity.

ODSR=ON		Enables Output DSR Sensitivity.

ODSR=OFF	Disables Output DSR Sensitivity.

OCTS=ON		Enables CTS output flow control.

OCTS=OFF        Disables CTS output flow control.

DTR=ON		Sets DTR to ON or HIGH.

DTR=OFF		Sets DTR to OFF or LOW.

DTR=HS		DTR used for input flow control.  RTS Signal Control

RTS=ON		Sets RTS to ON or HIGH.

RTS=OFF		Sets RTS to OFF or LOW.

RTS=HS		RTS used for input flow control.

RTS=TOG		Toggles RTS on every transmit request. (Not supported)

ALT=ON		Internally swaps DSR and DCD signals (see explanation below).

ALT=OFF		DSR and DCD are not swapped.

DigiCHANNEL products use 10-pin RJ-45 connectors in order to support all

of the signals normally available on a 9-wire RS-232 connection.

However, many commercially available RJ-45 connectors have only 8 pins.

8-pin RJ-45 plugs will fit in a 10-pin jack, but the first and last pins

(numbers 1 and 10) are not connected.  DigiBoard uses these pins for the

modem control signals RI (Ring Indicator, pin 1) and DCD (Data Carrier

Detect, pin 10).  While Ring Indicator is rarely a concern (since most

modems have auto-answer capability), Data Carrier Detect is often

monitored to insure that the modem has established, and is maintaining,

a valid connection with a remote modem.  To facilitate the use of 8-pin

RJ-45 connectors in modem applications, a special DMODE flag, ALT, has

been provided.  ALT instructs the device driver to monitor pin 1 of an

8-pin RJ-45 for DCD.  See the Hardware Installation guide for your board

for cabling diagrams and pin assignment charts for DB-25 and RJ-45

connectors.

Examples

dmode digi12

(Returns the current settings of device digi12.  No changes are made.)

dmode com5 2

(Configures COM5 for 2 stop bits.  All other parameters remain unaffected.)

dmode com5:38 rts=on

(Configures COM5 for a baud rate of 38,400 and sets RTS to ON or HIGH.)

dmode com5 8

(Configures COM5 for 8 data bits.  No other parameters are affected. )

dmode comdv83 idsr=off, to=off e 7 12

(Sets COMDV83 to disable Input DSR Sensitivity, disable output timeout,

select even parity, 7 data bits and a baud rate of 1200. )

dmode com5 38 alt=on

(Sets up COM5 for a baud rate of 38,400, and enables Alt-Pin Processing.)

dmode com5 e alt=off

(Sets COM5 for even parity checking and disables Alt-Pin processing.)

dmode com3 odsr=off octs=off

(Disables sensing of output flow control signals Clear To Send (CTS)

and Data Set Ready (DSR). )

===========

Appendix

Synchronous Communications Modes

The DigiCHANNEL C/X host adapter communicates with attached C/CON-16

concentrators via two RS-422 synchronous channels (newer concentrators and

host adapters also support RS-232 synchronous communicationsee your hardware

Installation Guide).  The default operating mode for these channels is

1.2 megabaud, 8-wire, internally clocked.  This provides the fastest

possible communication between the host adapter and locally connected

concentrators (in this context, local indicates that concentrators are

connected directly, via a 4 or 8 wire cable, to the host adapter;

remote refers to concentrators that are connected to the host adapter via

data communications devices such as modems or DSUs).  Local concentrators

always communicate with the host adapter in either an eight wire internally

clocked mode or a four wire self-clocked mode (in the four wire modes the

clock is encoded with the data instead of being carried on separate lines as

in the eight wire modes).  Remote concentrators always use an eight wire

externally clocked mode (the clock signal is provided by the synchronous modem).

The table on the following page shows the modes that are supported by

the XALL.SYS device driver.  Note that the external clock baud rates are

approximates used to set up time-outs and tuning parameters only.

Values do not need to be exact; select the speed closest to the actual

sped of your synchronous modem.