DigiWARE Release Notes   93000098m

for

DigiCHANNEL PC/X*, COM/Xi, MC/Xi, Xem, C/X Universal OS/2 Device Driver

XALL.SYS  V1.4.5

Microsoft OS/2          1.0, 1.1, 1.2, 1.21, 1.3

IBM OS/2                1.0, 1.1, 1.2, 1.3, 2.0, 2.1

IBM OS/2 Ext. Edition   1.1, 1.2, 1.3, 2.0

Diskettes               P/N's 40000593L (5) or 40000594L (3)

Diskettes               P/N's 40000943  (5) or 40000908  (3)

Software Manual         P/N 92000100C

6/5/93

1. Enhancements from previous intelligent card device driver (1.4.4).

- extra diagnostic information added for cases of failed

initialization.

- add support for changed IO Address tables for MC/2e

2. Bug Fixes

- changed shared RAM accessing functions for better OS/2 2.1 support.

- Device Header Attribute bit11 (OPEN/CLOSE requests required) could

cause interference problems with OS/2's provided COM.SYS driver.

Symptoms included hanging at boot time or missing the first few

device names on the DigiBoard.

- Last-CLOSE requests issued while hardware handshaking was in

effect and immediately following a FLUSH-OUTPUT command would

mistakenly block waiting for output to drain.

- Fixed Last-CLOSE process data loss bug. Buffers were not allowed

to drain, but were flushed at Last-CLOSE.

- Some side-effects of the SET_DCB command did not exactly mimic

the behavior of the stock OS/2 COM.SYS (or COM01.SYS) driver.

This related to conditions under which DTR/RTS signals would be

toggled as a result of changes in Hardware Handshaking paramerters.

- Fixed DMODE bug  that would not recognize the "DTR=" command line

parameter.

3. Known Limitations

- Maximum 256 channels.

- The COM/Xi is limited to 9600 or at most 19200 baud.  All other

intelligent cards will support 38.4K baud on all channels, and

57K baud on a few channels.

4. Manual Errata