Network Operating System

User Reference Manual

Phil Karn, KA9Q

and

Gerard van der Grinten, PA0GRI

This manual reflects version 911229 of NOS 2.0j

as released to the public by PA0GRI

- 2 -

1.  The NOS.EXE Program

The MS-DOS executable file NOS.EXE (Further  called  Nos)  provides  Internet

(TCP/IP),  NET/ROM  and  AX.25 facilities.  Because it has an internal multi-

tasking operating system, Nos can act simultaneously as a  client,  a  server

and  a  packet switch for all three sets of protocols. That is, while a local

user accesses remote services, the system can also provide  those  same  ser-

vices  to remote users while also switching IP, NET/ROM and AX.25 packets and

frames between other client and server nodes.

The keyboard and display is used by the local operator to control  both  host

and gateway level functions, for which a number of commands are provided.

1.1.  Startup

nos [-b] [-s <#sockets>] [-d </directory>] [-v] [<startup file>]

When Nos is  executed  without  arguments,  it  attempts  to  open  the  file

autoexec.nos in the root directory of the current drive.  If it exists, it is

read and executed as though its contents were typed on the  console  as  com-

mands.  This feature is useful for attaching communication interfaces, confi-

guring network addresses, and starting the various services.

Four command-line options are accepted:

1.1.1.  -b

The -b option specifies the use of BIOS for console output; the default is to

write  directly  to the video display buffer. Use this option if you are run-

ning under a  windowing  package  and  have  trouble  with  output  "bleeding

through" on top of other windows.

1.1.2.  -s <no_of_sockets>

The -s option specifies the size of the socket array to be  allocated  within

Nos.  This limits the number of network connections that may exist simultane-

ously; the default is 40.

1.1.3.  -d </directory>

The -d option allows the user to specify a "root" directory  for  the  confi-

guration and spool files; it defaults to the root directory of the system.

1.1.4.  -v

The -v option allows the user to view command execution during the startup of

Nos.  It  lets  the commands read from autoexec.nos echo before they are exe-

cuted. This is a nice help if Nos stops (hangs) during initialization.

After all command line options, the name of a alternate startup file  may  be

specified. This file is then opened and read instead of autoexec.nos.

- 3 -

1.2.  DOS environement variables.

The following DOS environement variables can be used  to  specify  things  to

NOS.

1.2.1.  TZ

The TZ variable should be set to the local timezone. Default is UTC.  This is

used on the timestamp in smtp.

1.2.2.  MAILER

The MAILER specifies what program should be started when the mail command  is

entered. Default is BM.EXE.

1.2.3.  COMSPEC

The COMSPEC specifies what command shell will be used to shell  out  of  Nos.

This  is  normaly  set  by  MS-DOS  startup  to COMMAND.COM.  Default is also

COMMAND.COM.

1.2.4.  TMP

The TMP variable is used to create a spot where temporary files are  created.

Without  TMP  being  set the temp files are created in the root directory.  A

sample is "set TMP=C:\tmp\".

1.2.5.  USER

The USER variable is used by ftp and Rlogin to set the username for the  rlo-

gin  daemon on the remote system. The default when not setting USER is guest.

Guess you don't like it, but that's life.  With ftp the user is suggested  in

the  user name prompt. If a cr only is given the suggested name is used, oth-

erwise the given name.

2.  Console modes

The console may be in one of two modes: command mode and converse  mode.   In

command  mode, the prompt net> is displayed and any of the commands described

in the Commands chapter may be entered.  In converse mode, keyboard input  is

processed according to the current session.

Sessions come in many types: Telnet, Ttylink, Rlogin, FTP, AX25, Finger, Com-

mand, NETROM, Ping, More, Dial, Dir, PPP PAP, Hopcheck and Tip.

In a Telnet, Ttylink, AX25, NETROM, Rlogin, or Tip session, keyboard input is

sent  to the remote system and any output from the remote system is displayed

on the console.  In an FTP session, keyboard input is first examined  to  see

if  it is a known local command; if so it is executed locally.  If not, it is

"passed through"  to  the  remote  FTP  server.   (See  the  FTP  Subcommands

chapter).   In a Ping session the user may test the path to a remote site, in

a More session, the user may examine a local file. A Hopcheck session is used

to trace the path taken by packets to reach a specified destination. A Finger

session is used to peek at a remote system for its users (and what  they  are

doing  on  some  extended  responses from UNIX systems). PPP PAP is used as a

link setup like slip between two systems.

The keyboard also has cooked and raw  states.   In  cooked  state,  input  is

line-at-a-time;  the  user may use the line editing characters ^U, ^R, ^B, ^W

and backspace to erase the line, redisplay the line, redisplay the  remainder

of  the  previous  line,  erase  last  word  and  erase  the  last character,

- 4 -

respectively.  Hitting either return or line feed passes the complete line up

to the application.  In raw mode, each character is immediately passed to the

application as it is typed.  The keyboard is always in cooked state  in  com-

mand  mode.   It  is  also cooked in converse mode on an AX25, FTP or NET/ROM

session.  In a Telnet or Ttylink session it depends on whether the remote end

has  issued  (and  the  local  end has accepted) the Telnet WILL ECHO option.

(See the echo command).

On the IBM-PC, the user may escape back to command mode by  hitting  the  F10

key  or  the  escape  key.   On other systems, the user must enter the escape

character, which is by default control-] (hex 1d, ASCII GS). (Note that  this

is  distinct from the ASCII character of the same name). The escape character

can be changed (see the escape command). The F10 key can  be  redefined  with

the  fkey  command  so the user is now warned to leave one escape possibility

open for himself.  Setting both F10 and escape to unreachable codes renders a

system unescapable and the user hung in a session.

In the IBM PC version, each session (including the command "session") has its

own  screen.   When a new session is created, the command display is saved in

memory and the screen is cleared.  When the command escape key  (usually  F10

or  ^]) is hit, the current session screen is saved and the command screen is

restored.  When a session is resumed, its screen is restored  exactly  as  it

appeared  when  it was last current.  NosFp expects that the driver NANSY.SYS

is loaded to support the screen and terminal emulation routines. ANSY.SYS has

many problems and should NOT been used. Version 24 of nansy.sys (nan24hyc.zip

on download systems) should be used  for  best  results.  Users  of  DeskView

should  be  ok  with dvansy.sys as it has been reported that tab expansion is

all right under DV.

3.  Commands

This section describes the commands recognized in command mode, or  within  a

startup  file  such  as autoexec.nos.  These are given in the following nota-

tion:

command

command literal_parameter

command subcommand <parameter>

command [<optional_parameter>]

command a | b

Many commands take subcommands  or  parameters,  which  may  be  optional  or

required.  In  general,  if a required subcommand or parameter is omitted, an

error message will summarize the available subcommands  or  required  parame-

ters.   (Giving  a '?' in place of the subcommand will also generate the mes-

sage.  This is useful when the command word alone is a valid command.)  If  a

command  takes  an  optional value parameter, issuing the command without the

parameter generally displays the current value of the  variable.  (Exceptions

to this rule are noted in the individual command descriptions.)

Two or more parameters separated by vertical bar(s) denote a  choice  between

the  specified values.  Optional parameters are shown enclosed in [brackets],

and a parameter enclosed in <angle  brackets>  should  be  replaced  with  an

actual value or string.  For example, the notation <hostid> denotes an actual

host or gateway, which may be specified in one of two ways:  as a numeric  IP

address  in  dotted  decimal  notation (eg. 44.0.0.1.), or as a symbolic name

listed in the file domain.txt,

All commands and many subcommands may be  abbreviated.  You  only  need  type

enough  of a command's name to distinguish it from others that begin with the

- 5 -

same series of letters. Parameters, however, must be typed in full.

Certain FTP subcommands (eg. put, get, dir, etc) are recognized only in  con-

verse  mode with the appropriate FTP session; they are not recognized in com-

mand mode.  (See the FTP Subcommands chapter.)

3.1.  <CR>

Entering a carriage return (empty line) while in command  mode  puts  you  in

converse  mode  with the current session. If there is no current session, Nos

remains in command mode and reissues the net> prompt.

3.2.  !

An alias for the shell command.

3.3.  #

Commands starting with the hash mark (#) are ignored. This is  mainly  useful

for comments in the autoexec.nos file.

3.4.  abort [<session #>]

Abort a FTP get, put or dir operation in progress. If issued without an argu-

ment,  the  current  session is aborted. (This command works only on FTP ses-

sions.) When receiving a file, abort simply resets the data  connection;  the

next  incoming  data packet will generate a TCP RST (reset) response to clear

the remote server.  When sending a file, abort sends a premature end-of-file.

Note  that  in  both cases abort will leave a partial copy of the file on the

destination machine, which must be removed manually if it is unwanted.

3.5.  arp

Display the Address Resolution Protocol table that maps IP addresses to their

subnet  (link) addresses on subnetworks capable of broadcasting.  For each IP

address entry the subnet type (eg. Ethernet, AX.25), subnet address and  time

to  expiration is shown. If the link address is currently unknown, the number

of IP datagrams awaiting resolution is also shown.

3.5.1.  arp add <hostid> ether |  ax25  |  netrom  |  arcnet  <ether_addr>  |

<ax25_addr>

Add a permanent entry to  the  table.  It  will  not  time  out  as  will  an

automatically-created entry, but must be removed with the arp drop command.

3.5.2.  arp drop <hostid> ether | ax25 | netrom | arcnet

Delete a permanent entry from the arp table.

3.5.3.  arp flush

Drop all automatically-created entries in the ARP  table;  permanent  entries

are not affected.

3.5.4.  arp publish <hostid> ether | ax25 | netrom |  arcnet  <ether_addr>  |

<ax25_addr>

This command is similar to the arp add command,  but  the  system  will  also

respond  to  any  ARP request it sees on the network that seeks the specified

address.  (Use this feature with great care.)

- 6 -

3.6.  autoroute <yes|no>

Displays or sets the IP autorouting option. When set all AX25 IP packets  are

analysed and remembered.

3.7.  asystat

Display statistics on attached asynchronous communications  interfaces  (8250

or  16550A),  if  any. The display for each port consists of three lines. The

first line gives the port label and the configuration flags;  these  indicate

whether  the port is a 16550A chip, the trigger character if any, whether CTS

flow control is enabled,  whether  RLSD  (carrier  detect)  line  control  is

enabled,  and the speed in bits per second.  (Receiving the trigger character

causes the driver to signal upper layer software that data is  ready;  it  is

automatically  set  to  the appropriate frame end character for SLIP, PPP and

NRS lines.)

The second line of the status display shows receiver (RX) event  counts:  the

total  number  of  receive interrupts, received characters, receiver overruns

(lost characters) and the receiver high water mark.  The high water  mark  is

the  maximum  number  of characters ever read from the device during a single

interrupt. This is useful for monitoring system interrupt latency margins  as

it  shows how close the port hardware has come to overflowing due to the ina-

bility of the CPU to respond to a receiver interrupt in time. 8250 chips have

no  FIFO,  so  the  high  water  mark cannot go higher than 2 before overruns

occur. The 16550A chip,  however,  has  a  16-byte  receive  FIFO  which  the

software  programs  to  interrupt  the CPU when the FIFO is one-quarter full.

The high water mark should typically be 4 or 5 when a 16550A is used;  higher

values  indicate  that  the  CPU  has at least once been slow to respond to a

receiver interrupt.

When the 16550A is used, a count of FIFO timeouts is also displayed on the RX

status line. These are generated automatically by the 16550A when three char-

acter intervals go by with more than 0 but less  than  4  characters  in  the

FIFO. Since the characters that make up a SLIP or NRS frame are normally sent

at full line speed, this count will usually be a lower bound on the number of

frames  received  on the port, as only the last fragment of a frame generally

results in a timeout (and then only when the frame is not  a  multiple  of  4

bytes long.)

Finally, the software fifo overruns and high water mark are displayed.  These

indicate  whether  the  <bufsize> parameter on the attach command needs to be

adjusted (see the Attach Commands chapter).

The third line shows transmit (TX) statistics, including  a  total  count  of

transmit interrupts, transmitted characters, the length of the transmit queue

in bytes, the number of status interrupts, and the number of  THRE  timeouts.

The  status interrupt count will be zero unless CTS flow control or RLSD line

control has been enabled.  The THRE timeout is a  stopgap  measure  to  catch

lost  transmit  interrupts,  which  seem  to  happen  when  there is a lot of

activity (ideally, this will be zero).

3.8.  attach <hw type> ...

Configure and attach a hardware interface to  the  system.  The  details  are

highly  interface  dependent and dependent on configuration flags in the file

config.h when the software is build. It can be that not  all  drivers  listed

below  will  be included in your copy of Nos.  Detailed instructions for each

driver are in the Attach Commands chapter.  Drivers  are  available  for  the

following hardware types:

- 7 -

3.8.1.  attach 3c500

Don't use this one anymore. Use the packet driver  instead.  This  driver  is

obsolete and not supported anymore.

3.8.2.  attach asy

Standard PC asynchronous interface (com port)  using  the  National  8250  or

16450 or 16550A chip or compatible equivalent.

3.8.3.  attach axip

A "wormhole" ax25 digipeater device.

3.8.4.  attach arcnet

A ARCnet driver via the PACKET driver.

3.8.5.  attach drsi

N6TTO driver for the DRSI PCPA 8530 card.

3.8.6.  attach eagle

WA3CVG/NG6Q driver for the Eagle Computer card (Zilog 8530).

3.8.7.  attach hapn

KE3Z driver for the Hamilton Amateur  Packet  Network  adapter  board  (Intel

8273).

3.8.8.  attach hs

Special "high speed" 8530 driver for the WA4DSY 56kb/s modem.

3.8.9.  attach kiss

This enables a multiplexed tnc type to be used for second channel. It is used

to  connect  a second port to an already attached asy interface. It will copy

most of the params of it's parent port.

3.8.10.  attach netrom

This is a psuedo interface to enable NET/ROM operations.

3.8.11.  attach packet

Driver for use with  separate  software  "packet  drivers"  meeting  the  FTP

Software, Inc, Software Packet Driver specification.

3.8.12.  attach pc100

Driver for the PACCOMM PC-100 (Zilog 8530) card.

3.8.13.  attach pi

Dma driven 8530 scc board from VE3IFB.

- 8 -

3.8.14.  attach scc

PE1CHL driver for generic 8530 cards.

3.8.15.  attach slfp

Serial Line Faming Protocol packet driver.

An easy way to obtain a summary of the parameters required for a given device

is  to  issue  a partial attach command (eg. attach packet.)  This produces a

usage message giving the complete command format.

3.9.  attended [off | on]

Displays or sets the global "I am present" flag in Nos. This flag is used  in

the welcome header by incoming ttylink connections.

3.10.  ax25 <subcommand>

These commands are for Ax25 interfaces.

3.10.1.  ax25 bc <interface>

The bc command enables broadcasts via interface interface.

3.10.2.  ax25 bcinterval [<seconds>]

The bcinterval displays or sets the time in seconds  between  bc  broadcasts.

On display both the interval and the countdown values are shown.

3.10.3.  ax25 blimit [<limit>]

Display or set the AX25 retransmission backoff limit. Normally  each  succes-

sive AX25 retransmission is delayed by twice the value of the previous inter-

val; this is called binary exponential backoff.  When the backoff reaches the

blimit  setting  it  is held at that value, which defaults to 30.  To prevent

the possibility of "congestive collapse" on a loaded channel,  blimit  should

be  set at least as high as the number of stations sharing the channel.  Note

that this is applicable only on actual AX25 connections; UI frames will never

be retransmitted by the AX25 layer.

3.10.4.  ax25 bctext ["broadcast text"]

The bctext command displays or sets the text to be send  for  broadcast  mes-

sages send out every bcinterval seconds.

3.10.5.  ax25 digipeat [on | off]

Display or set the digipeater enable flag. If the axip interface is used this

flag MUST be on , otherwise the digipeat function will not work at all.

3.10.6.  ax25 filter <0 | 1 | 2 | 3>

The filter commands enables or disables the logging in  the  heard  lists  of

source  and  destination  ax25_addresses. This is a bitwise or function where

the 01 value is for source stations and the 02  value  for  destination  sta-

tions. When the bit is off , logging is enabled, when on disabeled.

- 9 -

3.10.7.  ax25 flush

Clear the AX.25 "heard" list (see ax25 heard).

3.10.8.  ax25 heard [<interface>]

Display the AX.25 "heard" list. For each interface that is configured to  use

AX.25,  a  list  of all ax25_addresses heard through that interface is shown,

along with a count of the number of packets heard from each station  and  the

interval, in hr:min:sec format, since each station was last heard.  The local

station always appears first  in  the  listing;  the  packet  count  actually

reflects the number of packets transmitted. This entry is always present even

if no packets have been sent. If interface is given, only the heard list  for

that  interface  is  displayed.   Note that logging of heard stations is con-

trolled with the ax25 filter command.

3.10.9.  ax25 hearddest [<interface>]

Displays the destination list, i.e. the addressed to stations.  Next  to  the

time  the last transmission to that station the time that station replied (if

heard) is displayed. This gives a good reference  to  see  if  a  station  is

reachable and responding.

3.10.10.  ax25 irtt [<milliseconds>]

Display or set the initial value of smoothed round trip time to be used  when

a  new  AX25 connection is created. The value is in milliseconds.  The actual

round trip time will be learned by measurement once the connection  has  been

established.

3.10.11.  ax25 kick <axcb>

Force a retransmission on the specified AX.25  control  block.   The  control

block address can be found with the ax25 status command.

3.10.12.  ax25 maxframe [<count>]

Establish the maximum number of frames that will be allowed to remain  unack-

nowledged at one time on new AX.25 connections. This number cannot be greater

than 7.  Without count it will display the current  setting.  Note  that  the

maximum outstaning frame count only works with virtual connections. UI frames

are not affected.

3.10.13.  ax25 mycall [<ax25_addr>]

Display or set the default local AX.25 address.  The standard format is used,

(eg.  KA9Q-0 or WB6RQN-5).  This command must be given before any attach com-

mands using AX.25 mode are given.

3.10.14.  ax25 paclen [<size>]

Limit the size of I-fields on new AX.25  connections.   If  IP  datagrams  or

fragments  larger than this are transmitted, they will be transparently frag-

mented at the AX.25 level, sent as a series of I frames, and reassembled back

into a complete IP datagram or fragment at the other end of the link. To have

any effect on IP datagrams, this parameter should be less than  or  equal  to

the MTU of the associated interface.

- 10 -

3.10.15.  ax25 pthresh [<size>]

Display or set the poll threshold to be used for new AX.25 Version 2  connec-

tions.   The  poll  threshold controls retransmission behavior as follows. If

the oldest unacknowledged I-frame size is less than the  poll  threshold,  it

will  be  sent  with the poll (P) bit set if a timeout occurs.  If the oldest

unacked I-frame size is equal to or greater than the threshold, then a RR  or

RNR  frame,  as  appropriate, with the poll bit set will be sent if a timeout

occurs.

The idea behind the poll threshold is that the extra time needed  to  send  a

"small"  I-frame  instead of a supervisory frame when polling after a timeout

is small, and since there's a good chance the I-frame will have  to  be  sent

anyway  (i.e.,  if it were lost previously) then you might as well send it as

the poll. But if the I-frame is  large,  send  a  supervisory  (RR/RNR)  poll

instead  to  determine  first  if retransmitting the oldest unacknowledged I-

frame is necessary; the timeout might have been caused by a lost acknowledge-

ment.   This  is  obviously a tradeoff, so experiment with the poll threshold

setting. The default is 128 bytes, one half the default value of paclen.

3.10.16.  ax25 reset <axcb>

Delete the AX.25 connection control block at the specified address.

3.10.17.  ax25 retry [<count>]

Limit the number of successive unsuccessful retransmission  attempts  on  new

AX.25  connections.  If  this  limit  is  exceeded,  link re-establishment is

attempted. If this fails retry times, then the connection  is  abandoned  and

all queued data is deleted.

3.10.18.  ax25 route

Display the AX.25 routing table that specifies the digipeaters to be used  in

reaching a given station.

3.10.18.1.  ax25 route add <target> [digis ... ]

Add an entry to the AX.25 routing table.  An automatic ax25 route add is exe-

cuted  if  digipeaters are specified in an AX25 connect command, or if a con-

nection is received from a remote station  via  digipeaters.  Such  automatic

routing table entries won't override locally created entries, however.

3.10.18.2.  ax25 route drop <target>

Drop an entry for target from the AX.25 routing table.

3.10.18.3.  ax25 route mode <target> [vc | datagram | interface]

Sets the mode to vc | datagram |  interface  for  target.  Interface  is  the

default for that interface. Vc is a virtual circuit (ax25 connected mode) and

datagram is unconnected mode, (AX25 UI frames).

3.10.19.  ax25 status [<axcb>]

Without an argument, display a one-line summary of each AX.25 control  block.

If  the  address  of a particular control block is specified, the contents of

that control block are dumped in more detail. Note that the send queue  units

are frames, while the receive queue units are bytes.

- 11 -

3.10.20.  ax25 t3 [<milliseconds>]

Display or set the AX.25 idle "keep alive" timer. Value is in milliseconds.

3.10.21.  ax25 t4 [<seconds>]

Display or set the AX.25 Link "redundancy" timer. Value is in seconds.   When

no exchange has been had during this time the link is reset and closed.

3.10.22.  ax25 timertype [l | e | o]

Sets or displays the type of timer  used  for  retransmission  and  recovery:

linear, exponential or original.

3.10.23.  ax25 version [1 | 2]

Display or set the version of the AX.25 protocol to attempt  to  use  on  new

connections.  The  default is 1 (the version that does not use the poll/final

bits).

3.10.24.  ax25 window [<size>]

Set the number of bytes that can be pending on an AX.25 receive queue  beyond

which  I  frames  will  be  answered with RNR (Receiver Not Ready) responses.

This presently applies only to suspended interactive  AX.25  sessions,  since

incoming  I-frames  containing  network (IP, NET/ROM) packets are always pro-

cessed immediately and are not placed on the receive queue.  However, when an

AX.25  connection carries both interactive and network packet traffic, an RNR

generated because of backlogged interactive traffic will  also  stop  network

packet traffic from being sent.

3.11.  bbs

Enter the local bbs port (same as a telnet session to your own station).

Current commands are:

help      ? <command>

area      a [<area>]

bye       b

connect   c <node>

c <interface> <ax25_addr> [[<ax25_addr>] .... ]

download  d <filename>

du <filename>         (for uuencode bin file)

escape    e [<escape char.]

finger    f [<user>]

help      h [<command>]

info      i

jheard    j

kill      k <msg number>

list      l [<msg number>]

mbusers   m

nodes     n <node>

operator  o

ports     p

read      r [<msg number>]

send      s[f] <usename[@hostid] [<from_addr>] [$bul_id]

send repl sr [<mgs number>]

telnet    t <hostid>

upload    u <filename>

verbose   v <n>

what      w [<directory>]

expert    x

- 12 -

zap       z <filename>

sysop     @

3.12.  bootp <subcommand>

This is a bootp server / client, included into Nos. It is picked up from  the

University  of  Michigan in Ann Arbor. It is included into the sources of Nos

but is not used nor tested by me. (PA0GRI) Its usability  is  fague  but  the

discussions  on  tcp-group drove me to include this for possible use of reus-

able addresses.

3.12.1.  bootp start

Starts the bootp server.

3.12.2.  bootp stop

Stops the bootp server.

3.12.3.  bootp dns [<ipaddr>]

Display or set the list of domain name servers for bootp.

3.12.4.  bootp dyip [<iface> | <iface> <ipaddr1> <ipaddr2> | <iface> off]

Display or set the interface address range. The range is between ipaddr1  and

ipaddr2, both in dot notation.

3.12.5.  bootp host [<hostaddr> <hardware type>  <hardware  addr>  <ip  addr>

[boot file]]

Display or activate a bootp process. Hardware type is netrom,  ether,  macap-

pletalk  or ax25. Hardware addr is the interface name. Ip addr must be in dot

notation.

3.12.6.  bootp rmhost <ipaddr>

Delete ipaddr from the host table.

3.12.7.  bootp homedir [<directory> | default]

Display or set the directory where the bootp files reside. The default direc-

tory is bpfiles.

3.12.8.  bootp defaultfile [<bootfile> | default]

Display or set the file name of the bootp file. The default is boot.

3.12.9.  bootp logfile [<filename> | default] [on | off]

Starts or stops the logging of bootp requests to filename or the default file

name bootplog.

3.12.10.  bootp logscreen [on | off]

Enables or disables the logging of bootp to the screen.

3.13.  bootpd

This starts the server daemon for bootp.

- 13 -

3.14.  cd [<dirname>]

Change the current working directory, and display the new  setting.   Without

an  argument,  cd  simply displays the current directory without change.  The

pwd command is an alias for cd.

3.15.  close [<session>]

Close the specified session; without an argument, close the current  session.

On an AX.25 session, this command initiates a disconnect.  On a FTP or Telnet

session, this command sends a FIN (i.e., initiates a close) on the  session's

TCP  connection.   This is an alternative to asking the remote server to ini-

tiate a close (QUIT to FTP, or the logout command appropriate for the  remote

system  in  the case of Telnet).  When either FTP or Telnet sees the incoming

half of a TCP connection close, it automatically responds by closing the out-

going half of the connection.  Close is more graceful than the reset command,

in that it is less likely to leave the remote TCP in a "half-open" state.

3.16.  cls

Clears the current session screen (command screen).

3.17.  comm <interface> <text-string>

The comm command sends text-string via interface.  This can be used  to  send

straight  text  to an tnc still in TAPR command mode during Nos startup. Note

that to preserve spaces tabs etc. to include the string between double  quote

characters. Aka: comm ax1 "start kiss"

3.18.  connect <iface> <ax25_addr> [<digipeater> ... ]

Initiate a "vanilla" AX.25 session  to  the  specified  ax25_addr  using  the

specified interface. Data sent on this session goes out in conventional AX.25

packets with no upper layer protocol.   The  de-facto  presentation  standard

format  is  used, in that each packet holds one line of text, terminated by a

carriage return.  A single AX.25 connection  may  be  used  for  terminal-to-

terminal,  IP  and  NET/ROM traffic.  The three types of data being automati-

cally separated by their AX.25 Level 3 Protocol IDs.

Up to 7 optional digipeaters may be given; note that  the  word  via  is  NOT

needed.  If  digipeaters  are  specified, they are automatically added to the

AX25 routing table as though the ax25 route add command had been given before

issuing the connect command.

3.19.  delete <filename>

The filename is removed from the file system.

3.20.  detach <iface>

Detach a previously attached interface from the system. All IP routing  table

entries referring to this interface are deleted, and forwarding references by

any other interface to this interface are removed.

3.21.  dialer <iface> [<file> [<seconds> [<pings> [<hostid>]]]]

Setup an autodialer session for the interface.   Whenever  the  interface  is

idle  for  the  interval in <seconds>, the autodialer will ping the <hostid>.

If there is no answer after <pings> attempts, the autodialer will execute the

special commands contained in the <dialer-file>.

- 14 -

If the interval in <seconds> is zero, a previous dialer command process  will

be removed.  If the number of <pings> is zero, the <dialer-file> will be exe-

cuted without pinging the <hostid>.

The file may have any valid name, and must be located  in  the  configuration

root  directory  (see  the Installion section).  The commands in the file are

described in the Dialer Subcommands chapter.

Commands in file are:

3.21.1.  control <up | down>

3.21.2.  send <string> [<milliseconds>]

Sends string to the interface. If milliseconds is given, inter character tim-

ing is milliseconds milliseconds.

3.21.3.  speed <bps>

Displays or sets the current interface speed to bps baud.

3.21.4.  status <up | down>

3.21.5.  wait <milliseconds> [<string> [speed]]

Wait the amount of milliseconds. If string is given, incoming characters from

the  interface  are compared with string. If an compare is found and speed is

the string speed, the next numbers read from the interface is  the  new  bau-

drate  used.  This  works  like HAYES response CONNECT 9600. The wait command

could habe been "wait 10000 CONNECT speed".  This waits 10  seconds  for  the

CONNECT response from the modem.

3.22.  dir [<dirname>]

List the contents of the specified directory on the console. If  no  argument

is  given,  the  current directory is listed. Note that this command works by

first listing the directory into a temporary file, and then creating  a  more

session to display it. After this completes, the temporary file is deleted.

3.23.  disconnect [<session>]

An alias for the close command (for the benefit of AX.25 users).

3.24.  domain <subcommand>

The domain commands control and show the working  of  the  name  to  internet

address  mapping software. NOS currently only has a client with a simple file

reading local server. A real server is needed to service  the  community  for

their growing needs.

3.24.1.  domain addserver <hostid>

Add a domain name server to the list of name servers.  Note that,  when  this

command is given in the autoexec.nos file the ip address command should given

be before this command is used.  (If not, Nos will not kow how to resolve the

address,  and  an answer will never be recognized, or worse: just plain hangs

the system.)

- 15 -

3.24.2.  domain cache <subcommand>

Following commands work on the domain cache. These are resource records  (see

RFC 1033/1034) held in memory.

3.24.2.1.  domain cache clean [<yes | no>]

Displays or sets the discard of expired  resource  records.  Expired  records

have  their timeout value decremented to zero. Normaly resource records get a

default timeout value of 1800 seconds. After this time  they  are  considered

"old"  and  if  referenced  again the domain name resolver should be enquired

again.  When clean is off (the default), expired records will be retained; if

no replacement can be obtained from another domain name server, these records

will continue to be used.

When clean is on, expired records will be removed from the file whenever  any

new record is added to the file.

3.24.2.2.  domain cache list

This command shows the current content of the in memory  cache  for  resource

records.

3.24.2.3.  domain cache size [<size>]

Display or set the nominal maximum size  of  the  local  memory  cache.   The

default is 20.

(Note: The cache may be temporarily larger when waiting for new records to be

written to the domain.txt file.)

3.24.2.4.  domain cache wait [<seconds>]

Display or set the interval in seconds to wait for additional activity before

updating the domain.txt file.  The default is 300 seconds (5 minutes).

3.24.3.  domain dropserver <hostid>

Remove a domain name server from the list of name servers.   You  are  warned

when you delete the last name server.

3.24.4.  domain listservers

List the currently configured domain name servers, along with  statistics  on

how  many  queries  and  replies  have been exchanged with each one, response

times, etc.

3.24.5.  domain maxwait [<timeout>]

This sets a timeout value (1 to 255  seconds)  to  a  query  or  domain  name

server.   This is not set for a already defined server but will be used for a

newly defined name server. Also the value is used for domain nslookups.  Note

that  name  servers  can  have (PC based) trouble finding records in an large

database. The default is set to 30 seconds.

3.24.6.  domain retry [<retries>]

The retry value (number) limits the number of  queries  send  out  to  remote

domain   name   resolvers   before  giving  up  and  telling  you  that  host

xyzzy.ampr.org does not exist. The total time lost with a query is retries  *

timeout * number of domain servers defined.

- 16 -

3.24.7.  domain suffix [<domain suffix> | none]

Display or specify the default domain name suffix to be appended  to  a  host

name  when  it  contains  no  periods.  For  example, if the suffix is set to

ampr.org. and the user enters telnet ka9q, the domain resolver  will  attempt

to  find  ka9q.ampr.org..  If the host name being sought contains one or more

periods, however, the default suffix is NOT applied if the last part  of  the

name is less than 5 characters and contains only letters; e.g.,

telnet foo.bar would NOT be turned into foo.bar.ampr.org..

telnet foo.ka9q will be turned into foo.ka9q.ampr.org..  Note that a  trail-

ing  dot  (.)  is  required for the suffix.  If the suffix is the string none

(without trailing period) the current suffix is cleared and forgotten.

3.24.8.  domain trace [on | off]

Display or set the flag controlling the tracing of domain server requests and

responses.  Trace messages will be seen only if a domain name being sought is

not found in the local cache file, domain.txt.

3.24.9.  domain translate [off | on]

Display or set the flag that controls the translation of  ip  adress  in  dot

notation  into  symbolic  names. The translation process makes heavely use of

reverse domain name lookups. Do not set this flag unless you have a good  and

fast connection to a domain name server or have a fast domain.txt handler and

domain.txt contains all IN-ADDR.ARPA. records you ever wanted.

3.24.10.  domain verbose [off | on]

Display or set the flag controlling the return of a full name (true) or  only

the first name (dot delimiter) (false). This is for IP address to name trans-

lation only.

3.25.  drsistat

Shows the statistics for all configured drsi boards.

3.26.  dump <hex-address | .> [decimal-range]

The dump command shows memory in hex and ascii. Hex-address is a 32 bit value

for  a  PC  split into page address and page offset. A splitting colon is not

used nor accepted. If decimal-range is not given , 128 bytes  are  displayed.

dump . displayes memory starting at the end of a previous dump command.

3.27.  echo [accept | refuse]

Display or set the flag controlling client Telnet's response to a remote WILL

ECHO offer.

The Telnet presentation protocol specifies that in the  absence  of  a  nego-

tiated  agreement  to the contrary, neither end echoes data received from the

other.  In this mode, a Telnet client session echoes keyboard  input  locally

and  nothing  is  actually  sent until a carriage return is typed. Local line

editing is also performed: backspace deletes the last character typed,  while

control-U deletes the entire line.

When communicating from keyboard to keyboard the standard local echo mode  is

used, so the setting of this parameter has no effect. However, many timeshar-

ing systems (eg. UNIX) prefer to do their own echoing of typed input.   (This

makes  screen  editors  work  right, among other things). Such systems send a

Telnet  WILL  ECHO  offer  immediately  upon  receiving  an  incoming  Telnet

- 17 -

connection request. If echo accept is in effect, a client Telnet session will

automatically return a DO ECHO response. In  this  mode,  local  echoing  and

editing is turned off and each key stroke is sent immediately (subject to the

Nagle tinygram algorithm in TCP).  While this mode is  just  fine  across  an

Ethernet, it is clearly inefficient and painful across slow paths like packet

radio channels. Specifying echo refuse causes an incoming WILL ECHO offer  to

be  answered with a DONT ECHO; the client Telnet session remains in the local

echo mode.  Sessions already in the remote echo mode are  unaffected.  (Note:

Berkeley  Unix  has  a  bug  in  that it will still echo input even after the

client has refused the WILL ECHO offer. To get around this problem, enter the

stty -echo command to the shell once you have logged in.)

3.28.  eol [unix | standard]

Display or set Telnet's end-of-line behavior when in remote  echo  mode.   In

standard  mode,  each  key  is sent as-is. In unix mode, carriage returns are

translated to line feeds.  This command is not necessary with all  UNIX  sys-

tems;  use  it  only  when you find that a particular system responds to line

feeds but not carriage returns.  Only SunOS release 3.2 seems to exhibit this

behavior; later releases are fixed.

3.29.  escape [<char>]

Display or set the current command-mode escape character in hex.  On the  PC,

the  escape  character  is default ^]. The alternate escape key is F10 unless

F10 is redefined with fkey.

3.30.  etherstat

Display 3-Com Ethernet controller statistics (if configured).

3.31.  exit

Exit the nos program and return to MS-DOS.

3.32.  finger <user@hostid> | <@hostid>

Issue a network finger request for user user at host hostid. This  creates  a

client  session  which  may  be interrupted, resumed, reset, etc, just like a

Telnet client session. If only @hostid is given, all users on that  host  are

identified.

- 18 -

3.33.  fkey [<number> [<string>]]

Fkey displays or sets values for the programmable keys on  the  PC  keyboard.

fkey  alone  gives  a  display  of all remapable keys and their number.  fkey

number displays the current value for that key.  fkey number  string  assigns

string  to that key. Control characters can be created by prefixing then with

an ^ character. A cr is ^M.  To insert an ^ in the string 2 ^'s next to  each

other are needed.  Following is the map of keys and their number. F1 is func-

tion key 1.  Sf1 is Shift function key 1. Cf1 is Control function key 1.  Af1

is Alt function key 1. Etc. The right most row is the numeric keypad.

key number   key  number   key  number   key  number  key   number

f1   59      sf1   84      cf1   94      af1  104     pgup   73

f2   60      sf2   85      cf2   95      af2  105     pgdn   81

f3   61      sf3   86      cf3   96      af3  106     home   71

f4   62      sf4   87      cf4   97      af4  107     end    79

f5   63      sf5   88      cf5   98      af5  108     arup   72

f6   64      sf6   89      cf6   99      af6  109     ardn   80

f7   65      sf7   90      cf7  100      af7  110     ar l   75

f8   66      sf8   91      cf8  101      af8  111     ar r   77

f9   67      sf9   92      cf9  102      af9  112     ins    82

f10  68      sf10  93      cf10 103      af10 113     del    83

The mapping is taken to  look  like  a  vt100  /  ansi  keyboard.   Currently

assigned value strings for following keys:

number  string     key

59     " 33OP"   /* F1 */

60     " 33OQ"   /* F2 */

61     " 33OR"   /* F3 */

62     " 33OS"   /* F4 */

71     " 10"     /* home*/

72     " 33[A"   /* up arrow*/

73     " 25"     /* pgup */

75     " 33[D"   /* left arrow */

77     " 33[C"   /* right arrow */

79     " 05"     /* end */

80     " 33[B"   /* down arrow */

81     " 12"     /* pgdn */

82     " 01"     /* ins */

83     " 177"    /* del */

- 19 -

3.34.  ftp <hostid>

Open an FTP control channel to the specified remote host and  enter  converse

mode  on  the  new  session.   Responses from the remote server are displayed

directly on the screen. See theFTP Subcommands chapter  for  descriptions  of

the commands available in a FTP session.

3.35.  ftype [ascii | binary | image | logical <size>]

This command displays or sets the default start file mode (ascii  or  binary)

for  ftp  transfers.  If  ftype binary or image is given the next ftp session

started will be in binary type. No binary command is needed once the  session

is started. In case of logical, the "word" size is set to size.

3.36.  help

Display a brief summary of top-level commands.

3.37.  hop <subcommands>

These commands are used to test the connectivity of the network.

3.37.1.  hop check <hostid>

Initiate a hopcheck session to the specified host. This uses a series of  UDP

"probe"  packets  with  increasing IP TTL fields to determine the sequence of

gateways in the path to the specified destination. This function is patterned

after the UNIX traceroute facility.

ICMP message tracing should be turned off before  this  command  is  executed

(see the icmp trace command).

3.37.2.  hop maxttl [<hops>]

Display or set the maximum TTL value to be used in hop check sessions.   This

effectively bounds the radius of the search.

3.37.3.  hop maxwait [<seconds>]

Display or set the maximum interval, in seconds, that a hopcheck session will

wait for responses at each stage of the trace. The default is 5 seconds.

3.37.4.  hop queries [<count>]

Display or set the number of UDP probes that will be sent at  each  stage  of

the trace. The default is 3.

3.37.5.  hop trace [on | off]

Display or set the flag that controls the display of  additional  information

during a hop check session.

3.38.  hostname [<name>]

Display or set the local host's name. By convention this should be  the  same

as  the  host's primary domain name. This string is used only in the greeting

messages of the various network servers;  note  that  it  does  NOT  set  the

system's IP address.

If <name> is the same as an <iface> (see the Attach commands  chapter),  this

command  will  search for a CNAME domain resource record which corresponds to

- 20 -

the IP address of the <iface>.

3.39.  hs

Display statistics about the HS high speed HDLC  driver  (if  configured  and

active).

3.40.  icmp <subcommand>

These commands are used for the Internet Control Message Protocol service.

3.40.1.  icmp echo [on | off]

Display or set the flag controlling the asynchronous  display  of  ICMP  Echo

Reply packets.  This flag must be on for one-shot pings to work (see the ping

command.)

3.40.2.  icmp status

Display statistics  about  the  Internet  Control  Message  Protocol  (ICMP),

including the number of ICMP messages of each type sent or received.

3.40.3.  icmp trace [on | off]

Display or set the flag controlling the display of ICMP error messages. These

informational messages are generated by Internet routers in response to rout-

ing, protocol or congestion problems. This option should be turned off before

using  the  hop  check  facility because it relies on ICMP Time Exceeded mes-

sages, and the asynchronous display of these messages will  be  mingled  with

hop check command output.

3.41.  ifconfig

Display a list of interfaces, with a short status for each.

3.41.1.  ifconfig [<iface> [[[ <subcommand>  <param>]  <subcommand>  <param>]

.... ]

When only iface is given, a extended interface status is displayed.  Multiple

subcommand / parameter can be put on one line.

3.41.2.  ifconfig <iface> broadcast <addr>

Set the broadcast address of interface iface to addr.  Addr can eighter be an

ax25_addr  or an ether_addr, depending on the interface type, with 1's in the

host part of the address.  This is related to the netmask  sub-command.   See

also the arp command.

3.41.3.  ifconfig <iface> description ["description"]

This command sets the interface description to the string  specified.  If  no

string  is  supplied,  the current description is cleared. The description is

displayed with the ifconfig iface command (no parameters) and with the  mail-

box commands.

3.41.4.  ifconfig <iface> encapsulation <slip | ax25 | ether | encap | ppp>

Sets the encapsulation for interface iface to slip / ax25 / ether /  encap  /

ppp.

- 21 -

3.41.5.  ifconfig <iface> forward <iface-2>

When a forward is defined, all output for interface iface  is  redirected  to

the  interface  directed by iface-2.  To remove the forward, set <iface-2> to

<iface>.

3.41.6.  ifconfig <iface> ipaddress <addr>

Set the IP address to addr for this interface. This might be nessesary when a

system  acts  as  a gateway. Like an system with IP address 44.137.1.8 has an

Internet  access  via  its  ethernet.  The  Internet  IP  address  could   be

129.179.122.10.  An  ifconfig  ec0  ipaddress 129.179.122.10 sets the correct

address for that interface. Now routing to that system will work.  (Note that

the  44.x.x.x  address is NOT connected to the Internet.)  See also the host-

name and ip address commands.

3.41.7.  ifconfig <iface> linkaddress <hardware-dependant>

Set the hardware dependant address for this interface.  For AX.25 this can be

the callsign, for ethernet a new ethernet address.

3.41.8.  ifconfig <iface> mtu <param>

Set the maximum transfer unit to param octeds (bytes).  See the  Setting  ...

MTU, MSS and Window chapter for more information.

3.41.9.  ifconfig <iface> netmask <address>

Set the sub-net mask for this interface.  The <address> takes the form of  an

IP  address  with 1's in the network and subnet parts of the address, and 0's

in the host part of the address.  Sample:  ifconfig  ec0  netmask  0xffffff00

for  a  class  C  network  (24  bits).  This is related to the broadcast sub-

command.  See also the route command.

3.41.10.  ifconfig <iface> rxbuf <size>

Set the receive buffer size.

3.42.  info

Info gives information about the version of Nos  currently  running  and  its

buildin  configuration.  The  configuration  info  is  build  with defines in

config.h in the source distribution. That way it gives automatically  correct

configuration information.

3.43.  ip <subcommand>

These commands are used for the Internet Protocol service.

3.43.1.   ip  access  <permit|deny|delete>  <dest   addr>[/<bits>]   <ifname>

[lowport [higport]]

Set or display access control for IP routing functions. This  command  imple-

ments  router  access functions to NosFp. Permit enables dest-addr packets to

be routed via ifname. Deny disables those. If lowport is not given, all ports

are  assumed. If only lowport is given, that port is only checked for permis-

sion If  lowport  and  highport  are  given,  that  is  the  range  of  ports

permitted/denied.   Dest-addr can be the word all for all addresses possible.

Lowport can be the word none for all ports.  The ip access delete must  match

a previous defined permit or deny to be able to delete that definition.  Some

samples:

- 22 -

ip access permit 44/8 ax0

ip access deny all ax0 1 1023

ip access permit all ax0 If no access list is created, all interfaces  might

carry  all types. If an access control is defined for an interface there must

be an permit defined for that interface to  allow  trafic.  Thus  an  partial

denyal without an permit is an complete denyal.

3.43.2.  ip address [<hostid>]

Display or set the default local IP  address.  This  command  must  be  given

before  an  attach  command if it is to be used as the default IP address for

the interface.

3.43.3.  ip rtimer [<seconds>]

Display or set the IP reassembly timeout. The default is 30  seconds.   Value

is in seconds.

3.43.4.  ip status

Display Internet Protocol (IP) statistics, such as total  packet  counts  and

error counters of various types.

3.43.5.  ip ttl [<hops>]

Display or set the default time-to-live value  placed  in  each  outgoing  IP

datagram.  This limits the number of switch hops the datagram will be allowed

to take. The idea is to bound the lifetime of the  packet  should  it  become

caught  in  a routing loop, so make the value slightly larger than the number

of hops across the network you expect to transit packets.  The default is set

at compilation time to the official recommended value for the Internet.

3.44.  isat [on | off]

Display or set the AT flag.  Normaly this flag is set when  an  interface  is

attached  with an interrupt of 8 or higher. This is to signal that the second

interrupt controllerc in a AT also needs an return of interrupt  signal.   If

an  AT  type  clock is in use, this command will allow measurement of time in

milliseconds, rather than clock ticks (55 milliseconds per clock tick).  Dur-

ing  I/O initialisation this flag is set if the monitor prom has the standard

byte 0xfc at address f000:fffe.

3.45.  kick [<session>]

Kick all sockets associated with a session; if no argument is given, kick the

current  session.   Performs  the same function as the ax25 kick and tcp kick

commands, but is easier to type.

3.46.  lock [password <"password string">]

Locks the keyboard or defines a password string.  If password is  given  then

<password> is saved as the unlock string.  If no parameters are supplied, the

keyboard becomes locked when a password was specified earlier.  If  the  key-

board  is  locked,  the password is requested.  If a correct password is sup-

plied, the keyboard becomes unlocked.  The setting of the password and  lock-

ing  of  the  keyboard  can  only been done by the system console keyboard or

autoexec.nos startup file.  The password can not been shown.

- 23 -

3.47.  log [stop | <filename>]

Display the current log filename or set the filename for logging server  ses-

sions.  If  stop is given as the argument, logging is terminated (the servers

themselves are unaffected).  If a file name is given as an  argument,  server

session log entries will be appended to it.

3.48.  lzw [<subcommand>]

Lzw is the data compression capability on  some  sockets.  This  command  set

defines or changes their definition. Normaly leave this set default.

3.48.1.  lzw mode <fast|compact>

Displays or sets  the  compression  methode  used  on  data  compression  for

specific sockets. Currently SMTP can use compression.

3.48.2.  lzw bits <number>

Displays or sets the number of bits used for the compression size.  The  more

bits defined the larger the table space needed. Range is 9 to 16.

3.49.  mail

This command will start a shell escape command. The mailer  used  is  defined

with the DOS environment variable MAILER, wich defaults to BM.EXE.

3.50.  mbox [<subcommand>]

Display the status of the mailbox server system (if configured).

3.50.1.  mbox attend [yes | no]

Displays or sets the attended flag on. This is used to announce in the  mail-

box if the station mannager is willing to attend his station (chat).

3.50.2.  mbox expert <on|off>

Displays or set the level expected of a mailbox user. When set (s)he  gets  a

short  ( > ) prompt. When not set a long prompt is send with the first letter

of each mailbox command as prompt. The user can then set Xpert  mode  to  get

the short prompt.

3.50.3.  mbox fwdinfo ["forward info"]

Displays or sets the mailbox forward info to be included in the R:  line  for

forwarded BBS bulletins.  An empty string ("") as info clears the info field.

Sample: netrom fwdinfo "HNLNET BBS"

Will show '[HNLNET BBS]' in the R: line.

3.50.4.  mbox haddress ["home-address"]

Displays or sets the home address field to be included in  the  R:  line  for

forwarded BBS bulletins.  An empty string ("") clears the field.

Sample: netrom haddress "#CRV.OR.USA"

Will show '@WG7J#CRV.OR.USA' in the R: line. (when ax mycall is WG7J)

- 24 -

3.50.5.  mbox jumpstart <on|off>

Displays or sets the mailbox jumpstart code. When set  and  an  "known"  node

connects to the mailbox, no extra line needs to be send to activate the mail-

box but the prompt is send directly. Warning: When set, it takes a  while  to

hear  all  nodes and an AX25 connect could have an wrong start as there is no

wait for a level 3 protocol check.  Especialy  RSPF  in  virtual  mode  could

course serious problems.

3.50.6.  mbox kick

To kick the mailbox back in activity after retry timeouts.

3.50.7.  mbox maxmsg

To display or set the maximum number of messages per area when  an  notesfile

is shown to a user. This reserves lots of memory for every mailbox session.

3.50.8.  mbox motd ["message string"]

Display ot set the mailbox welcoming Message Of The Day.

3.50.9.  mbox nrid <on|off>

DIsplays or sets the Netrom id flag. When set the node id is displayed on the

prompt line.

3.50.10.  mbox password <"password string">

Sets a password string to be presented to Sysops entering that  mode  in  the

mailbox  (using  the  @ command and having that priviledge set to their login

name/password in f/ftpusers). When a password is defined (max 30  characters)

then  (s)he  is  prompted with 5 numbers before letting her/him in.  The five

numbers represent the 5 character locations in the  string  defined,  whereby

the  first character is number 0.  Multiple lines of 5 characters can be send

to fool "snoopers".  The end of password sending is signaled  with  an  empty

line.  If  there  was a good response, sysop mode is entered.  The setting of

the password can only been done by the system console or autoexec.nos startup

file. The password can not been shown.

3.50.11.  mbox qth ["qth info"]

Sample: netrom qth "Driebruggen, NL"

Displays or sets the info for qth in the R: line for forwarded BBS messages.

3.50.12.  mbox secure <yes|no>

Displays or sets the security option for mailbox gateway users. If set  users

comming  in via telnet to the bbs are not allowed to use the gateway.  If not

set anybody can use the gateway. (Note: No check for Bozo's).  Also the mail-

box send command is disabled except for ax25 and netrom connects.

3.50.13.  mbox smtptoo <yes|no>

Displays or sets the flag to include SMTP headers in BBS messages.  When  set

SMTP headers are included in messages. When not set not included.

- 25 -

3.50.14.  mbox status

An alias for just entering mbox on the prompt line.

3.50.15.  mbox timer [<seconds>]

Display the current interval and time remainig or set the mailbox  forwarding

timer.

3.50.16.  mbox tiptimeout

Displays or sets the timeout value for tip connection timeout. After  timeout

seconds of no activity the connection is closed.

3.50.17.  mbox trace [yes | no]

Displays or sets mailbox forward trace code flag. The trace is  very  minimal

but every one working on mailbox/forward code now has a common flag Mtrace.

3.50.18.  mbox utc <offset>

Displays or sets the offset you are according  to  ZULU  time.  Positive  and

negative  numbers  are supported and full leap year and month calculation are

done. This is used by the mailbox forwarding to give a standard time  in  the

R: line.

3.50.19.  mbox zipcode zip

Displays or sets the info for the zip field for R:  line  BBS  header  lines.

This  field  is  max 7 characters long as released. For USA the zip is only 6

numbers long. Netherlands has 4 figures a space and 2  letters.  Guess  every

PTT wants something they "invented" their own.

Sample: netrom zip "3465 TJ"    or:  netrom zip 54551

3.51.  memory <subcommand>

These commands are used for memory allocation.

3.51.1.  memory debug [on|off]

Displays or sets the memory allocator debug flag. If set debug information is

written  to the log file containing most flags and parameters from the memory

allocation routines.

3.51.2.  memory efficient [yes | no]

Displays or sets the search algorithm for buffer memory. When set the  search

is  always  started  from  the beginning of the free list. This is slower but

keeps memory fragmentation to a minimum. When clear search is started on  the

end,  accounting  for  more  memory  fragmentation but keeping speed. Roumers

state that you should include this command as the first line in autoexec.nos.

The current releases default sets efficient to yes.

3.51.3.  memory freelist

Display the storage allocator free list. Each entry consists  of  a  starting

address, in hex, and a size, in decimal bytes.

- 26 -

3.51.4.  memory ibufsize [<size>]

Display or set the size of the buffers in the  interrupt  buffer  pool.   The

size  should  be  set to the largest type of buffer plus a header size of 22.

For example: If your ax25 is the omly interface and a packet length of 256 is

defined,  the  ibufsize  should  be 256 + 10 * 6 + 22. The 10 * 6 is the ax25

header (source , destination and 8 digipeaters).

3.51.5.  memory minheap [<number>]

Displays or sets the minimum heap size to be allocated before shell  escaping

to  DOS.  This  assures a free heap so that Nos can run trough without gettig

short on memory for a while.

3.51.6.  memory nibufs [<number>]

Display or set the number of interrupt buffer pool buffers.  If the number of

buffers  is  set,  the  statistics in the memory status display are reset for

number of interrupt buffer fails. The minimum available value is set  to  the

requested  number  of buffers.  A roule of thumb for the number of buffers is

to watch the statistics and keep a minimum of 2  free  buffers.  Increase  or

decrease as required.

3.51.7.  memory sizes

Display a histogram of storage allocator request sizes. Each histogram bin is

a binary order of magnitude (i.e., a factor of 2).

3.51.8.  memory status

Display a summary of storage allocator statistics. The first line  shows  the

base address of the heap, its total size, the amount of heap memory available

in bytes and as a percentage of the total heap size, and the amount of memory

left  over  (i.e., not placed on the heap at startup) and therefore available

for shell subcommands.

The second line shows the total number of calls to allocate and  free  blocks

of  memory, the difference of these two values (i.e., the number of allocated

blocks outstanding), the number of allocation requests that were  denied  due

to  lack  of memory, and the number of calls to free() that attempted to free

garbage (eg. by freeing the same block twice or freeing a garbled pointer).

The third line shows the garbage collection status.  Garbage collection is  a

separate  process running every second. If memory is "low" garbage collection

routines are called in "Red alert" mode, else in "Yellow alert".

The fourth line shows the number of calls to malloc and  free  that  occurred

with  interrupts  off. In normal situations these values should be zero.  The

fourth line shows statistics for the special pool of fixed-size buffers  used

to satisfy requests for memory at interrupt time. The variables shown are the

number of buffers currently in the  pool,  their  size,  and  the  number  of

requests that failed due to exhaustion of the pool.

The fifth line shows the current setting of the interrupt  buffer  pool,  its

minimal  value  and the number of no buffers available.  These statistics are

reset when a memory nibufs <number> command is given.

3.51.9.  memory thresh [<size>]

Displays or sets the memory threshold size in bytes.  If  memory  gets  below

this value , no more new sessions are started or accepted.

- 27 -

3.52.  mkdir <dirname>

Create a sub-directory in the current working directory.

3.53.  mode <iface> [vc | datagram]

Control the default transmission mode on the specified AX.25  interface.   In

datagram mode, IP packets are encapsulated in AX.25 UI frames and transmitted

without any other link level mechanisms, such as connections or  acknowledge-

ments.

In vc (virtual circuit) mode, IP packets are encapsulated in AX.25  I  frames

and are acknowledged at the link level according to the AX.25 protocol.  Link

level connections are opened if necessary.

In both modes, ARP is used to map IP to AX.25 addresses.  The defaults can be

overridden  with the type-of-service (TOS) bits in the IP header.  Turning on

the "reliability" bit causes I frames to be used, while turning on  the  "low

delay"  bit uses UI frames.  (The effect of turning on both bits is undefined

and subject to change).

In both modes, IP-level fragmentation is done if the datagram is larger  than

the interface  MTU.  In virtual circuit mode, however, the resulting datagram

(or fragments) is further fragmented at the AX.25 layer if it (or  they)  are

still  larger  than  the  AX.25  paclen  parameter.  In  AX.25 fragmentation,

datagrams are broken into several I frames and reassembled at  the  receiving

end  before  being passed to IP. This is preferable to IP fragmentation when-

ever possible because of decreased overhead (the IP header isn't repeated  in

each  fragment)  and  increased  robustness  (a  lost fragment is immediately

retransmitted by the link layer).

3.54.  more <file> [<file> ...]

Display the specified file(s) a screen at a time.  To  proceed  to  the  next

screen,  press  the  space bar; to cancel the display, hit the 'q' key.  A CR

advances a line.  The more command creates a session that you can suspend and

resume just like any other session.

3.55.  motd [<"message of the day">]

Display the current message or  sets  the  welcome  message  to  the  defined

string.

3.56.  multitask [on | off]

Displays or sets the multi task flag. If set Nos  continues  to  service  its

interrupts  and  handling  of  normal operation when put in the background by

executing a shell escape. Some programs do not like  to  miss  valuable  time

taken away this way and weard results can happen. Clearing the flag stops all

activiity fron Nos when put in the background.

3.57.  netrom <subcommand>

3.57.1.  netrom acktime [<milliseconds>]

Displays or sets the ack delay timer, similary to ax25 t2.

- 28 -

3.57.2.  netrom alias <alias>

This sets the netrom alias for this station. Other stations  can  connect  to

the  ax25  callsign  and  to the Netrom alias (when set). The alias is broad-

casted with a Netrom broadcast.

3.57.3.  netrom bcnodes <iface>

Initiates an immediate broadcast of nodelist on iface.

3.57.4.  netrom call <call>

Displays or sets the call to be used by the netrom interface. Note: this is a

shortcut for the "ifconfig netrom linkaddress call" command.

3.57.5.  netrom connect <node>

Start a connection to node.

3.57.6.  netrom choketime [<milliseconds>]

Display or set the time breaking a send choke. Choke is the term netrom  uses

for flow control conditions.

3.57.7.  netrom derate [on | off]

Display or set automatic derating of netrom routes on link failure.

3.57.8.  netrom interface <iface> <quality>

Define a netrom interface iface.  The quality can be between 1 and  255.  For

compatibility  with other implementations set this to 192 normaly. A check is

made that the interface is of type CL_AX25,  meaning  NETROM  capable  media.

Note:  the alias is not specified on this line anymore. Use netrom alias com-

mand.

3.57.9.  netrom irtt [<milliseconds>]

Display or set the initial round trip time.

3.57.10.  netrom kick <&nrcb>

Give the control block a kick to get activity going again.

3.57.11.  netrom load [<filename>]

When fully implemented reloads a saved netrom  node  list  to  continue  from

where you left when you saved the list. All entry's read are time decremented

as if normal time just went on. If it took some time  (obsotimer  *  x)  your

list could be empty as they all timed out.

3.57.12.  netrom minquality [<value>]

Display or set the minimum quality for recognizing  a  node  entry.   Entry's

below this value are not considered valuable for usage.

3.57.13.  netrom nodefilter <subcommand>

Node filtering.

- 29 -

3.57.13.1.  netrom nodefilter add <neighbor> <iface>

Allow neighbor to give us node updates.

3.57.13.2.  netrom nodefilter drop <neighbor> <iface>

Disallow neighbor to give us node updates.

3.57.13.3.  netrom nodefilter mode [none | accept | reject]

Display or set the initial filter schema. None accepts  all.   Accept,accepts

only  from  nodes  defined in netrom nodefilter add lists.  Reject,no accepts

from nodes defined in netrom nodefilter add lists.

3.57.14.  netrom nodetimer [<seconds>]

Display or set the interval our node list is transmitted.

3.57.15.  netrom obsotimer [<seconds>]

Display or set the time a node quality gets decremented.

3.57.16.  netrom promiscious [off | on]

Enables nodes with a higher quality than defined with minquality.  If on, all

nodes are received, independent od nodefilter.

3.57.17.  netrom qlimit [<bytes>]

Display or set the maximum queue limit for chat. Similar to ax25 window.

3.57.18.  netrom reset <&nrcb>

Remove the control block. You can find the control block with socket.

3.57.19.  netrom retries [<value>]

Display or set the maximum number of retries on connect, disconnect or data.

3.57.20.  netrom route <subcommand>

Netrom routing commands.

3.57.20.1.  netrom route add <alias> <destination> <iface> <quality>  <neigh-

bor>

Add a netrom route.

3.57.20.2.  netrom route drop <destination> <neighbor> <iface>

Delete a netrom route.

3.57.20.3.  netrom route info <destination>

Display the route it would take to get to destination.

3.57.21.  netrom status

Displays all netrom connections.

- 30 -

3.57.22.  netrom save [<filename>]

When fully implemented, saves the current  netrom  node  list  in  memory  to

/netrom.sav or filename, if given.

3.57.23.  netrom timertype [exponential | linear]

Displays or sets the type of backoff.

3.57.24.  netrom ttl [<hops>]

Display or set the maximum number of hops a frame might hop before discarded,

if it did not reach its destination before.

3.57.25.  netrom user [<username>]

Display or set our netrom user name. This is used on outgoing connects.

3.57.26.  netrom verbose [off | on]

Display or set the verbose flag. If set all nodes known to us are transmitted

each  time  the  nodetimer  expires. If off, only your stations node ident is

send out.

3.57.27.  netrom window [<frames>]

Display or set the size of the sliding window. This is the largest  send  and

receive window we might negotiate.

3.58.  nntp <subcommand>

Network news transfer protocol has the following subcommands:

3.58.1.  nntp addserver <nntpserver> <seconds> [<range>] [<groups>]

Add a nntp news server to quiry  every  seconds  interval  for  new  articles

specified  by  the nntp groups command. Range can be a limit in time to query

like nntp addserver someserver 600 22:00-23:00,  to  only  query  the  server

between 10 and 11pm.  Multiple nntp add can be used co concatenate groups (up

to 512 bytes) The interval seconds can be set to 0, so that normaly the  nntp

client  does not query the server regulary but an nntp kick nntpserver can be

used to start a session.

3.58.2.  nntp directory <directory>

Show or set default directory for spooling news.  This  is  not  the  control

directory but the (alternate) place to spool articles to.

3.58.3.  nntp dropserver <nntpserver>

3.58.4.  nntp groups <group> [<group> ...]

Set or display currently set newsgroups.

3.58.5.  nntp  kick <nntpserver>

Kick the client to get in touch with the named server.

- 31 -

3.58.6.  nntp listservers

Lists the curently defined servers.

3.59.  nntp quiet [yes | no]

Shows or sets the flag controlling the display of newly  received  nntp  news

messages.  The  smpt  quiet  fag controls the beep following the message.  If

both are off, nothing is shown, keeping the  display  from  getting  hung  on

-more- when running unattended.

3.60.  nntp trace <level>

Sets or shows the current trace level for the nntp client.  0  No  tracing  1

(default) displays serious errors.  2  displays as 1 and transient errors.  3

displays as 2 and session progress.  4  displays as  3  and  actual  received

articles.  5  displays errors.

3.61.  nrstat

Displays the netrom interface statistics.

3.62.  param <iface> [<param> ...]

Invoke a device-specific control routine.  On  a  KISS  TNC  interface,  this

sends  control  packets  to the TNC.  Data bytes are treated as decimal.  For

example, param ax0 1 255  will set the keyup timer (type field =  1)  on  the

KISS TNC configured as ax0 to 2.55 seconds (255 x .01 sec).  On a SLIP inter-

face, the param command allows the baud rate to be read  (without  arguments)

or  set.  On  asy  lines DTR and RTS can be (and possibly should be) set with

param iface dtr 1 and param iface rts 1.  The implementation of this  command

for the various interface drivers is incomplete and subject to change.

Current commands defined are:

0  Data       - Normal KISS data

1  TxDelay    - set TX delay for TNC

2  Persist    - set persistence delay for TNC

3  Slottime   - set slottime delay for TNC

4  TxTail     - set TX tail delay for TNC

5  Fulldup    - set Full duplex for TNC

6  Hardware   - Hardware dependent

7  TxMute     -

8  DTR        - 0 = down, 1 = up

9  RTS        - 0 = down, 1 = up

10  Speed      - baud rate

11  Enddelay   -

12  Group      -

13  Idle       -

14  Min        -

15  Maxkey     -

16  Wait       -

17  Parity     - 0 = none, 1 = even, 2 = odd parity

129  Down       -

130  Up         -

254  Return2    - Some tnc's need this

255  Return     - Reset a tnc from KISS into command mode

3.63.  ping <hostid> [<length> [<milliseconds> [<incflag>]]]

Ping (send ICMP Echo Request packets to) the specified host. By  default  the

data  field  contains only a small timestamp to aid in determining round trip

time; if the optional length argument is given,  the  appropriate  number  of

- 32 -

data bytes (consisting of hex 55) are added to the ping packets.

If milliseconds is specified, pings will  be  repeated  indefinitely  at  the

specified  number  of  milliseconds;  otherwise  a single, "one shot" ping is

done.  Responses to one-shot  pings  appear  asynchronously  on  the  command

screen,  while  repeated  pings  create  a  session that may be suspended and

resumed.  Pinging continues until the session is manually reset.

The incflag option causes a repeated ping to increment the target IP  address

for  each  ping;  it  is  an  experimental feature for searching blocks of IP

addresses for active hosts.

3.64.  pop <subcommand>

The pop client provides an automatic interface to  the  pop  server  that  is

totaly  transparen to the user; all that the user needs to do is to set a few

key parameters and the client will take it from there, appart from registring

with  the server station manager for setting up the user data. (see pop user-

data command.  To read more about pop, read rfc937.  The following  paragraph

describe  the pop subcommands.

3.64.1.  pop mailbox [<name>]

Show the defined name or sets the name to name for POP mail  queries  to  the

mailhost.  The  server  keeps  the  mail  in  the  mailbox  name  located  in

/spool/mail/name.txt. This is a mandatory parameter.

3.64.2.  pop mailhost [<ipaddr>]

Show the currently defined host address for a  pop  server  or  sets  the  IP

address  to the system named ipaddr for POP connections.  This is a mandatory

parameter.

3.64.3.  pop kick

Start a POP connection now. (one shot connection)

3.64.4.  pop timer [<seconds>]

Show the current setting and time remaining untill  next  server  enquiry  or

when  seconds is given, defines the interval that a pop connection is started

every <seconds> to retrieve mail from the server system.  When the  timer  is

not set the client is only started with a pop kick command.

3.64.5.  pop userdata <name> <password>

Sets up the userdata used for validation on the POP server system. Note  that

<name>  and  <password> are case sensitive. When only pop userdata is entered

to show the values only the <name> is shown. (ultimate in  security,  if  you

realy  want to see the password look in the "ether").  Remember: the name and

password should also be defined at the server site in the popusers file.

3.65.  popmail <subcommand>

popmail is a newer implementation of the pop  client/server.  It  can  handle

both pop2 and pop3 type pop client/servers. For functionality see pop but the

subcommands are different.

- 33 -

3.65.1.  popmail addserver <host> [<seconds>] [hh:mm-hh:mm] <protocol> <mail-

box> <username> <password>

Add hostP as a pop server. When seconds is given, a timer is started to query

the  host with that interval for mail. If not specified no quering to the pop

host  will be started. You have to do that manualy with a kick.  When  hh:mm-

hh:mm is given then only in that exact timeframe are queries to the host made

(allowed).  Protocol is eighter POP2 or POP3, depending on the  mail  service

the host is providing. Note: pop2 is superceded by pop3. Mailbox is the mail-

box name on the host where fore mail has to be picked up. Username and  pass-

word  are  this  systems  validation  parameters  for the host.  Note that on

entering this command the host name is looked up. If non exsistant  an  error

message is presented.

3.65.2.  popmail dropserver <host>

Drops host from the list of pop servers to be queried.   All   references  to

the entry are deleted from the current system.

3.65.3.  popmail kick <host>

Starts a pop session with host to retrieve mail. This command is needed  when

no interval is specified with the popmail addserver command.

3.65.4.  popmail list

Lists the current popmail server table.

3.65.5.  popmail quiet <yes|no>

Displays or sets the notification of new mail ariving via pop.

3.65.6.  popmail trace <level>

Displays or sets the trace level of pop sessions. Current trace levels are:

0 - No tracing

1 - Serious errors reported

2 - Transient errors reported

3 session progress reported Note that tracing only goes to the log file.

3.66.  ppp <subcommands>

These commands are used for Point to Point Protocol interfaces.

This implementation of PPP  is  designed  to  be  as  complete  as  possible.

Because  of  this,  the number of options can be rather daunting.  However, a

typical PPP configuration might include the following commands:

attach asy 0x3f8 4 ppp pp0 4096 1500 9600

dial pp0 30 <hostid> 3 dialer.pp0

#

ppp pp0 lcp local accm 0

ppp pp0 lcp local compress address on

ppp pp0 lcp local compress protocol on

ppp pp0 lcp local magic on

ppp pp0 lcp open active

#

ppp pp0 ipcp local compress tcp 16 1

ppp pp0 ipcp open active

#

route add default pp0

- 34 -

3.66.1.  ppp <iface>

Display the status of the PPP interface.

3.66.2.  ppp <iface> lcp ...

These commands are used for the LCP [Link Control Protocol] configuration.

3.66.2.1.  ppp <iface> lcp close

Shutdown the PPP interface.

3.66.2.2.  ppp <iface> lcp local ...

These commands control the configuration of the local side of the  link.   If

an  option is specified, the parameters will be used as the initial values in

configuration requests.  If not specified, that option will not be requested.

For each of these options, the allow parameter  will  permit  the  remote  to

include  that option in its response, even when the option is not included in

the request.  By default, all options are allowed.

3.66.2.2.1.  ppp <iface> lcp local accm [ <bitmap> | allow [on | off] ]

Display or set the Async Control Character Map.  The default is 0xffffffff.

3.66.2.2.2.  ppp <iface> lcp local authenticate [ pap | none |  allow  [on  |

off] ]

Display or set the authentication protocol.  The default is none.

3.66.2.2.3.  ppp <iface> lcp local compress address/control  [  on  |  off  |

allow [on | off] ]

Display or set the option to compress the address and control fields  of  the

PPP  HLDC-like  header.   This  is  generally desirable for slow asynchronous

links, and undesirable for fast or synchronous links.  The default is off.

3.66.2.2.4.  ppp <iface> lcp local compress protocol [ on | off | allow [on |

off] ]

Display or set the option to compress the protocol field of the PPP HLDC-like

header.   This  is  generally  desirable  for  slow  asynchronous  links, and

undesirable for fast or synchronous links.  The default is off.

3.66.2.2.5.  ppp <iface> lcp local magic [ on | off | <value> | allow  [on  |

off] ]

Display or set the initial Magic Number.  The default is off (zero).

3.66.2.2.6.  ppp <iface> lcp local mru [ <size> | allow [on | off] ]

Display or set the Maximum Receive Unit.  The default is 1500.

3.66.2.2.7.  ppp <iface> lcp local default

Reset the options to their default values.

- 35 -

3.66.2.3.  ppp <iface> lcp open active | passive

Wait for the physical layer to come up.  If  active,  initiate  configuration

negotiation.  If passive, wait for configuration negotiation from the remote.

3.66.2.4.  ppp <iface> lcp remote ...

These commands control the configuration of the remote side of the link.  The

options are identical to those of the local side.  If an option is specified,

the parameters will be  used  in  responses  to  the  remote's  configuration

requests.  If not specified, that option will be accepted if it is allowed.

For each of these options, the allow parameter  will  permit  the  remote  to

specify that option in its request.  By default, all options are allowed.

3.66.2.5.  ppp <iface> lcp timeout [<seconds>]

Display or set the interval to  wait  between  configuration  or  termination

attempts.  The default is 3 seconds.

3.66.2.6.  ppp <iface> lcp try ...

These commands are used for the various counters.

3.66.2.6.1.  ppp <iface> lcp try configure [<count>]

Display or set the number of configuration requests sent.  The default is 10.

3.66.2.6.2.  ppp <iface> lcp try failure [<count>]

Display or set the number of bad  configuration  requests  allowed  from  the

remote.  The default is 5.

3.66.2.6.3.  ppp <iface> lcp try terminate [<count>]

Display or set the number of termination requests sent before shutdown.   The

default is 2.

3.66.3.  ppp <iface> ipcp ...

These commands are used for the IPCP  [Internet  Protocol  Control  Protocol]

configuration.

The close, open, timeout and  try  sub-commands  are  identical  to  the  LCP

(described above).

3.66.3.1.  ppp <iface> ipcp local ...

These commands control the configuration of the local side of the  link.   If

an  option is specified, the parameters will be used as the initial values in

configuration requests.  If not specified, that option will not be requested.

For each of these options, the allow parameter  will  permit  the  remote  to

include  that option in its response, even when the option is not included in

the request.  By default, all options are allowed.

3.66.3.1.1.  ppp <iface> ipcp local address [ <hostid> | allow [on | off] ]

Display or set the local address for negotiation purposes.  If an address  of

0  is  specified,  the  other  side  of the link will supply the address.  By

default, no addresses are negotiated.

- 36 -

3.66.3.1.2.  ppp <iface> ipcp local compress [ tcp <slots> [<flag>] | none  |

allow [on | off] ]

Display or set the compression protocol.  The default is none.

The tcp <slots> specifies the number of "conversation" slots, which must be 1

to  255.   (This  may be limited at compilation time to a smaller number.)  A

good choice is in the range 4 to 16.

The tcp <flag> is 0 (don't compress the slot number) or 1 (OK to compress the

slot number).  KA9Q can handle compressed slot numbers, so the default is 1.

3.66.3.2.  ppp <iface> ipcp remote ...

These commands control the configuration of the remote side of the link.  The

options are identical to those of the local side.  If an option is specified,

the parameters will be  used  in  responses  to  the  remote's  configuration

requests.  If not specified, that option will be accepted if it is allowed.

For each of these options, the allow parameter  will  permit  the  remote  to

specify that option in its request.  By default, all options are allowed.

3.66.4.  ppp <iface> pap ...

These commands are used for the PAP [Password Authentication Protocol] confi-

guration.

The timeout and try sub-commands are identical to the LCP (described  above).

However, the terminate counter is unused.

3.66.4.1.  ppp <iface> pap user [ <username> [<password>] ]

Display or set the username (the password may be  set,  but  not  displayed).

When  the  username  is  specified, but no password is supplied, the ftpusers

file is searched for the password.  When a username/password  is  unknown  or

rejected,  a  session  will  appear  at  the  console  to  prompt  for  a new

username/password.

3.66.5.  ppp <iface> trace [<flags>]

Display or set the flags that control the logging of information  during  PPP

link configuration.

The flag value is 0 for none, 1 for basic, and 2 for general.  Values greater

than  2 are usually not compiled, and are described in the appropriate source

files where they are defined.

3.67.  ps

Display all current processes in the system. The fields are as follows:

PID - Process ID (the address of the process descriptor).

SP - The current value of the process stack pointer.

stksize - The size of the stack allocated to the process.

maxstk - The apparent peak stack utilization of this process. This is done in

a  somewhat  heuristic  fashion, so the numbers should be treated as approxi-

mate. If this number reaches or exceeds the stksize  figure,  the  system  is

almost certain to crash; the nos.exe program should be recompiled to give the

- 37 -

process a larger allocation when it is started.

event - The event this task is waiting for, if it is not runnable.

fl - Process status flags. There are three: I (Interrupts enabled), W  (Wait-

ing  for event) and S (suspended). The I flag is set whenever a task has exe-

cuted a pwait() call (wait for event) without first disabling hardware inter-

rupts.  Only tasks that wait for hardware interrupt events will turn off this

flag; this is done to avoid critical sections and missed  interrupts.  The  W

flag  indicates  that  the  process is waiting for an event; the event column

will be non-blank. Note that although there may be several runnable processes

at  any  time  (shown  in the ps listing as those without the W flag and with

blank event fields) only one process is actually running at any  one  instant

(The  Refrigerator  Light  Effect  says that the ps command is always the one

running when this display is generated.)

3.68.  pwd [<dirname>]

An alias for the cd command.

3.69.  rarp <subcommand>

This uses the reverse address resolution protocol.

3.69.1.  rarp query <iface> <callsign>

This starts a reverse resolution request via iface to find the IP address for

callsign.  It  counts  down  for 10 seconds before giving up listening for an

answer.

3.70.  record [off | <filename>]

Append to filename all data received on the current session.   Data  sent  on

the  current session is also written into the file except for Telnet sessions

in remote echo mode.  The command record off stops recording and  closes  the

file.

3.71.  remote [-p <port>] [-k <key>] [-a <kickaddr>] <hostid> exit | reset  |

kick

Send a UDP packet to the specified host commanding it to exit  the  Nos  pro-

gram, reset the processor, or force a retransmission on TCP connections.  For

this command to be accepted, the remote system must  be  running  the  remote

server  and  the  port  number specified in the remote command must match the

port number given when the server was started on the remote system.   If  the

port numbers do not match, or if the remote server is not running on the tar-

get system, the command packet is ignored.  Even if the command  is  accepted

there is no acknowledgement.

The kick command forces a retransmission timeout on all TCP connections  that

the remote node may have with the local node.  If the -a option is used, con-

nections to the specified host are kicked instead. No key is required for the

kick subcommand.

The exit and reset subcommands are mainly useful for restarting the nos  pro-

gram  on  a  remote  unattended  system after the configuration file has been

updated.  The remote system should invoke the Nos program automatically  upon

booting,  preferably in an infinite loop.  For example, under MS-DOS the boot

disk should contain the following in autoexec.bat:

- 38 -

:loop

nos

goto :loop

3.71.1.  remote -s <key>

The exit and reset subcommands of remote require a password.  The password is

set on a given system with the -s option, and it is specified in a command to

a remote system with the -k option. If no password is set with the -s option,

then the exit and reset subcommands are disabled.

Note that remote is an experimental feature in NOS; it is not  yet  supported

by any other TCP/IP implementation.

3.72.  rename <oldfilename> <newfilename>

Rename oldfilename to newfilename.

3.73.  reset [<session>]

Reset the specified session; if no argument is given, reset the current  ses-

sion.   This  command  should be used with caution since it does not reliably

inform the remote end that the connection no longer exists.  (In TCP a  reset

(RST) message will be automatically generated should the remote TCP send any-

thing after a local reset has been done.  In AX.25 the DM message performs  a

similar  role.   Both are used to get rid of a lingering half-open connection

after a remote system has crashed.)

3.74.  rip <subcommand>

These commands are used for the RIP service.

3.74.1.  rip accept <gateway>

Remove the specified gateway from  the  RIP  filter  table,  allowing  future

broadcasts from that gateway to be accepted.

3.74.2.  rip add <hostid> <seconds> [<flags>]

Add an entry to the RIP broadcast table. The IP routing table will be sent to

hostid  every  interval  of  seconds. If flags is specified as 1, then "split

horizon" processing will be performed for this destination. That is,  any  IP

routing  table  entries  pointing  to the interface that will be used to send

this update will be removed from the update.  If split horizon processing  is

not  specified,  then all routing table entries except those marked "private"

will be sent in each update.  (Private entries are never sent  in  RIP  pack-

ets).

Triggered updates are always done. That is, any change in the  routing  table

that  causes  a  previously  reachable destination to become unreachable will

trigger an update that advertises the destination with metric 15, defined  to

mean "infinity".

Note that for RIP packets to be sent properly to a broadcast  address,  there

must exist correct IP routing and ARP table entries that will first steer the

broadcast to the correct interface and  then  place  the  correct  link-level

broadcast  address  in  the  link-level  destination  field. If a standard IP

broadcast address convention is used (eg. 128.96.0.0 or 128.96.255.255)  then

- 39 -

chances  are  you  already  have  the  necessary  IP routing table entry, but

unusual subnet or cluster-addressed networks may require  special  attention.

However, an arp add command will be required to translate this address to the

appropriate link level broadcast address. For example,

arp add 128.96.0.0 ethernet ff:ff:ff:ff:ff:ff

for an Ethernet network (ip address is a sample only!!!), and

arp add 44.255.255.255 ax25 qst-0

for an AX25 packet radio channel. (If multiple AX25 interfaces, make a unique

address for each interface.)

3.74.3.  rip drop <dest>

Remove an entry from the RIP broadcast table.

3.74.4.  rip merge [on | off]

This flag  controls  an  experimental  feature  for  consolidating  redundant

entries  in  the  IP routing table. When rip merging is enabled, the table is

scanned after processing each RIP update. An entry is considered redundant if

the  target(s)  it  covers  would  be routed identically by a less "specific"

entry already in the table. That is, the target address(es) specified by  the

entry  in  question must also match the target addresses of the less specific

entry and the two entries must have the same interface  and  gateway  fields.

For example, if the routing table contains

Dest            Len Interface    Gateway          Metric  P Timer  Use

1.2.3.4         32  ethernet0    128.96.1.2       1       0 0      0

1.2.3           24  ethernet0    128.96.1.2       1       0 0      0

then the first entry would be deleted as  redundant  since  packets  sent  to

1.2.3.4  will  still  be  routed correctly by the second entry. Note that the

relative metrics of the entries are ignored.

3.74.5.  rip refuse <gateway>

Refuse to accept RIP updates from the specified gateway by adding the gateway

to the RIP filter table. It may be later removed with the rip accept command.

3.74.6.  rip request <gateway>

Send a RIP Request packet to the specified gateway, causing it to reply  with

a RIP Response packet containing its routing table.

3.74.7.  rip status

Display RIP status, including a count of  the  number  of  packets  sent  and

received,  the  number  of  requests and responses, the number of unknown RIP

packet types, and the number of refused RIP updates from hosts in the  filter

table.  A  list  of the addresses and intervals to which periodic RIP updates

are being sent is also shown, along with the contents of the filter table.

- 40 -

3.74.8.  rip trace [0 | 1 | 2]

This variable controls the tracing of  incoming  and  outgoing  RIP  packets.

Setting  it to 0 disables all RIP tracing. A value of 1 causes changes in the

routing table to be displayed, while packets that cause no changes  cause  no

output.  Setting the variable to 2 produces maximum output, including tracing

of RIP packets that cause no change in the routing table.

3.74.9.  rip ttl <seconds>

Displays or sets the time to live timer to seconds. Normal timeout  value  is

240  seconds.  This  is  not the ttl in a rip broadcast (16 = infinite).  Set

this timer before starting rip. Change this timer only  in  cooperation  with

your surrounding nodes. Default is 240 seconds.

3.75.  rlogin host

Sets up an rlogin session via port 511 to an *NIX compatible station.  Defaut

terminal is an ansi (as defined with the fkeys) compatible terminal.  Default

user name is guest. (Redefine with set USER= environement variable).

3.76.  rmdir <dirname>

Remove a sub-directory from the current working directory.

3.77.  route

With no arguments, route displays the IP routing table.

3.77.1.  route add <dest_hostid>[/bits] | default <iface> [<gateway_hostid> |

direct [<metric>]]

This command adds an entry to the routing table. It  requires  at  least  two

more  arguments,  the  hostid  of  the target destination and the name of the

interface to which its packets should be sent.  If  the  destination  is  not

local,  the gateway's hostid should also be specified. (If the interface is a

point-to-point link, then gateway_hostid may be omitted even if the target is

non-local  because  this  field  is only used to determine the gateway's link

level  address,  if  any.   If  the  destination   is   directly   reachable,

gateway_hostid  is  also unnecessary since the destination address is used to

determine the interface link address).  If rspf is used and the system  is  a

switch  /  router  to  multiple routes the keyword direct can be used i.s.o a

gateway_hostid to set the metric higher as the default  1.  This  way  routes

advertized by other rspf stations can be cheaper and get selected.  If direct

is given but metric not, an new algoritm is used to set the metric  dependent

on the number of subnet mask bits.

The optional /bits suffix to the destination host id specifies how many lead-

ing  bits in the host id are to be considered significant in the routing com-

parisons.  If not specified, 32 bits (i.e., full  significance)  is  assumed.

With  this  option,  a single routing table entry may refer to many hosts all

sharing a common bit string prefix in their IP addresses.  For example,  ARPA

Class A, B and C networks would use suffixes of /8, /16 and /24 respectively;

the command

route add 44/8 sl0 44.64.0.2

causes any IP addresses beginning with "44" in the first 8 bits to be  routed

to 44.64.0.2; the remaining 24 bits are "don't-cares".

- 41 -

When an IP address to be routed matches more than one entry  in  the  routing

table,  the  entry  with  largest  bits parameter (i.e., the "best" match) is

used. This allows individual hosts or blocks of hosts to be exceptions  to  a

more general rule for a larger block of hosts.

The special destination default is used to route datagrams to  addresses  not

matched by any other entries in the routing table; it is equivalent to speci-

fying a /bits suffix of /0 to any destination hostid.   Care  must  be  taken

with  default  entries  since two nodes with default entries pointing at each

other will route packets to unknown addresses back and forth in a loop  until

their   time-to-live  (TTL)  fields  expire.   (Routing  loops  for  specific

addresses can also be created, but this is  less  likely  to  occur  acciden-

tally).

There are two build in interfaces:  loopback  and  encap.   Loopback  is  for

internal  purposes  only.  The encap is an IP encapsulator interface. This is

used to encapsulate a complete IP datagram into a IP datagram so that it gets

"piggy-backed". It is often used to carry ampr.org datagrams (net44) over the

Internet. Note that the Internet is fully connected and that ampr.org is  (at

least)  very  loose.   This  way two sites can interchange net44 datagrams to

each other.  Some extra notes: A Internet gateway has 2 IP addresses: one  on

ampr.org  and  one  on  the Internet. You should make sure that the interface

connected to the Internet has the ifconfig ipaddr set correctly.  Note:  This

previously only worked as gateway for other stations. There was some guessing

done in picking an IP address when encap is used localy.  The guessing  takes

the  worst  guess.  (always wrong with a 2.7 Murphy factor) The code now uses

the local IP address as source when the route starts at the local station. If

this is not what you want, you can overrule this by setting the IP address of

the psuedo encap interface to what you want it to be.

Some extra notes on encap: I have 3  systems  on  an  ethernet  with  network

129.179.122.128/25.  In the office I have another net linked to the Internet.

The addresses there are in the 129.179.122.0/25 range. Now  i  have  a  radio

link  with  44.137.0.2  and 44.137.1.2 on respective sites. On the 44.137.0.2

system i have route add 44.0.0.0/8 encap 44.137.1.1 99. On a next  system  on

my local ethernet i have 44.137.0.1 / 129.179.122.129.  To get from that sys-

tem to say 44.62.0.1 i have to add an encap to my local  gateway  with  route

add   44.0.0.0/8   encap   129.179.122.130.    A   route   add   default  ec0

129.179.122.130 gives me access to the Internet.  Otherwise  it  will  lookup

the  address  for  the interface to be used to reach 129.179.122.130 and will

use 129.179.122.129. Now 44.62.0.1 will NEVER know where  it  came  from.  So

adding the encap on the second system solves the problem.

Here are some examples of the route command:

- 42 -

# Route datagrams to IP address 44.0.0.3 to SLIP line #0.

# No gateway is needed because SLIP is point-to point.

route add 44.0.0.3 sl0

# Route all default traffic to the gateway on the local Ethernet

# with IP address 44.0.0.1

route add default ec0 44.0.0.1

# The local Ethernet has an ARPA Class-C address assignment;

# route all IP addresses beginning with 192.4.8 to it

route add 192.4.8/24 ec0

# The station with IP address 44.0.0.10 is on the local AX.25 channel

route add 44.0.0.10 ax0

# An encapsulation link to 192.4.8.12 where the subnet 44.64.0.0 is

accessible. The internet does not know where we are but we just use them

with what they know:

route add 44.64.0.0/16 encap 192.4.8.12 4

3.77.2.  route addprivate <dest hostid>[/bits] |  default  <iface>  [<gateway

hostid> [<metric>]]

This command is identical to route add except that  it  also  marks  the  new

entry as private; it will never be included in outgoing RIP updates.

3.77.3.  route drop <dest hostid>

route drop deletes an entry from the table.  If  a  packet  arrives  for  the

deleted address and a default route is in effect, it will be used.

3.78.  rspf <subcmd>

RSPF is the Radio Shortest Path First protocol. Each station listens for  RRH

(Router  to  Router Hello) messages. When such a RRH message is received, Nos

will figure out if the link is bi-directional by pinging the  other  station.

The protocol is described in the RSPF 2.1 specification.

3.78.1.  rspf interface <interface> <quality> <horizon>

<interface> is the required interface rspf should use.  quality is from 1  to

127  horizon  is between 1 to 255 End nodes should have the quality set to 1.

Immediate nodes normaly set the quality to 8. The normaly used value for hor-

izon is 32.

3.78.2.  rspf mode [vc | datagram | none]

Without arguments, displays the preferred mode for RSPF.  Modes are VC  (Vir-

tual Cirquit) and Datagram. none resets the preferred mode.

3.78.3.  rspf rrhtimer [seconds]

Without arguments, displays the rrh timer value.

3.78.4.  rspf suspecttimer [seconds]

Without arguments, displays the suspect timer value.

- 43 -

3.78.5.  rspf timer [seconds]

Without arguments, displays the update timer value.

To activate RSPF, do the following.  Set the broadcast address for the desti-

nation interface, example ax0.

ifconfig ax0 broadcast 44.255.255.255

This automatically creates a routing entry for 44.255.255.255 in the  routing

table.  If  intend  to  use RSPF on more than one interface, those interfaces

must each have different broadcast adresses. Else the routing entries will be

overwriten by the next definition.

Configure ax0 as an RSPF interface with horizon 32 and a quality of 1 (hops).

This is typical for an end node. replace the 1 with an 8 for immediate nodes.

rspf interface ax0 1 32

Set the interval between RRH messages.

rspf rrhtimer 900

Define how long it takes until an idle link is suspected to be bad.

rspf suspecttimer 2000

Set the interval between routing updates.

rspf timer 900

3.79.  sccstat

Display the defined PE1CHL scc driver statistics.

3.80.  session [[<session>] [flowmode [on | off]]]

Without arguments, displays the list of current sessions,  including  session

number,  remote TCP or AX.25 address and the address of the TCP or AX.25 con-

trol block.  An asterisk (*) is shown  next  to  the  current  that  session.

Entering  a session number as an argument to the session command will put you

in converse mode with that session.  If the Telnet session; entering a  blank

line at this point puts you in converse mode with server is enabled, the user

is notified of an incoming request and  a  session  number  is  automatically

assigned.  The user may then select the session normally to converse with the

remote user as though the session had been  locally  initiated.   Adding  the

flowmode  displays  or enables / disables setting of *more* handling for that

session. This is handy for example by long directory listings comming from an

ftp  session.  Escaping  to  command  mode before issuing the dir command and

entering "session # flowmode on" gives a page at a time to look  at.  At  any

time  you  can escape out again and switch flowmode off. Note that a ftp ses-

sion has it's own flow command now buildin.  See FTP commands later  in  this

manual.

3.81.  shell [

Suspends Nos and executes a sub-shell  ("command  processor"  under  MS-DOS).

When the sub-shell exits, Nos resumes (under MS-DOS, enter the exit command).

Note: see the COMSPEC environement variable.  When Background  activity  (FTP

servers,  etc)  is also suspended while the subshell executes. Note that this

will fail unless there is sufficient unused  memory  for  the  sub-shell  and

whatever  command  the user tries to run.  When shelled out, Mailbox Operator

connects and ttylink incomming connections are refuced. A  System  unattended

message is send to the "connector" of that socket.

3.82.  skick #socket

This is a shorthand for the various kick subcommands. This one  searches  the

socket for correct type and kicks the transport layer.

- 44 -

3.83.  smtp <subcommand>

These commands are used for the Simple  Message  Transport  Protocol  service

(that is, mail).

3.83.1.  smtp batch [yes | no]

If set smtp will batch the commands into one frame. When off only one command

is  send and a response is waited for. Some old and flaky smtp servers cannot

handle more than one command at a time. Nos can handle multiple. If  you  are

not hindered by an old smpt server, setting batch reduces bandwith.

3.83.2.  smtp gateway [<hostid>]

Displays or sets the host to be used as a "smart" mail relay. Any  mail  sent

to  a host not in the host table will instead be sent to the gateway for for-

warding.

3.83.3.  smtp kick

Run through the outgoing mail queue and attempt to deliver any pending  mail.

This  command  allows the user to "kick" the mail system manually.  Normally,

this command is periodically invoked by a timer whenever Nos is running.

3.83.4.  smtp kill [<jobid>]

Kill the job and delete the message.

3.83.5.  smtp list

List the current jobs. A "L" means locked and in progress.  It is wise to add

in autoexec.bat a "del /spool/mqueue/*.lck" command

3.83.6.  smtp maxclients [<count>]

Displays or sets the maximum number of simultaneous  outgoing  SMTP  sessions

that will be allowed. The default is 10; reduce it if network congestion is a

problem.

3.83.7.  smtp mode [queue | route]

Sets the smtp delivery mode. If queue, all messages are left in /spool/rqueue

for  external forwarding and handling. If route, messages are handled, if for

local, appended to a mailbox or if remote, they are forwarded.

3.83.8.  smtp mxlookup [yes | no]

Displays or sets a flag enabling or disabling MX record lookups. This can  be

enabled if a domain server is available in the near distance (reachable).  It

should be disabled (default) if no domain server is in reach to  satisfy  the

MX  query.  Note that MX record handling is very limited in NOS. If an answer

from a domain name server comes in it is taken to be the destination.

3.83.9.  smtp quiet [yes | no]

Enables or disables the message that new mail arrived at this system.

3.83.10.  smtp timer [<seconds>]

Displays or sets the interval, between scans of the outbound mail queue.  For

example,  smtp  timer  600  will  cause the system to check for outgoing mail

- 45 -

every 10 minutes and attempt to deliver anything it finds, subject of  course

to the smtp maxclients limit. Setting a value of zero disables queue scanning

altogether, note that this is the default!  This  value  is  recommended  for

stand  alone IP gateways that never handle mail, since it saves wear and tear

on the floppy disk drive.

3.83.11.  smtp trace [<value>]

Displays or sets the trace flag in the SMTP client,  allowing  you  to  watch

SMTP's  conversations as it delivers mail.  Zero (the default) disables trac-

ing.

3.84.  socket [[<socket #>] [flowmode [yes | no]]]

Without an argument, displays all active  sockets,  giving  their  index  and

type,  the address of the associated protocol control block and the and owner

process ID and name. If the index to an active socket is supplied, the status

display  for  the appropriate protocol is called.  For example, if the socket

refers to a TCP connection, the display will be that given by the tcp  status

command  with  the  protocol  control block address.  Flow mode can be set or

cleared on a session if so wanted. This comes in handy when a large directory

is  displayed  by ftp. Escaping from the session just before entering dir and

setting flowmode and returning to that session, gives a screen full at a time

with -more- processing.

3.85.  source <filename>

Read subsequent commands from filename until EOF. Then  resume  reading  com-

mands  from  the previous stream. This can be used to keep routing statements

in a separate file, wich can be read at some point in autoexec.nos.

3.86.  start ax25 | convers | discard | echo | finger | ftp | lpd | netrom  |

nntp | pop | pop2 | pop3 | remote | rip | smtp | telnet | tip | ttylink

Start the specified Internet server, allowing remote connection requests.

3.87.  status

Displays load information on Nos. When started, how long running, open  files

etc.

3.88.  stop ax25 | convers | discard | echo | finger | ftp | lpd |  netrom  |

nntp | pop | pop2 | pop3 | remote | rip | smtp | telnet | tip | ttylink

Stop the specified Internet server,  rejecting  any  further  remote  connect

requests. Existing connections are allowed to complete normally.

3.89.  tail <filename>

This does a more on the last couple of lines of file filename.

3.90.  tcp <subcommand>

These commands are used for the Transmission Control Protocol service.

3.90.1.  tcp irtt [<milliseconds>]

Display or set the initial round trip time estimate, in milliseconds,  to  be

used  for  new TCP connections until they can measure and adapt to the actual

value.  The default is 5000 milliseconds (5 seconds).  Increasing  this  when

operating  over  slow  channels will avoid the flurry of retransmissions that

- 46 -

would otherwise occur as the smoothed estimate settles down  at  the  correct

value.  Note  that this command should be given before servers are started in

order for it to have effect on incoming connections.

TCP also keeps a cache of measured  round  trip  times  and  mean  deviations

(MDEV)  for current and recent destinations. Whenever a new TCP connection is

opened, the system first looks in this cache. If the  destination  is  found,

the cached IRTT and MDEV values are used. If not, the default IRTT value men-

tioned above is used, along  with  a  MDEV  of  0.   This  feature  is  fully

automatic,  and  it  can improve performance greatly when a series of connec-

tions are opened and closed to a given destination (eg. a series of FTP  file

transfers or directory listings).

3.90.2.  tcp kick <tcb_addr>

If there is unacknowledged data on the send queue of the specified TCB,  this

command forces an immediate retransmission.

3.90.3.  tcp mss [<size>]

Display or set the TCP Maximum Segment Size in bytes that will be sent on all

outgoing  TCP  connect request (SYN segments).  This tells the remote end the

size of the largest segment (packet) it may send. Changing MSS  affects  only

future connections; existing connections are unaffected.

3.90.4.  tcp reset <tcb_addr>

Deletes the TCP control block at the specified address.

3.90.5.  tcp rtt <tcb_addr> <milliseconds>

Replaces the automatically computed round trip time in the specified TCB with

the  rtt in milliseconds.  This command is useful to speed up recovery from a

series of lost packets since it provides a manual bypass  around  the  normal

backoff retransmission timing mechanisms.

3.90.6.  tcp status [<tcb_addr>]

Without arguments, displays several TCP-level statistics, plus a  summary  of

all  existing  TCP connections, including TCB address, send and receive queue

sizes, local and remote sockets, and connection state. If tcb_addr is  speci-

fied,  a more detailed dump of the specified TCB is generated, including send

and receive sequence numbers and timer information.

3.90.7.  tcp syndata [yes | no]

Display or set the tcp syn + data pigybacking flag. Some tcp  systems  cannot

handle syn + data together.

3.90.8.  tcp timertype [linear | exponential]

Displays the current setting or sets the timer type backoff algoritm.

3.90.9.  tcp trace [yes | no]

Display or set the tcp trace flag on or off.

3.90.10.  tcp window [<size>]

Displays or sets the default receive window size in bytes to be used  by  TCP

when creating new connections. Existing connections are unaffected.

- 47 -

3.91.  telnet <hostid> [<port>]

Creates a Telnet session to the specified host and enters converse mode.   If

<port> is given that number is used. Default port is 23.

3.92.  test

Start an internal test for an overflow problem that might appear in the clock

function of some AT computers.

3.93.  thirh-party [yes | no]

This is a restriction setter  for  some  countries  where  stations  are  not

allowed to handle thirth party mail traffic.

3.94.  ttylink <hostid> [<port>]

Creates a Telnet session to the specified host and enters converse mode.   If

<port>  is  given that number is used. Default port is 87.  This uses a split

screen interface for easy conversation.

3.95.  tip <iface>

Creates a tip session that connects to the specified interface in "dumb  ter-

minal"  mode.   The interface must have already been attached with the attach

command.  Any packet traffic (IP datagrams,  etc)  routed  to  the  interface

while this session exists will be discarded.  To close a tip session, use the

reset command. It will then revert to normal slip, nrs or  kiss  mode  opera-

tion.

This feature is primarily useful for manually establishing SLIP  connections.

At present, only the built-in "com" ports can be used with this command.

3.96.  trace [<iface> [off | <btio> [<tracefile>]]]

Controls packet tracing by the interface drivers. Specific bits enable  trac-

ing  of the various interfaces and the amount of information produced.  Trac-

ing is controlled on a per-interface basis; without arguments, trace gives  a

list  of all defined interfaces and their tracing status.  Output can be lim-

ited to a single interface by specifying it, and the  control  flags  can  be

change  by  specifying  them  as  well.  The flags are given as a hexadecimal

number which is interpreted as follows:

O - Enable tracing of output packets if 1, disable if 0

I - Enable tracing of input packets if 1, disable if 0

T - Controls type of tracing:

0 - Protocol headers are decoded, but data is not displayed

1 - Protocol headers are decoded, and data (but not the

headers themselves) are displayed as ASCII characters,

64 characters/line. Unprintable characters are displayed

as periods.

2 - Protocol headers are decoded, and the entire packet

(headers AND data) is also displayed in hexadecimal

and ASCII, 16 characters per line.

B - Broadcast filter flag. If set, only packets specifically addressed

to this node will be traced; broadcast packets will not be displayed.

If tracefile is not specified, tracing will be to the console.

- 48 -

3.97.  udp status

3.98.  verbose [0 | 1 | 2 | 3]

Set or display the level of message output  in  file  transfers.   Verbose  0

gives the least output, and verbose 3 the most, as follows:

0 - Display error messages only.

1 - Display error messages plus a one-line summary after each transfer

giving the name of the file, its size, and the transfer time and rate.

2 - Display error and summary messages plus the progress messages generated

by the remote FTP server. (This setting is the default.)

3 - Display all messages. In addition, a "hash mark" (#) is displayed for

every 1,000 bytes sent or received.

If a command is sent to the  remote  server  because  it  is  not  recognized

locally,  the response is always displayed, regardless of the setting of ver-

bose.  This is necessary for commands like pwd (display  working  directory),

which  would  otherwise produce no message at all if verbose were set to 0 or

1.

Displays the status of all UDP receive queues.

3.99.  upload [<filename>]

Opens filename and sends it on the current session as though it were typed on

the terminal.

3.100.  watch

Displays the current software stopwatch values, with min and max readings for

each.  This  facility  allows  a  programmer to measure the execution time of

critical sections of code with microsecond resolution.  This command is  sup-

ported only on the IBM PC, and the meaning of each stopwatch value depends on

where the calls have been inserted for test purposes; the  distribution  copy

of nos usually has no stopwatch calls.

3.101.  watchdog [on | off]

Enables or disables the watchdog timer. If internal operations cease for  300

second  and  watchdog is enabled, a system reset is performed. See the remote

command for setting up autoexec.bat.

3.102.  ?

Same as the help command.

- 49 -

4.  Attach Commands

This chapter details the attach commands for the various  hardware  interface

drivers.  Not all of these drivers may be configured into every Nos binary; a

list of the available types may be obtained by entering the command attach ?.

Some parameters are accepted by several drivers. They are:

4.0.1.  <bufsize>

For asynchronous devices (eg. COM ports operating in SLIP or NRS  mode)  this

parameter  specifies  the  size  of the receiver's ring buffer.  It should be

large enough to hold incoming data at full line speed for  the  longest  time

that  the system may be busy in MS-DOS or the BIOS doing a slow I/O operation

(eg. to a floppy disk). A kilobyte is usually more than sufficient.

For synchronous devices (eg. the scc, hs, pc100,  hapn  and  drsi  interfaces

operating  in  HDLC mode), the bufsize parameter specifies the largest packet

that may be received on the interface.  This should be set by  mutual  agree-

ment  among  stations  sharing the channel. For standard AX.25 with a maximum

I-frame data size of 256 bytes, a value of 325  should  provide  an  adequate

safety  margin.  On  higher speed channels (eg. 56kb/s) larger values (eg. 2K

bytes) will provide much better performance  and  allow  full-sized  Ethernet

packets to be carried without fragmentation.

4.0.2.  <ioaddr>

The base address of the interface's control registers.This might be specified

in hex as 0xnnn or decimal. (nnn is the hexadecimal number).

4.0.3.  <vector>

The interface's hardware interrupt (IRQ) vector, in decimal. When a vector is

followed  by the character 'c' then the vector is added in a interrupt chain.

This way multiple devices can  share  the  same  interrupt  vector  (hardware

changes might be nessesary). A sample is the use of a 4 port comm board shar-

ing the same vector. The first attach command has a plain vector and the fol-

lowing  have  the  c  appended.  Note  that  the highest speed port should be

defined last (as it is served first by the chaining).  Do NOT specify  the  c

with  the  first attach statement in that group as unpredictable results will

occure.

attach asy 0x3f8 4 ax25 ax25 2048 256 1200

attach asy 0x3f0 4c ax25 ax25 2048 256 9600

4.0.4.  <iface>

The name (an arbitrary character string) to be assigned to this interface. It

is used to refer to the interface in ifconfig and route commands and in trace

output.

4.0.5.  <mtu>

The Maximum Transmission Unit size, in bytes.   Datagrams  larger  than  this

limit  will  be  fragmented at the IP layer into smaller pieces. For AX.25 UI

frames, this limits the size of the information field.  For AX.25  I  frames,

however,  the  ax25  paclen  parameter  is also relevant.  If the datagram or

fragment is still larger than paclen, it is  also  fragmented  at  the  AX.25

level (as opposed to the IP level) before transmission.  (See the ax25 paclen

command for further information).

- 50 -

4.0.6.  <speed>

The speed in bits per second (eg. 2400).

4.1.  attach 3c500 <ioaddr> <vector> arpa <iface> <qlen> <mtu> [<ip_addr>]

Attach a 3Com 3C501  Ethernet  interface.   qlen  is  the  maximum  allowable

transmit  queue  length.   If  the  ip_addr parameter is not given, the value

associated with a prior ip address command will be used.

The use of this driver is not recommended; use the  packet  driver  interface

with the loadable 3C501 packet driver instead.

4.2.  attach asy <ioaddr> <vector> ax25 | nrs | ppp  |  slip  |  raw  <iface>

<bufsize> <mtu> <speed> [<vf>]

Attach a standard  PC  "com  port"  (asynchronous  serial  port),  using  the

National  8250  or 16550A chip.  Standard values on the IBM PC and clones for

ioaddr and vector are 0x3f8 and 4 for COM1, and 0x2f8 and 3 for COM2.  If the

port  uses  a  16550A  chip,  it will be detected automatically and the FIFOs

enabled.

4.2.1.  ax25

Similar to slip, except that an AX.25 header and a KISS  TNC  control  header

are added to the front of the datagram before SLIP encoding.  Either UI (con-

nectionless) or I (connection-oriented) AX.25 frames can  be  used;  see  the

mode command for details.

4.2.2.  nrs

Use the NET/ROM asynchronous framing technique for communication with a local

NET/ROM TNC.

4.2.3.  ppp

Point-to-Point-Protocol.  Encapsulates datagrams in an HDLC-like frame.  This

is  a new Internet standard for point-to-point communication, compatible with

CCITT standards.

4.2.4.  slip

Serial Line Internet Protocol.  Encapsulates IP datagrams  directly  in  SLIP

frames  without  a link header. This is for operation on point-to-point lines

and is compatible with 4.2BSD UNIX SLIP.

4.2.5.  raw

Raw serial line without protocol , special for lpd server.

4.2.6.  <vf>

The optional flags are a string of the characters "vf":  v enables Van Jacob-

son TCP/IP Header Compression, and is valid only for SLIP.  f forces the fifo

on for 16550AFN compatible chips not hindered by the  original  "design  bug"

and not needing the work-around. Unfortunate these good chips don't get their

fifo enabled. Specifying f on the attach line forces the  fifo  to  be  used.

(unpredictable results occure when specified on a non 16550 type chip).

- 51 -

4.3.  attach axip <iface> <mtu> <ip_addr> <callsign>

This creates a RFC1226 compatible AX.25 frame encapsulator  for  transmission

of  AX.25  frames over the Internet. Iface will be the name of the interface,

ip_addr the address of the remote system and callsign the AX.25 callsign this

station is listening on for frames to digipeat.  Note that each attached axip

interface should have a different callsign to listen to and this should  also

be different from other callsign's used on this station.

4.4.  attach drsi <ioaddr> <vector> ax25 <iface> <bufsize> <mtu> <ch_a_speed>

<ch_b_speed>

N6TTO driver for the Digital Radio Systems PCPA 8530 card.  Since  there  are

two  channels  on the board, two interfaces are attached.  They will be named

iface with 'a' and 'b' appended.  bufsize is  the  receiver  buffer  size  in

bytes;  it  must be larger than the largest frame to be received.  ch_a_speed

and ch_b_speed are the speeds, in bits/sec, for the A and B channels, respec-

tively.

4.5.  attach eagle <ioaddr> <vector> ax25 <iface> <bufsize> <mtu> <speed>

WA3CVG/NG6Q driver for the Eagle Computer card (Zilog 8530).

4.6.  attach hapn <ioaddr> <vector> ax25 <iface> <bufsize> <mtu> csma | full

KE3Z driver for the Hamilton Amateur Packet  Network  adapter  (Intel  8273).

The  csma  | full parameter specifies whether the port should operate in car-

rier sense multiple access (CSMA) mode or in full duplex.

4.7.  attach hs <ioaddr> <vector> ax25 <iface> <bufsize> <mtu>  <keyup_delay>

<p>

Attach a DRSI PCPA or Eagle Computer interface card  using  a  special  "high

speed"  8530  driver.   This  driver uses busy-wait loops to send and receive

each byte instead of interrupts, making it  usable  with  high  speed  modems

(such  as  the WA4DSY 56kb/s modem) on slow systems.  This does have the side

effect of "freezing" the system whenever the modem transmitter or receiver is

active.   This  driver  can  operate only in CSMA mode, and it is recommended

that no other interfaces requiring small interrupt latencies be  attached  to

the same machine.

The keyup_delay parameter specifies the transmitter keyup delay in byte  time

intervals.  The  p  value  specifies the transmitter persistence value in the

range 1-255; the corresponding slot time is fixed at one hardware clock tick,

about 55 ms on the PC.

As with the other 8530 drivers, this driver actually attaches two interfaces,

one for each 8530 channel.

4.8.  attach packet <intvec> <iface> <txqlen> <mtu>

Driver for use with  separate  software  "packet  drivers"  meeting  the  FTP

Software,  Inc,  Software  Packet Driver specification.  The driver must have

already been installed before the attach command is given. Packet drivers  in

the Ethernet, ARCNET, SLIP, SLFP, and KISS/AX25 classes are supported.

intvec is the software interrupt vector used for communication to the  packet

driver,  and  txqlen is the maximum number of packets that will be allowed on

the transmit queue.

- 52 -

4.9.  attach pc100 <ioaddr> <vector> ax25 <iface> <bufsize> <speed>

Driver for the PACCOMM PC-100 (Zilog 8530) card.   Only  AX.25  operation  is

supported.

4.10.  attach scc <devices> init <addr>  <spacing>  <Aoff>  <Boff>  <Dataoff>

<intack> <vec> [p | r]<clock> [<hdwe>] [<param>]

PE1CHL driver to initialize a generic SCC (8530)  interface  board  prior  to

actually attaching it. The parameters are as follows:

4.10.1.  <devices>

The number of SCC chips to support.

4.10.2.  <addr>

The base address of the first SCC chip (hex).

4.10.3.  <spacing>

The spacing between the SCC chip base addresses.

4.10.4.  <Aoff>

The offset from a chip's base address to its channel A control register.

4.10.5.  <Boff>

The offset from a chip's base address to its channel B control register.

4.10.6.  <Dataoff>

The offset from each channel's control register to its data register.

4.10.7.  <intack>

The address of the INTACK/Read Vector port. If none, specify 0 to  read  from

RR3A/RR2B.

4.10.8.  <vec>

The CPU interrupt vector for all connected SCCs.

4.10.9.  <clock>

The clock frequency (PCLK/RTxC) of all SCCs in hertz.  Prefix  with  'p'  for

PCLK, 'r' for RTxC clock (for baudrate gen).

4.10.10.  <hdwe>

Optional hardware type. The following values are currently  supported:   1  -

Eagle  card,  2  -  PACCOMM PC-100, 4 - PRIMUS-PC card (DG9BL), 8 - DRSI PCPA

card.

4.10.11.  <param>

Optional extra parameter. At present, this is used only with the  PC-100  and

PRIMUS-PC cards to set the modem mode. The value 0x22 is used with the PC-100

and 0x2 is used with the PRIMUS-PC card.

- 53 -

The attach scc ... init command must be given before the interfaces are actu-

ally attached with the following command.

4.11.  attach scc <chan> slip | kiss | nrs | ax25 <iface> <mtu> <speed> <buf-

size> [<call>]

Attach an initialized SCC port to the system. The parameters are as follows:

4.11.1.  <chan>

The SCC channel number to attach, 0 or 1 for the first chip's A or B port,  2

or 3 for the second chip's A or B port, etc.

4.11.2.  slip | kiss | nrs | ax25

The operating mode of the interface. slip, kiss and nrs all operate the  port

hardware in asynchronous mode; slip is Internet-standard serial line IP mode,

kiss generates SLIP frames containing KISS TNC commands and AX.25 packets and

nrs uses NET/ROM local serial link framing conventions to carry NET/ROM pack-

ets. Selecting ax25 mode puts the interface into synchronous HDLC  mode  that

is suitable for direct connection to a half duplex radio modem.

4.11.3.  <speed>

The interface speed in bits per second (eg. 1200).  Prefix with 'd'  when  an

external divider is available to generate the TX clock. When the clock source

is PCLK, this can be a /32 divider between TRxC and RTxC. When the  clock  is

at  RTxC,  the TX rate must be supplied at TRxC. This is needed only for full

duplex synchronous operation. When this arg is given as 'ext',  the  transmit

and  receive  clocks are external, and the internal baud rate generator (BRG)

and digital phase locked loop (DPLL) are not used.

- 54 -

4.12.  Attach Examples

Here are some examples of the attach command:

# Attach a 3Com Ethernet controller using the standard 3Com address and

# vector (i.e., as it comes out of the box) to use ARPA-standard encapsulation.

# The receive queue is limited to 5 packets, and outgoing packets larger

# than 1500 bytes will be fragmented

attach 3c500 0x300 3 arpa ec0 5 1500

# Attach the PC asynch card normally known as "com1" (the first controller)

# to operate in point-to-point slip mode at 9600 baud, calling it "sl0".

# A 1024 byte receiver ring buffer is allocated. Outgoing packets larger

# than 256 bytes are fragmented.

attach asy 0x3f8 4 slip sl0 1024 256 9600

# Attach the secondary PC asynch card ("com2") to operate in AX.25 mode

# with an MTU of 576 bytes at 9600 baud with a KISS TNC, calling it "ax0".

# By default, IP datagrams are sent in UI frames

attach asy 0x2f8 3 ax25 ax0 1024 576 9600

# Attach a axip wormhole

attach axip ai0 256 129.179.122.10 pa0gri-11

# on the other side of the wormhole

attach axip ai0 256 129.179.122.130 pa0gri-12

# Now assume 129.179.122.10 has a AX.25 interface with callsign pa0gri-10

# and 129.179.122.130 a interface with callsign pa0gri-8

# Now a AX.25 frame like:

# pe1chl->pa0gri-11->pa0gri-8->pe1dna [ data]

# Received by pa0gri-11, set the has-been-digipeated change the interface

# callsign with the one it realy came in from and ecapsulates it

# in a IP frame type 93 and ships it to 129.179.122.130

# pe1chl->pa0gri-10*->pa0gri-8->pe1dna [ data]

# Arived at 129.179.122.130 the next digi is searched for and found.

# The frame is changed into:

# pe1chl->pa0gri-10*->pa0gri-12*->pe1dna [ data]

# so that on the way back the frame will find the "right" interface.

# Attach the packet driver loaded at interrupt 0x7e

# The packet driver is for an Ethernet interface

attach packet 0x7e ethernet 8 1500

- 55 -

5.  FTP Subcommands

During converse mode with an FTP server, everything typed on the  console  is

first  examined  to see if it is a locally-known command. If not, the line is

passed intact to the remote server on the control channel. If it  is  one  of

the following commands, however, it is executed locally. (Note that this gen-

erally involves other commands being sent to the remote server on the control

channel.)

5.1.  dir [<file> | <directory> [<local file>]]

Without arguments, dir requests that a full directory listing of  the  remote

server's  current  directory  be  sent  to  the terminal.  If one argument is

given, this is passed along in the LIST command; this can be a specific  file

or  subdirectory  that  is meaningful to the remote file system. If two argu-

ments are given, the second is taken as the local file into which the  direc-

tory  listing should be put (instead of being sent to the console).  The PORT

command is used before the LIST command is sent.

5.2.  flow [off|on]

Displays or sets the -more- processing on the current ftp session.  When  set

to  on,  a -more- prompt is displayed after each screen full of data. You can

do the same with a session # flow command.  Note that this is a local  exten-

sion to the standard ftp command set.

5.3.  get <remote file> [<local file>]

Asks the remote server to send the file specified in the first argument.  The

second argument, if given, will be the name of the file on the local machine;

otherwise it will have the same name as on the remote machine.  The PORT  and

RETR commands are sent on the control channel.

5.4.  hash

A synonym for the verbose 3 command.

5.5.  ls [<file> | <directory> [<local file>]]

ls is identical to the dir command except that the "NLST" command is sent  to

the  server  instead  of  the  "LIST" command. This results in an abbreviated

directory listing, i.e., one showing only the file names  themselves  without

any other information.

5.6.  mget <file> [<file> ...]

Fetch a collection of files from the server. File names may include wild card

characters; they will be interpreted and expanded into a list of files by the

remote system using the NLST command. The files will have the  same  name  on

the local system that they had on the server.

5.7.  mkdir <remote directory>

Creates a directory on the remote machine.

5.8.  mput <file> [<file> ...]

Send a collection of files to the server. File names may  include  wild  card

characters;  they  will  be expanded locally into a list of files to be sent.

The files will have the same name on the server as on the local system.

- 56 -

5.9.  put <local file> [<remote file>]

Asks the remote server to accept data, creating the file named in  the  first

argument.  The second argument, if given, will be the name of the file on the

remote machine; otherwise it will have the same name as on the local machine.

The PORT and STOR commands are sent on the control channel.

5.10.  rmdir <remote directory>

Deletes a directory on the remote machine.

5.11.  type [a | i | l <bytesize>]

Tells both the local client and remote server the type of file that is to  be

transferred.   The  default,  (wich can be changed with the ftype command) is

'a', which means ASCII (i.e., a text file).   Type  'i'  means  image,  i.e.,

binary.   In  ASCII  mode,  files are sent as varying length lines of text in

ASCII separated by cr/lf sequences; in IMAGE mode, files are sent exactly  as

they appear in the file system.  ASCII mode should be used whenever transfer-

ring text between dissimilar systems (e.g., UNIX and MS-DOS) because of their

different  end-of-line  and/or end-of-file conventions.  When exchanging text

files between machines of the same type, either mode will work but IMAGE mode

is usually faster.  Naturally, when exchanging raw binary files (executables,

compressed archives, etc) IMAGE mode must be used.  Type  'l'  (logical  byte

size) is used when exchanging binary files with remote servers having oddball

word sizes (e.g., DECSYSTEM-10s and  20s).  Locally  it  works  exactly  like

IMAGE,  except that it notifies the remote system how large the byte size is.

bytesize is typically 8.  The type command sets the local transfer  mode  and

generates the TYPE command on the control channel.

5.12.  verbose [0 | 1 | 2 | 3]

Set or display the level of message output  in  file  transfers.   Verbose  0

gives the least output, and verbose 3 the most, as follows:

0 - Display error messages only.

1 - Display error messages plus a one-line summary after each transfer

giving the name of the file, its size, and the transfer time and rate.

2 - Display error and summary messages plus the progress messages generated

by the remote FTP server. (This setting is the default.)

3 - Display all messages. In addition, a "hash mark" (#) is displayed for

every 1,000 bytes sent or received.

If a command is sent to the  remote  server  because  it  is  not  recognized

locally,  the response is always displayed, regardless of the setting of ver-

bose.  This is necessary for commands like pwd (display  working  directory),

which  would  otherwise produce no message at all if verbose were set to 0 or

1.

- 57 -

6.  Dialer Subcommands

Each dialer command may (should) have a  different  dialer  file.   The  file

resides in the configuration directory, as specified in the Installation sec-

tion (see chapter 1).  A typical dialer file might be:

# Set the speed, and toggle DTR to ensure modem is in command mode.

control down

wait 3000

speed 2400

control up

wait 3000

# Dial, and wait for connection

send "atdt555-1212#7"

wait 45000 "CONNECT " speed

wait 2000

# PAD specific initialization

send "#7"

wait 15000 "Terminal ="

send "ppp#7"

wait 10000 "#70

6.0.1.  control down | up

Control asy interface.  The down option drops DTR and  RTS.   The  up  option

asserts DTR and RTS.

6.0.2.  send "string"

This dialer command will write the specified string to  the  interface.   The

string quote marks are required, and the string may not contain embedded con-

trol characters.  However, the standard C string escape sequences are  recog-

nized (\0 should not be used).

6.0.3.  speed [ 9600 | 4800 | 2400 | 1200 | 300 ]

This dialer command will set the speed of the interface to one of the  avail-

able  speeds.   If  the  speed is missing, the speed will be displayed in the

dialer session window.

6.0.4.  wait <milliseconds> [ "test string" ] [ speed ]

If only the time is specified, the dialer pauses for the  desired  number  of

milliseconds.

Otherwise, the dialer reads until the test string is detected on  the  inter-

face.   If the string is not detected within the desired time, the autodialer

will reset.  The string quote marks are required, and the string may not con-

tain  embedded  control  characters.   However,  the standard C string escape

sequences are recognized (\0 should not be used).

Finally, if the speed parameter is specified, the  dialer  will  continue  to

read  characters until a non-digit is detected.  The string read is converted

to an integer, and used to set the interface speed.   If  the  trailing  non-

digit  is not detected within the desired time, or the integer value is not a

valid speed, the autodialer will reset.

- 58 -

7.  Installation

Nos uses the following file and directory structure:

~/alias

~/autoexec.nos

~/dialer

~/domain.txt

~/ftpusers

~/net.rc

~/netron.sav

~/popusers

~/finger/

~/etc/printcap

~/etc/lpdperms

~/etc/log

~/spool/areas

~/spool/mail.log

~/spool/net.log

~/spool/forward.bbs

~/spool/history

~/spool/rewrite

~/spool/help/

~/spool/mail/

~/spool/mqueue/

~/spool/news/

~/spool/news/active

~/spool/news/pointer

~/spool/news/info

~/spool/news/help

~/spool/news/history

~/spool/news/forward

~/spool/news/poll

~/spool/rqueue/

~/spool/signatur/

~/spool/lpd/

The ~ in front of all files is a  directory  offset  definable  with  the  -d

option  on the Nos command line. Any name may be chosen and is default empty.

(eg. just /) If for example  -d  /net  is  given,  the  structure  shifts  to

/net/...   The  alias,  autoexec.nos, dialer, domain.txt net.rc, popusers and

ftpusers configuration files are located here.  The netrom.sav file  will  be

created there.

The "/spool" directory and its sub-directories are used by the bbs, SMTP  and

NNTP  services.  The areas, forward.bbs, history, mail.log and rewrite confi-

guration files are located here.  The /spool/news  directory  can  have  many

subdirectories  and each subdirectory can have subdirectories. Newsgroups are

split into heirarchical directory structures. A  news  article  in  newsgroup

rec.amateur.radio.packet           will           end          up          in

/spool/news/rec/amateur/radio/packet.txt.

7.1.  The /ftpusers File

Since MS-DOS is a single-user operating system (some might say it is a glori-

fied bootstrap loader), it provides no access control; all files can be read,

written or deleted by the local user.  It is usually undesirable to give such

open  access to a system to remote network users.  Net therefore provides its

own access control mechanisms.

- 59 -

The file ftpusers controls remote FTP and mailbox access.  The FTP default is

no  access;  if this file does not exist, the FTP server will be unusable.  A

remote user must first "log in" to the system with the  USER  and  PASS  com-

mands,  giving a valid name and password listed in ftpusers, before he or she

can transfer files.

Each entry in ftpusers consists of a single line of the form

username password /path permissions ip_address

There must be at least four fields, and  there  must  be  exactly  one  space

between  each  field.   Comments  may be added after the last field.  Comment

lines begin with '#' in column one.

username is the user's login name.

password is the required password.  Note that this is in plain  text;  there-

fore it is not a good idea to give general read permission to the root direc-

tory.  A password of '*' (a single  asterisk)  means  that  any  password  is

acceptable.

/path is the allowable prefix on accessible files.  Before any file or direc-

tory  operation,  the current directory and the user- specified file name are

joined to form an absolute path name in "canonical" form (i.e., a  full  path

name  starting at the root, with "./" and "../" references, as well as redun-

dant /'s, recognized and removed).  The result MUST begin with the  allowable

path  prefix;  if not, the operation is denied.  This field must always begin

with a "/", i.e., at the root directory.  Multiple directories can be  speci-

fied by separating them with a ";" caracter and no whitespace around them.

permissions is a decimal number granting  permission  for  read,  create  and

write  operations.  If the low order bit (0x1) is set, the user is allowed to

read a file subject to the path name prefix restriction.   If  the  next  bit

(0x2)  is  set,  the  user  is  allowed  to  create a new file if it does not

overwrite an existing file.  If the third bit  (0x4)  is  set,  the  user  is

allowed  to write a file even if it overwrites an existing file, and in addi-

tion he may delete files.  Again, all operations are allowed subject  to  the

path  name  prefix restrictions.  Permissions may be combined by adding bits,

for example, 0x3 (= 0x2 + 0x1) means that the user is given read  and  create

permission, but not overwrite/delete permission.

Additional permission bits used by the mailbox and PPP are:

1   Read files

2   Create new files

4   Overwrite and delete existing files

8   AX.25 gateway operation allowed

16  Telnet gateway operation allowed

32  NET/ROM gateway operation allowed

64  Remote sysop access allowed (DANGEROUS)

128 This user is banned from BBS access (illegal user)

256 Priv bit for PPP connection

512 Priv bit for peerID/pass lookup

1024 disallow send commands (except to SYSOP)

2048 disallow read commands

4096 disallow 3rd party mail

8192 This station is a known BBS

ip_address is used for PPP only and is the remote IP address of the connected

- 60 -

system.

A username of univperm has special meaning in the  validation  meganism.   If

univperm  is  included as a valid user in ftpusers then any unknown user (not

in ftpusers) will be mapped into univperm and get  its  permission  bits  and

file path. If univperm is not included in ftpusers unknown users are not per-

mitted nor validated.

For example, suppose ftpusers on machine pc.ka9q.ampr.org contains the line

friendly test /testdir 7

A session using this account would look like this:

net> ftp pc.ka9q.ampr.org

Resolving pc.ka9q.ampr.org... Trying 128.96.160.1...

FTP session 1 connected to pc.ka9q.ampr.org

220 pc.ka9q.ampr.org FTP version 900418 ready at Mon May 7 16:27:18 1990

Enter user name: friendly

331 Enter PASS command

Password: test [not echoed]

230 Logged in

ftp>

The user now has read, write, overwrite and delete privileges  for  any  file

under /testdir; he may not access any other files.

Here are some more sample entries in ftpusers:

karn foobar / 7         # User "karn" with password "foobar" may read,

# write, overwrite and delete any file on the

# system.

guest bletch /g/bogus;/public 3

# User "guest" with password "bletch" may read

# any file under /g/bogus and its subdirectories,

# and /public and its subdirectories,

# and may create a new file as long as it does

# not overwrite an existing file. He may NOT

# delete any files.

anonymous * /public 1   # User "anonymous" (any password) may read files

# under /public and its subdirectories; he may

# not create, overwrite or delete any files.

This last entry is the standard convention for keeping a repository of public

files; in particular, the username "anonymous" is an established ARPA conven-

tion.

7.2.  The /popusers File

Here are the username / password combinations defined for the POP users.   It

has a simple convention:

user:password:

for every POP user such a line has to be added. The user and password  fields

sould  match  the  pop  userdata statement of the remote user.  Both user and

- 61 -

password have to be delimited with a colon character.

7.3.  The /net.rc File

The net.rc file is a fast login file for known ftp stations. Each line starts

with the name of the ftp station. Following are a user and password statemend

to be send to the server for validation.  The name,  user  and  password  are

separated  with a space. Not a tab or more than 1 space character.  Following

is a sample net.rc file.

ucsd.edu anonymous gvdg@gvdgpc.cdh.cdc.com

ka9q.ampr.org guest pa0gri

7.4.  The /domain.txt File

Nos translates domain names (eg. "pc.ka9q.ampr.org")  to  IP  addresses  (eg.

128.96.160.3) through the use of an Internet Domain Name resolver and a local

"cache" file, domain.txt.  Whenever the user specifies  a  domain  name,  the

local cache is searched for the desired entry.  If it is present, it is used;

if not, and if domain name server(s) have been configured, a  query  is  sent

over  the  network to the current server.  If the server responds, the answer

is added to the domain.txt file for future  use.   If  the  server  does  not

respond,  any  additional  servers  on  the  list  are tried in a round-robin

fashion until one responds, or the retry limit is  reached  (see  the  domain

retry  command).   If domain.txt does not contain the desired entry and there

are no configured domain name servers, then the request immediately fails.

If a domain name server is available, and if all references  to  host-ids  in

your  /autoexec.nos  file  are  in  IP address format, then it is possible to

start with a completely empty domain.txt file and have Nos build it for  you.

However,  you  may wish to add your own entries to domain.txt, either because

you prefer to use symbolic domain names in your  /autoexec.nos  file  or  you

don't  have  access to a domain server and you need to create entries for all

of the hosts you may wish to access.

Each entry takes one line, and the fields are separated by tabs.   For  exam-

ple:

pc.ka9q.ampr.org.       IN      A       128.96.160.3

IN is the class of the record.  It means Internet, and it will  be  found  in

all  entries.   A  is  the  type  of the record, and it means that this is an

address record.  Domain name pc.ka9q.ampr.org therefore has Internet  address

128.96.160.3.

Another possible entry is the CNAME (Canonical Name) record.  For example:

ka9q.ampr.org.          IN      CNAME   pc.ka9q.ampr.org.

This says that domain name "ka9q.ampr.org" is actually an alias for the  sys-

tem  with  (primary,  or  canonical)  domain name "pc.ka9q.ampr.org."  When a

domain name having a CNAME record is given to Nos, the  system  automatically

follows  the reference to the canonical name and returns the IP address asso-

ciated with that entry.

Entries added automatically by Nos will have an additional field between  the

domain name and the class (IN) field.  For example:

pc.ka9q.ampr.org.       3600    IN      A       128.96.160.3

- 62 -

This is the time-to-live  value,  in  seconds,  associated  with  the  record

received  from  the  server.  Clients (such as Nos) caching these records are

supposed to delete them after the time-to-live interval has expired, allowing

for  the  possibility  that  the  information in the record may become out of

date.

This implementation of Nos will decrement the  TTL  to  zero,  but  will  not

delete  the  record unless the "clean" flag is on (see the domain cache clean

command).  When a remote server is not available, the old entry will be used.

When the TTL value is missing (as in the examples  above),  the  record  will

never  expire, and must be managed by hand.  Since domain.txt is a plain text

file, it may be easily edited by the user to add, change or delete records.

Additional types of records, include NS  (name  server)  and  SOA  (start  of

authority)  may  appear in domain.txt from remote server responses. These are

not currently used by Nos but are retained for future  development  (such  as

the incorporation of a domain name server into Nos itself).

7.5.  The /alias file.

SMTP server ALIAS file. This is for resolving a given target address  into  a

single or multiple entry mail list.

Format:

mail_list_name call_1@host_1 [call_2@host_2]......# comments

pa0gri gvdg@fridley.cdh.cdc.com

kelvin g1emm@g1emm.ampr.org

#

bob    gb3xp@gb3xp.ampr.org

ian    g3rra@g3rra.ampr.org

jim    g1wkk@g1wkk.ampr.org

john   g5ds%gb3kp@gb3xp.ampr.org

ted    gb3kp%gb3kp@gb3xp.ampr.org

ron    g6vug@g6vug.ampr.org

tim    g4uqe@g4uqe.ampr.org

gareth g6kvk%gb7spv@gb3xp.ampr.org

bolton gb7tcp%gb7crg@gb3xp.ampr.org

julian g7efe%gb7cfb@gb3xp.ampr.org

#

world  g3rra@g3rra.ampr.org gb3xp@gb3xp.ampr.org g1plt@g1plt.ampr.org

g1wkk@g1wkk.ampr.org g8kwi@g8kwi.ampr.org g6kvk%gb7spv@gb3xp.ampr.org

g1ttg@g1ttg.ampr.org g6vug@g6vug.ampr.org g6kqz@g6kqz.ampr.org

g4uqe@g4uqe.ampr.org g8ogr@g8ogr.ampr.org g4xwv@g4xwv.ampr.org

#

locals g3rra@g3rra.ampr.org g1wkk@g1wkk.ampr.org g8kwi@g8kwi.ampr.org

g4bio@g4bio.ampr.org g1plt@g1plt.ampr.org g4uqe@g4uqe.ampr.org

g6kqz@g6kqz.ampr.org g4tnu@g4tnu.ampr.org g6xqb@g6xqb.ampr.org

Note that it is reaonable, and sometimes desiable, to have alias  records  in

the form:

area  area dest1 dest2

As the /alias file is scanned only once, this does not result in an  infinite

recursion.

7.6.  The /spool/areas file.

This file is a header file shown to a mailbox user when  he  requests  the  a

display. It should show all public mailboxes to read. Here is a sample:

- 63 -

-------------  Public  --  Mail  --  Areas  -------------

|

General    --    Any old chit-chat that is clean.       |

TcpIp      --    General Tcp/Ip messages. NOS etc.      |

Bugs       --    Where to report bugs in Ka9q-NOS       |

Updates    --    Info - NOS version UPDATES by G1EMM.   |

|

---------------------------------------------------------

7.7.  The /spool/forward.bbs file.

The mailbox reads a forwarding file,  spool/forward.bbs.  Here  is  a  sample

file.

wb0ttw 0006

ax25 ax0 wb0ttw

wb0ttw

w0tn

mspbul

all

------

wb0gdb

netrom #msparh

..c msparh

all

------ The first word on the first line in a  forwarding  record  is  the

name  of the BBS to forward to. This should be the same type of name which is

shown by the mbox status command. The second word is optional. It specifies a

range  of  hours  when  forwarding may take place. 0006 means that there will

only be forwarding to this station between midnight and 6am.

The second line specifies how to establish the connection.  It  should  start

with  the  protocol (ax25, connect, tcp, telnet or netrom) and be followed by

all the parameters which are nessesary when Nos has to  establish  a  connec-

tion.

Directly after the second line, there may be lines that  start  with  a  dot.

What follows after the dot will be send to the remote BBS as soon as the con-

nection is established.

Then follows the names of a number  of  message  areas,  publib  or  private.

Finally,  there  should  be a couple of '-' signs to separate one  forwarding

record from another. Also terminate the file with a last line of dashes.

7.8.  The /spool/rewrite file.

Read the rewrite file for lines where the first word is a regular  expression

and the second word are rewriting rules.  An optional third field, containing

just the letter "r", when present, instructs Nos to restart the rewrite file,

using  the  new destination address.  The special character '$' followed by a

digit denotes the string that matched a '*' character. The '*' characters are

numbered from 1 to 9.  Example: the line "*@*.* $2@$1.ampr.org" would rewrite

the address "foo@bar.xxx" to "bar@foo.ampr.org".

#

*@g1emm.ampr.org $1

*@g1emm.ampr $1

*@g1emm $1

#

*!*!*!*!*!*!* $7%$6@$5@$4@$3@$2@$1

*!*!*!*!*!* $6%$5@$4@$3@$2@$1

*!*!*!*!* $5%$4@$3@$2@$1

*!*!*!* $4%$3@$2@$1

- 64 -

*!*!* $3%$2@$1

*!* $2@$1

!* $1 r

#

# The End

- 65 -

8.  Setting Bufsize, Paclen, Maxframe, MTU, MSS and Window

Many Nos users are confused by these parameters and do not know  how  to  set

them  properly. This chapter will first review these parameters and then dis-

cuss how to choose values for them. Special emphasis  is  given  to  avoiding

interoperability  problems  that  may  appear when communicating with non-Nos

implementations of AX.25.

8.1.  Hardware Parameters

8.1.1.  Bufsize

This parameter is required by most of Nos's built-in HDLC drivers (eg.  those

for  the  DRSI  PCPA  and  the  Paccomm PC-100). It specifies the size of the

buffer to be allocated for each receiver port. HDLC frames larger  than  this

value cannot be received.

There is no default bufsize; it must be specified in the attach  command  for

the interface.

8.2.  AX25 Parameters

8.2.1.  Paclen

Paclen limits the size of the data field in an AX.25 I-frame. This value does

not  include  the  AX.25  protocol header (source, destination and digipeater

addresses).

Since unconnected-mode (datagram) AX.25 uses UI frames, this parameter has no

effect in unconnected mode.

The default value of paclen is 256 bytes.

8.2.2.  Maxframe

This parameter controls the number of I-frames that Nos may send on an  AX.25

connection  before  it  must stop and wait for an acknowledgement.  Since the

AX.25/LAPB sequence number field is 3 bits wide, this number cannot be larger

than 7.

Since unconnected-mode (datagram) AX.25 uses  UI  frames  that  do  not  have

sequence numbers, this parameter does not apply to unconnected mode.

The default value of maxframe in Nos is 1 frame.

8.3.  IP and TCP Parameters

8.3.1.  MTU

The MTU (Maximum Transmission Unit) is an interface parameter that limits the

size  of  the largest IP datagram that it may handle.  IP datagrams routed to

an interface that are larger than its MTU are each split  into  two  or  more

fragments.  Each fragment has its own IP header and is handled by the network

as if it were a distinct IP datagram, but when it arrives at the  destination

it  is held by the IP layer until all of the other fragments belonging to the

original datagram have arrived. Then they are reassembled back into the  com-

plete,  original  IP  datagram.  The  minimum  acceptable interface MTU is 28

bytes: 20 bytes for the IP (fragment) header, plus 8 bytes of data.

There is no default MTU in Nos; it must  be  explicitly  specified  for  each

- 66 -

interface as part of the attach command.

8.3.2.  MSS

MSS (Maximum Segment Size) is a TCP-level parameter that limits the amount of

data  that  the  remote  TCP will send in a single TCP packet. MSS values are

exchanged in the SYN (connection request) packets that open a TCP connection.

In  the  Nos  implementation  of TCP, the MSS actually used by TCP is further

reduced in order to avoid fragmentation at the local IP interface.  That  is,

the local TCP asks IP for the MTU of the interface that will be used to reach

the destination. It then subtracts 40 from the MTU value  to  allow  for  the

overhead  of  the  TCP  and  IP  headers.  If the result is less than the MSS

received from the remote TCP, it is used instead.

The default value of MSS is 512 bytes.

8.3.3.  Window

This is a TCP-level parameter that controls how much data the local TCP  will

allow the remote TCP to send before it must stop and wait for an acknowledge-

ment. The actual window value used by TCP when deciding how much more data to

send  is  referred  to  as  the effective window.  This is the smaller of two

values: the window advertised by the remote TCP minus the unacknowledged data

in  flight, and the congestion window, an automatically computed time-varying

estimate of how much data the network can handle.

The default value of Window is 2048 bytes.

8.4.  Discussion

8.4.1.  IP Fragmentation vs AX.25 Segmentation

IP-level fragmentation often makes it possible to interconnect two dissimilar

networks,  but it is best avoided whenever possible.  One reason is that when

a single IP fragment is lost, all  other  fragments  belonging  to  the  same

datagram   are  effectively  also  lost  and  the  entire  datagram  must  be

retransmitted by the source.  Even without loss, fragments require the  allo-

cation of temporary buffer memory at the destination, and it is never easy to

decide how long to wait for missing fragments before giving up and discarding

those  that  have already arrived.  A reassembly timer controls this process.

In Nos it is  (re)initialized  with  the  ip  rtimer  parameter  (default  30

seconds)  whenever  progress  is made in reassembling a datagram (i.e., a new

fragment is received).  It is not necessary that all of the fragments belong-

ing  to  a  datagram  arrive  within a single timeout interval, only that the

interval between fragments be less than the timeout.

Most subnetworks that carry IP have MTUs of 576 bytes or more,  so  intercon-

necting  them  with subnetworks having smaller values can result in consider-

able fragmentation. For this reason, IP implementors working  with  links  or

subnets having unusually small packet size limits are encouraged to use tran-

sparent fragmentation, that is, to  devise  schemes  to  break  up  large  IP

datagrams  into  a  sequence  of  link  or subnet frames that are immediately

reassembled on the other end of the link or subnet into the  original,  whole

IP  datagram without the use of IP-level fragmentation. Such a scheme is pro-

vided in AX.25 Version 2.1.  It can break a large IP or NET/ROM datagram into

a  series  of  paclen-sized  AX.25 segments (not to be confused with TCP seg-

ments), one per AX.25 I-frame, for transmission and reassemble  them  into  a

single  datagram  at the other end of the link before handing it up to the IP

or NET/ROM module.   Unfortunately,  the  segmentation  procedure  is  a  new

feature  in  AX.25  and is not yet widely implemented; in fact, Nos is so far

- 67 -

the only known implementation. This creates  some  interoperability  problems

between  Nos  and  non-Nos nodes, in particular, standard NET/ROM nodes being

used to carry IP datagrams. This problem is discussed further in the  section

on setting the MTU.

8.4.2.  Setting paclen and bufsize

The more data you put into an AX.25 I frame, the smaller  the  AX.25  headers

are  in  relation  to  the  total  frame  size. In other words, by increasing

paclen, you lower the AX.25  protocol  overhead.  Also,  large  data  packets

reduce  the overhead of keying up a transmitter, and this can be an important

factor with higher speed modems. On the other hand, large frames make  bigger

targets  for noise and interference. Each link has an optimum value of paclen

that is best discovered by experiment.

Another thing to remember when setting paclen is that the AX.25  version  2.0

specification  limits  it  to  256 bytes. Although Nos can handle much larger

values, some other AX.25 implementations (including digipeaters)  cannot  and

this may cause interoperability problems. Even Nos may have trouble with cer-

tain KISS TNCs because of fixed-size buffers. The original KISS TNC code  for

the  TNC-2  by  K3MC can handle frames limited in size only by the RAM in the

TNC, but some other KISS TNCs cannot.

Nos's built-in HDLC drivers (SCC, PC-100, DRSI, etc) allocate receive buffers

according  to  the maximum expected frame size, so it is important that these

devices be configured with the correct bufsize. To do this, you must know the

size of the largest possible frame that can be received. The paclen parameter

controls only the size of the data field in an I-frame and not the total size

of  the  frame  as it appears on the air. The AX.25 spec allows up to 8 digi-

peaters, so the largest possible frame is (paclen + 72) bytes. So you  should

make bufsize at least this large.

Another important consideration is that  the  more  recent  versions  of  NOS

improve  interrupt  response by maintaining a special pool of buffers for use

by the receive routines.  These buffers are currently fixed in size  to  2048

bytes  and  this can be changed only by editing config.h and recompiling NOS.

This limits bufsize; in fact, attempting to set a larger value may cause  the

driver  not  to  work  at  all. This situation can be detected by running the

memory status command and looking for a non-zero count  of  Ibuffail  events,

although these events can also occur occasionally during normal operation.

One of the drawbacks of AX.25 that there is no way for one  station  to  tell

another  how  large a packet it is willing to accept.  This requires the sta-

tions sharing a channel to agree beforehand on a maximum packet size.  TCP is

different, as we shall see.

8.4.3.  Setting Maxframe

For best performance on a half-duplex radio channel, maxframe  should  always

be  set  to  1.  The  reasons are explained in the paper Link Level Protocols

Revisited by Brian Lloyd and Phil Karn, which appeared in the proceedings  of

the ARRL 5th Computer Networking Conference in 1986.

8.4.4.  Setting MTU

TCP/IP header overhead considerations similar to those  of  the  AX.25  layer

when  setting paclen apply when choosing an MTU.  However, certain subnetwork

types supported by Nos have well-established MTUs, and these should always be

used  unless  you  know  what  you're doing: 1500 bytes for Ethernet, and 508

bytes for ARCNET.  The MTU for PPP is automatically negotiated, and  defaults

to  1500.   Other  subnet  types,  including  SLIP and AX.25, are not as well

- 68 -

standardized.

SLIP has no official MTU, but the most common implementation (for  BSD  UNIX)

uses  an MTU of 1006 bytes.  Although Nos has no hard wired limit on the size

of a received SLIP frame, this is not true for other systems.  Interoperabil-

ity problems may therefore result if larger MTUs are used in Nos.

Choosing an MTU for an AX.25 interface is more complex.  When  the  interface

operates  in  datagram  (UI-frame) mode, the paclen parameter does not apply.

The MTU effectively becomes the paclen of the link.   However,  as  mentioned

earlier,  large packets sent on AX.25 connections are automatically segmented

into I-frames no larger than paclen bytes.  Unfortunately, as also  mentioned

earlier, Nos is so far the only known implementation of the new AX.25 segmen-

tation procedure. This is fine as long as all of the NET/ROM  nodes  along  a

path  are  running  Nos, but since the main reason Nos supports NET/ROM is to

allow use of existing NET/ROM networks, this is unlikely.

So it is usually important to avoid AX.25 segmentation when running  IP  over

NET/ROM.   The way to do this is to make sure that packets larger than paclen

are never handed to AX.25.  A NET/ROM transport header is 5 bytes long and  a

NET/ROM  network header takes 15 bytes, so 20 bytes must be added to the size

of an IP datagram when figuring the size of the AX.25 I-frame data field.  If

paclen is 256, this leaves 236 bytes for the IP datagram. This is the default

MTU of the netrom pseudo-interface, so as long as  paclen  is  at  least  256

bytes,  AX.25  segmentation can't happen. But if smaller values of paclen are

used, the netrom MTU must also be reduced with the ifconfig command.

On the other hand, if you're running IP directly on top of AX.25, chances are

all  of  the  nodes  are running Nos and support AX.25 segmentation.  In this

case there is no reason not to use a larger MTU and let AX.25 segmentation do

its  thing.  If  you  choose  an MTU on the order of 1000-1500 bytes, you can

largely avoid IP-level fragmentation and reduce TCP/IP-level header  overhead

on  file transfers to a very low level.  And you are still free to pick what-

ever paclen value is appropriate for the link.

8.4.5.  Setting MSS

The setting of this TCP-level parameter is somewhat less critical than the IP

and  AX.25 level parameters already discussed, mainly because it is automati-

cally lowered according to the MTU of the local interface when  a  connection

is  created.  Although this is, strictly speaking, a protocol layering viola-

tion (TCP is not supposed to have any knowledge  of  the  workings  of  lower

layers)  this  technique  does  work  well  in  practice.  However, it can be

fooled; for example, if a routing change occurs after the connection has been

opened  and  the new local interface has a smaller MTU than the previous one,

IP fragmentation may occur in the local system.

The only drawback to setting a large MSS is that  it  might  cause  avoidable

fragmentation  at  some  other point within the network path if it includes a

"bottleneck" subnet with an MTU smaller than that  of  the  local  interface.

(Unfortunately,  there  is  presently  no  way to know when this is the case.

There is ongoing work within the Internet Engineering Task Force  on  a  "MTU

Discovery"  procedure to determine the largest datagram that may be sent over

a given path without fragmentation, but it is not yet complete.)  Also, since

the  MSS  you specify is sent to the remote system, and not all other TCPs do

the MSS-lowering procedure yet, this might cause the remote  system  to  gen-

erate IP fragments unnecessarily.

On the other hand, a too-small MSS can result in a  considerable  performance

loss,  especially  when operating over fast LANs and networks that can handle

- 69 -

larger packets. So the best value for MSS is probably 40 less than the  larg-

est  MTU  on your system, with the 40-byte margin allowing for the TCP and IP

headers. For example, if you have a SLIP interface with a 1006 byte  MTU  and

an  Ethernet  interface  with  a  1500  byte MTU, set MSS to 1460 bytes. This

allows you to receive maximum-sized Ethernet packets, assuming  the  path  to

your system does not have any bottleneck subnets with smaller MTUs.

8.4.6.  Setting Window

A sliding window protocol like TCP cannot transfer  more  than  one  window's

worth  of data per round trip time interval. So this TCP-level parameter con-

trols the ability of the remote TCP to keep a long "pipe" full. That is, when

operating  over  a path with many hops, offering a large TCP window will help

keep all those hops busy when you're  receiving  data.  On  the  other  hand,

offering  too  large a window can congest the network if it cannot buffer all

that data. Fortunately, new algorithms for dynamic controlling the  effective

TCP  flow  control window have been developed over the past few years and are

now widely deployed.  Nos includes them, and you can  watch  them  in  action

with  the  tcp  status  <tcb> or socket <sockno> commands.  Look at the cwind

(congestion window) value.

In most cases it is safe to set the TCP window to a small integer multiple of

the  MSS,  (eg.  4times),  or  larger  if  necessary  to fully utilize a high

bandwidth*delay product path. One thing to keep in  mind,  however,  is  that

advertising a certain TCP window value declares that the system has that much

buffer space available for incoming data.  Nos does not actually  preallocate

this space; it keeps it in a common pool and may well "overbook" it, exploit-

ing the fact that many TCP connections are idle for long periods and gambling

that  most  applications will read incoming data from an active connection as

soon as it arrives, thereby quickly freeing the buffer memory.   However,  it

is possible to run Nos out of memory if excessive TCP window sizes are adver-

tised and either the applications go to  sleep  indefinitely  (eg.  suspended

Telnet  sessions)  or  a  lot of out-of-sequence data arrives.  It is wise to

keep an eye on the amount of available memory and to decrease the TCP  window

size (or limit the number of simultaneous connections) if it gets too low.

Depending on the channel access method and link level protocol, the use of  a

window  setting  that exceeds the MSS may cause an increase in channel colli-

sions. In particular, collisions between  data  packets  and  returning  ack-

nowledgements  during  a  bulk file transfer may become common. Although this

is, strictly speaking, not TCP's fault, it is possible  to  work  around  the

problem  at  the  TCP  level  by  decreasing  the window so that the protocol

operates in stop-and-wait mode.  This is done  by  making  the  window  value

equal to the MSS.

8.5.  Summary

In most cases, the default values provided by Nos for each of  these  parame-

ters  will  work  correctly  and give reasonable performance. Only in special

circumstances such as operation over a very poor link or experimentation with

high speed modems should it be necessary to change them.