Personal Communications Library

For PowerBASIC

(PCL4PB)

USERS MANUAL

Version 4.3

April 25, 1995

This software is provided as-is.

There are no warranties, expressed or implied.

Copyright (C) 1994

All rights reserved

MarshallSoft Computing, Inc.

Post Office Box 4543

Huntsville AL 35815

Voice 205-881-4630

FAX   205|880|0925

BBS   205-880-9748

_______

____|__     |                (R)

--+       |    +-------------------

|   ____|__  |  Association of

|  |       |_|  Shareware

|__|   o   |    Professionals

--+--+   |   +---------------------

|___|___|    MEMBER

PCL4PB Users Manual                                               Page 1

C O N T E N T S

Chapter                                                   Page

1.0 Introduction................................................3

1.1 User Support............................................4

1.2 ASP Ombudsman...........................................4

1.3 A Typical Application...................................5

1.4 Installation............................................6

2.0 Library Organization........................................7

2.1 Configuration...........................................7

2.2 Initialization & Termination............................7

2.3 Modem Control & Status..................................8

2.4 Serial I/O..............................................8

2.5 Error Detection.........................................9

2.6 General Support.........................................9

3.0 Library Overview...........................................10

3.1 Using the Library......................................10

3.2 Application Notes......................................10

3.2.1 Terminal Programs................................10

3.2.2 Door Programs....................................10

3.2.3 BBS Programs.....................................11

3.3 Compiling & Linking....................................11

4.0 Talking to Your Modem......................................12

4.1 Modem Standards........................................12

4.2 Flow Control...........................................13

4.3 MODEM_IO functions.....................................14

4.3.1 ModemEcho........................................14

4.3.2 ModemSendTo......................................14

4.3.3 ModemWaitFor.....................................14

4.3.4 ModemQuiet.......................................14

4.3.5 ModemCmdState....................................14

4.3.6 ModemHangup......................................14

4.4 Modem Initialization...................................15

5.0 Problems...................................................16

6.0 Serial Communications......................................17

6.1 Communications Basics..................................17

6.2 Standard Port Addresses................................18

6.3 Running 3 or 4 Ports Concurrently......................19

6.4 Using Multiport Cards..................................20

6.4.1 The DigiBoard....................................20

6.4.2 The BOCA Board...................................20

6.5 Transmitter Interrupts.................................21

6.6 RS232 Signals..........................................22

6.7 National INS8250, INS16450, and INS16550 UARTs.........23

6.8 Register Summary.......................................24

7.0 Example Programs...........................................26

7.1 MINIMAL................................................26

7.2 SIMPLE.................................................26

7.3 LOGIN..................................................26

7.4 DOOR...................................................26

7.5 SELFTEST...............................................26

8.0 Legal Issues...............................................27

8.1 Registration...........................................27

8.2 License................................................28

8.3 Warranty...............................................28

9.0 Summary....................................................29

9.1 Revision History.......................................29

9.2 Function Summary.......................................29

9.3 Further Reading........................................30

10.0 Other MarshallSoft Computing Products......................30

10.1 The Personal Protocol Library for PowerBASIC..........30

PCL4PB Users Manual                                               Page 2

1.0 Introduction

The   Personal   Communications   Library  for  PowerBASIC  (PCL4PB)  is  an

asynchronous  communications  library  designed  for  experienced   software

developers  programming  in  PowerBASIC,  version  3.0c  and above.  The PCL

features:

o Use IRQ2 through IRQ15 with any UART address.

o 35 communications and support functions.

o Supports the high performance 16550 UART.

o Supports the PC/4 and PC/8 DigiBoard.

o Supports the BOCA BB1004, BB1008, and BB2016 boards.

o Supports hardware (RTS/CTS) flow control.

o Interrupt driven receiver & (optionally) transmitter.

o Supports 300 baud to 115,200 baud.

o Supports COM1 through COM8 (through COM20 with multiport boards)

o Adjustable receive queues from 8 bytes to 32 KB.

o Control-BREAK error exit.

o 18 communications error conditions trapped.

o Allows 4 ports to run concurrently (more with multiport boards).

o Complete modem control & status.

o Written in assembly language for small size & high speed.

Why should you buy PCL4PB ?  Several good reasons are:

COMPLETE - PCL4PB  is complete since it provides absolute control  of the

serial ports (including the high performance INS16550).

COMPACT - PCL4PB  is  very  compact at less than 8 KB.  Your application

doesn't carry a lot of excess code.

FAST - PCL4PB is fast since it will run at 38400  baud  on  even  slow

8088 PCs (4.77 MHZ) and at 115200 baud on most everything else.

SUPPORT - If you get stuck, you talk to the  programmer  that  wrote  the

code, not a person hired to answer the phone.

BBS  - A  BBS  is  available (2400 through 14400 baud, N81) to provide

immediate support as necessary.

NEWSLETTER - A  one  year  subscription  to  the  MSC newsletter discusses

communications problems and solutions (published quarterly).

PRICE - You get PCL4PB for a very reasonable price!

PCL4PB Users Manual                                               Page 3

1.1 User Support

We  want  you to be successful in developing your applications using PCL4PB!

We depend on our customers to let us know what they need in a communications

library.  This means we are committed to providing the  best  communications

library  that we can. If you have any suggestions or comments, please let us

know!

We provide customer support for registered customers by voice, FAX, BBS, and

mail.  We provide limited support for unregistered users by  voice  and  BBS

only.

If you are having a problem using PCL4PB, call us  at  205-881-4630  between

1:30  PM and 9:30 PM (CST) Monday through Friday. You can also call at other

times and leave a message, and call back later for a reply. Registered users

(ONLY) can also FAX us at 205-880-0925 at any time (24 hours).

However,  we  can  only  answer  questions  with respect to using the PCL4PB

library.  We cannot help you program your application, but we'll be glad  to

discuss it with you.

You  may  also  call  our User Support BBS (2400 to 14400 baud, no parity, 8

data bits, 1 stop bit) at 205-880-9748 and leave a message  (address  it  to

the SYSOP).  We will usually have a reply ready for you within 24 hours.

The BBS is available 24 hours per day. All files are in standard ZIP format.

The  BBS  will  contain  the  latest  shareware  version of all MarshallSoft

Computing products as well as related files such as:

BUGS.ZIP     -  Bug report.

NEWS.ZIP     |  Latest news regarding our products.

PRODUCTS.ZIP -  List of all shareware products.

The MarshallSoft  Computing,  Inc.   newsletter  "Comm  Talk"  is  published

quarterly.  It discusses various communications problems and solutions using

PCL4PB  as  well  as related information. Registered users receive a one year

complimentary subscription  when  first  registering  and  for  each  update

purchased.

1.2 ASP Ombudsman

MarshallSoft Computing, Inc.  is a member of the  Association  of  Shareware

Professionals  (ASP).   ASP  wants to make sure that the shareware principle

works for you.  If you are unable to  resolve  a  shareware-related  problem

with  an  ASP  member  by contacting the member directly, ASP may be able to

help. The ASP Ombudsman can help you resolve a dispute or  problem  with  an

ASP  member,  but  does not provide technical support for members' products.

Please write to the ASP Ombudsman at  545  Grover  Road,  Muskegon,  MI  USA

49442-9427,  Fax  616-788-2765,  or send a CompuServe message via CompuServe

Mail to ASP Ombudsman 70007,3536.

PCL4PB Users Manual                                               Page 4

1.3 A Typical Application

The following code is the complete MINIMAL.BAS program. It talks on COM1  at

2400 baud. The code is as follows:

+--------------------------------------------------------+

|  '                                                     |

|  '  MINIMAL.BAS:  Talk on COM1 @ 2400 baud             |

|  '                                                     |

|  $ERROR ALL ON                                         |

|  $LIB ALL OFF                                          |

|  $LINK "PCL4PB.OBJ"                                    |

|  $INCLUDE "PCL4PB.BI"                                  |

|  DIM Buffer(256+16) AS BYTE                            |

|  DIM BufSeg AS WORD                                    |

|  DIM Code AS INTEGER                                   |

|  'Reset port                                           |

|  BufSeg = 1 +VARSEG(Buffer(0)) +(VARPTR(Buffer(0))\16) |

|  Code = SioRxBuf(%COM1, BufSeg, %Size256)              |

|  Code = SioReset(%COM1, %Baud2400)                     |

|  DO                                                    |

|    'Anything incoming ?                                |

|    Code = SioGetc(%COM1, 1)                            |

|    IF Code = 13 THEN                                   |

|      PRINT                                             |

|    ELSEIF Code >= &H20 THEN                            |

|      PRINT CHR$(Code);                                 |

|    END IF                                              |

|    'Anything to send ?                                 |

|    AnyKey$ = INKEY$                                    |

|    IF AnyKey$ <> "" THEN                               |

|      'Exit if user types Escape (ESC=27)               |

|      IF AnyKey$ = CHR$(27) THEN EXIT DO                |

|      Code = SioPutc(%COM1,ASC(AnyKey$))                |

|    END IF                                              |

|  LOOP                                                  |

|  Code = SioDone(%COM1)                                 |

|  END                                                   |

|                                                        |

+--------------------------------------------------------+

The  above code can be compiled by using either the command line compiler by

typing

PBC MINIMAL.BAS

or choosing Open  from  the  File  menu  in  the  IDE,  and  then  selecting

MINIMAL.BAS.

PCL4PB Users Manual                                               Page 5

1.4 Installation

(1)  Before  installation of PCL4PB, your PowerBASIC compiler should already

be installed on your system and tested.

Each test  program  can  be  compiled  either  from  within  the  integrated

development environment or by using a command file.  For example, the SIMPLE

application can be compiled either by choosing SIMPLE.BAS from the IDE or by

typing SIMPLE_ which invokes the command line compiler.

(2)  Make  a  backup  copy  of  your  distribution  disk.  Put your original

distribution disk in a safe place.

(3) Create a work directory on your work disk (normally your harddisk).  For

example, to create a work directory named PCL4PB, we first log onto the work

disk and then type:

MKDIR PCL4PB

(4)  Copy  all  the  files from your backup copy of the distribution disk to

your work directory.  For example, to copy from the A: drive  to  your  work

directory, we type:

CD PCL4PB

COPY A:*.*

(5)  Compile  MINIMAL.BAS.   To  use  the integrated development environment

(IDE), simply choose MINIMAL.BAS from the File menu, then Open, and choose

MINIMAL.BAS.

You can also use the command line compiler by typing MINIMAL_.

(6)  Also  compile SIMPLE, LOGIN, and DOOR.

(7)  The  recommended  way  to  test  SIMPLE  is  to run it on two computers

connected by a null modem cable.  Whatever is typed on one  computer  should

be displayed on the other. SIMPLE can also be tested by connecting your port

to a modem.

(8)  Repeat  the above with the LOGIN and DOOR example programs.

PCL4PB Users Manual                                               Page 6

2.0 Library Organization

The  PCL4PB library is organized into six categories of functions.  Refer to

the PCL Reference Manual (PCL4PB.REF) for details on individual functions.

2.1 Configuration

There are three functions in the configuration category.  SioPorts sets  the

number  of PC and DigiBoard (or BOCA board) ports. SioUART is used to change

the UART base address for a communications port to a  non-standard  address,

while  SioIRQ  is  used  to  assign  a  nonstandard IRQ line to a port.  See

chapter 6 for more details on standard UART addresses and IRQ lines.

The configuration functions SioPorts, SioUART  and  SioIRQ  must  be  called

before  calling any other library functions.  Be very careful in using these

functions.  Remember that your serial hardware must support the UART and IRQ

that you specify.  Always test any new configuration immediately.

SioPorts -  Sets number of PC and DigiBoard (or BOCA board) ports.

SioUART  |  Sets the UART base address.

SioIRQ   -  Assigns an IRQ line to a port.

THE IRQ GOLDEN RULE: You may open (via  SioReset)  only  one  port  per  IRQ

(except for the DigiBoard and BOCA board).

2.2 Initialization & Termination

There are eight functions in the initialization  and  termination  category.

Together,  SioParms,  SioFIFO,  SioRxBuf,  SioTxBuf, and SioReset initialize

your serial communications system.  Your application must call SioParms  and

SioRxBuf  before  calling  SioReset,  and SioReset must be called before any

serial I/O processing can be done.

After initialization, SioParms and SioBaud can be called again to change the

communications parameters without resetting the serial port. SioFlow can  be

called to enable hardware flow control.

Before  exiting  from  your  application, SioDone must be called. Failure to

call SioDone can crash your system later.

SioRxBuf   - Sets up receive buffer.

SioTxBuf   | Sets up transmitter buffer.

SioFIFO    | Sets the interrupt level for the INS16550.

SioParms   | Sets parity, stop bits, and word length.

SioReset   | Initialize a serial port for processing.

SioDone    | Terminates further serial processing.

SioBaud    | Sets the baud rate of the selected port.

SioFlow    - Enables / disables flow control.

PCL4PB Users Manual                                               Page 7

2.3 Modem Control & Status

There are nine functions in the modem  control  and  status  category  which

provide  your  application with complete control over the status and control

bits of your modem.

There are two modem control bits, "Data Terminal Ready" (DTR)  and  "Request

To  Send"  (RTS).   These  bits  can  be read, set, or cleared by SioDTR and

SioRTS.

There are four modem status bits, "Data Set Ready" (DSR),  "Clear  To  Send"

(CTS),  "Ring Indicator" (RI), and "Data Carrier Detect" (DCD). SioModem can

read any of the modem status bits.  SioDSR, SioCTS, SioRI,  and  SioDCD  can

only  read  their respective modem status bit. SioGetDiv reads the baud rate

divisor register so the baud rate can be determined.

Refer to the chapter entitled "RS232 Signals" for a discussion  of  each  of

the control and status bits.

SioDTR    - Set, clear, or read the Data Terminal Ready (DTR) bit.

SioRTS    | Sets, clears, or reads the Request to Send (RTS) line.

SioModem  | Reads the modem status register.

SioDSR    | Reads the Data Set Ready (DSR) modem status bit.

SioCTS    | Reads the Clear to Send (CTS) modem status bit

SioDCD    | Reads the Data Carrier Detect (DCD) modem status bit.

SioRI     | Reads the Ring Indicator (RI) modem status bit.

SioRead   | Reads the contents of the 7 UART registers.

SioGetDiv - Reads the baud rate divisor registers.

2.4 Serial I/O

There  are  eight  library  functions in the serial I/O category.  Together,

these functions give the programmer complete control over serial I/O. Higher

level functions such as protocols and  smart  modem  communications  can  be

completely  implemented  in  terms of these functions.  Refer to the example

code.

SioGetc and SioPutc perform all the actual serial I/O.   SioUnGetc  "ungets"

the  last  serial  byte  read.   SioRxFlush  clears  the receive queue while

SioTxFlush clears the transmit queue. SioLine can be used to test  for  UART

errors.   SioRxQue  returns  the  number of bytes in the receive queue while

SioTxQue returns the number of bytes in the transmit queue.

SioGetc    - Reads the next character from the serial line.

SioPutc    | Transmit a character over a serial line.

SioUnGetc  | "Un-gets" (puts back) a specified character.

SioRxFlush | Flush (clears) the receive buffer.

SioRxQue   | Returns the number of characters in the RX queue.

SioTxFlush | Flush (clears) the transmit buffer.

SioTxQue   | Returns the number of characters in the TX queue.

SioLine    - Reads the line status register.

PCL4PB Users Manual                                               Page 8

2.5  Error Detection

There are four functions in the error detection category. They are concerned

with detecting or reporting communications errors.  Use of  these  functions

can make your application significantly more robust.

SioBrkKey  can  be  used  as  an  "emergency"  exit  from  your application.

SioBrkSig can read or modify  the  UART  break  bit.   This  is  useful  for

signalling   the   remote  system  that  a  fatal  condition  has  occurred.

SioLoopBack can be used to  test  the  integrity  of  your  UART.   SioError

displays  a  error  message  corresponding  to an error code returned from a

PCL4PB function (every PCL4PB function returns a code).

SioBrkKey - Returns non-zero if the Control-BREAK key was pressed

SioBrkSig | Asserts, cancels, or detects the RS232 BREAK  signal.

SioError  - Displays  error  in  text.

SioLoopBack - Performs a UART loopback test.

2.6 General Support

There are three functions in the general support category.  SioInfo  returns

information  about  PCL4PB,  SioDelay  delays  a  number of system tics, and

SioTimer returns the number of system tics since midnight.

SioInfo     - Returns the library version & memory model.

SioDelay    | Delays one or more tics (18.2 tics per second).

SioTimer    - Returns the number of system clock tics.

PCL4PB Users Manual                                               Page 9

3.0 Library Organization

3.1 Using the Library

The PCL4PB has been tested on a Tandy 1000 (4.77 MHZ 8088 IBM PC  clone),  a

Tandy  3000  (80286  IBM AT clone), a Tandy 1400LT (IBM XT clone), a Gateway

2000 386 (25 MHZ 80386-DX), and a Gateway 2000 486 (66MHZ 80486-DX).  PCL4PB

has also been tested under MSDOS 2.11, 3.2, 3.3, 4.01, 5.0, 6.0 and 6.2.

Please examine the PCL4PB.BI file. Note that COM1 is defined as  port  zero,

not  port  one.   The  user  must  assume  the responsibilty for passing the

correct information when calling PCL4PB functions.

If there are any conflicts between PCL4PB definitions  and  those  in  other

libraries,  the  PCL4PB definitions can be changed in the PCL4PB.BI file and

any file that uses the definition. There is  no  change  necessary  for  the

library code itself.

The  PCL4PB  libraries contain no references to any runtime libraries.  Only

BIOS and MSDOS functions are called.

3.2 Application Notes

3.2.1 Terminal Programs

The  "terminal  program" is the most common class of communications program.

It is used to call up a BBS or on-line service such as  CompuServe,  America

On-Line,  etc.   Refer  to  the  programs  SIMPLE and LOGIN in section 7 for

examples of simple terminal programs.

A  more  sophisticated terminal program featuring ASCII, XMODEM, YMODEM, and

ZMODEM protocol file transfers can be found in our  sister  product  --  The

Personal  Protocol Libray (PPL4PB). Source code is included in the shareware

distribution for everything except ZMODEM and the script intepreter.

3.2.2 Door Programs

In  order  to  write a door program which "takes over" a serial port without

resetting the port or changing the baud rate, call SioReset with NORESET  as

the  second  argument  rather  than the baud rate. Call SioGetDiv to get the

baud rate divisor if the baud rate must  be  determined.  Be  sure  to  call

SioDone  before  returning  to  the invoking program.  Refer to the DOOR.BAS

example program.

PCL4PB Users Manual                                               Page 10

3.2.3 BBS Programs

If you are designing a BBS program (also known as HOST  programs),  consider

using  16550  UARTS.   You  should  also choose a multiport card such as the

DigiBoard or BOCA board if you wish to run more than 4 ports simultaneously.

If  you  are using an error correcting modem, then you should be sure to set

flow control and fix your baud rate at the highest possible transfer  rates.

For  14,400  modems, this means 19200 or 38400. You may need a 16550 UART in

order to run at the higher speed.

If you are using an older multi-speed modem (say  1200,  2400,  4800,  9600)

that doesn't use flow control, you should change your baud rate to match the

baud rate returned by the CONNECT message from the modem.

Most  multi-port  (dumb) cards are compatible to either the DigiBoard or the

BOCA board.  Some of these are:

DigiBoard  612-943-9020  PC/4, PC/8, PC/16

BOCA       407|241|8088  BB1004, BB1008, BB2016

Gtek       800|282|4835  BlackBoard-4, BBS550

Commtech   316-636-1131  Fastcom:4, Fastcom:8

3.3 Compiling and Linking

Registered  users  may  wish  to  assemble  PCL4PB.ASM. To assemble using the

Microsoft assembler:

MASM PCL4PB /DPASCAL_MODEL /MX;

To enable transmitter interrupts, add "/DSET_TBE" to the MASM  command  line

above:

MASM PCL4PB /DPASCAL_MODEL /DSET_TBE /MX;

PCL4PB Users Manual                                               Page 11

4.0 Talking to Your Modem

A modem is used to extend the  distance  over  which  you  may  communicate.

Without  a  modem, your RS232 cable is limited to a maximum of approximately

50 feet.  But with a modem, you can communicate literally around the world.

Also refer to section 4.3  "MODEM_IO  functions"  for  details  on  MODEM_IO

functions. These functions facilitate communications with modems.

4.1 Modem Standards

Two modems can communicate over a telephone line only if they are both using

the  same  signaling frequencies and modulation, which are determined by the

the modem standards used.  Modem standards can be divided into  three  sets:

(1) speed, (2) data compression used, and (3) error control.

The  Bell  standards  (103  &  212A)  are  those  of  AT&T.   The CCITT (The

International Consultative Committee for Telephone and Telegraph)  standards

are designated as "V. ".

Speed

Bell 103  -   300 baud

Bell 212A |  1200 baud

V.21      |   300 baud

V.22bis   |  1200 & 2400 baud

V.32      |  4800 & 9600 baud

V.32bis   |  4800, 7200, 9600, 12000, and 14400 baud

V.34      -  through 28800 baud.

Data Compression

MNP 5     -  Microcom Networking Protocol (proprietary).

V.42bis   -  International data compression standard.

Error Control

MNP 2,3,4 -  Three level error correction (public domain).

V.42      -  International error correction standard.

Most of the newer high speed modems use  several  of  the  above  standards.

However,  not  all  combinations of modem makes communicate easily with each

other, especially at high speeds (9600 and up).

PCL4PB Users Manual                                               Page 12

4.2 Flow Control

With  modems  using data compression, the modem to modem connection will run

at various speeds depending on the quality of the  line.   The  computer  to

modem  connection  will be at a fixed baud rate. Therefore, a protocol (flow

control) is necessary to synchronize the data flow between a modem  and  the

computer  to  which  it  is  connected.   Refer  to  your  modem  manual for

information on flow control protocols supported.

Two flow control protocols are used by most all modems  which  require  flow

control.   Software  flow  control is called "XON/XOFF" (other software flow

control character pairs are defined but operate the same  as  XON/XOFF)  and

hardware  flow  control is called "RTS/CTS".  Most modems which require flow

control enable hardware flow control by default.

In XON/XOFF (software) flow control, the computer suspends transmitting data

if it receives a XOFF character (13  hex)  from  the  modem,  and  continues

transmitting  when  it  receives  a XON character (11 hex).  Similiarly, the

computer can signal the modem not to send any more data  by  transmitting  a

XOFF  to  it,  and  can tell the modem to continue transmission be sending a

XON.

In RTS/CTS (hardware) flow control, the RTS line is used by the computer  to

signal  the  modem  ,  while the CTS line is used by the modem to signal the

computer.  The RTS line is set OFF by the computer  to  tell  the  modem  to

suspend  transmission,  and  set  to  ON  to  tell  the  modem  to  continue

transmission.  The CTS line is set to OFF by the modem to tell the  computer

to  stop  transmitting,  and  set  to  ON  to  tell the computer to continue

transmitting.

Given the choice, always choose hardware flow  control  over  software  flow

control  so  that  all  data  transmission is transparent.  If hardware flow

control is not the default (which it almost always is),  you  should  modify

your modem initialization string to turn hardware flow control on.

You will not need to use flow control when exchanging data between computers

connected  by  a  null  modem  cable  (no  modems) as long as each packet is

acknowledged (as in XMODEM and YMODEM) and the buffer size is  at  least  as

large as the packet size.

PCL4PB Users Manual                                               Page 13

4.3 MODEM_IO Functions

The  file  MODEM_IO.BAS  contains  several functions that ease communicating

with your modem.  Look in the SIMPLE.BAS  code  for  examples  of  their  use.

ModemSendTo   : Sends string (including control chars) to the modem.

ModemWaitFor  : Waits for a particular string from the modem, passing

all else through.

ModemQuiet    : Waits for continuous quiet of specified duration.

ModemHangup   : Hangs up the modem.

ModemCmdState : Goes into the modem's command state.

ModemEcho     : Echos all serial incoming bytes to the display.

4.3.1 ModemEcho

Syntax: FUNCTION ModemEcho(Port,Echo)

The  ModemEcho  functions  copies bytes from serial input to the display for

'Echo' timer tics (18.2 tics per second).

4.3.2 ModemSendTo

Syntax: FUNCTION ModemSendTo(Port,Pace,Text$)

The ModemSendTo function sends the  characters  in  the  string  'Text$'  to

serial output. There is a delay of 'Pace' timer tics between characters.

4.3.3 ModemWaitFor

Syntax: FUNCTION ModemWaitFor(Port,Tics,CaseFlag,Text$)

The  ModemWaitFor function waits for characters from serial input that match

the string 'Text$'. A total of 'Tics' timer tics are allowed  before  timing

out  and  returning  FALSE.   If  the  'CaseFlag'  is  TRUE, then the string

comparison is case sensitive.

4.3.4 ModemQuiet

Syntax: FUNCTION ModemQuiet(Port,Tics)

The ModemQuiet function waits for continuous  quiet  of  'Tics'  timer  tics

before  returning.  Any  incoming character is echoed to the display and the

wait begins anew.

4.3.5 ModemCmdState

Syntax: FUNCTION ModemCmdState(Port)

The ModemCmdState sets the modem to command state, provided that your  modem

uses  a guard time of 1 second and the standard 3 characters "+-+". Refer to

your Modem manual for information on setting command states.

4.3.6 ModemHangup

Syntax: FUNCTION ModemHangup(Port)

The ModemHangup command hangs up the modem by  first  calling  ModemCmdState

and then sending the string "ATH0".

PCL4PB Users Manual                                               Page 14

4.4 Modem Initialization

If  your  application uses a modem (as opposed to using a null modem cable),

then you should always send an initialization string to your modem if it  is

a  programmable  modem  such as those made by Hayes.  Communication programs

such as PROCOMM and TELIX always send such a string automatically as soon as

they start up.

The particular initialization string depends on the make of your modem.  For

Hayes and Hayes AT command  set  compatible  modems,  the  following  string

(followed by a carriage return) may work:

AT E1 S7=60 S11=60 V1 X1 Q0 S0=0

Refer to your Modem User's Guide for a full discussion of these commands.  A

brief description is as follows:

AT     Modem attention command.

E1     Modem will echo what you send to it.

S7=60  Wait 60 seconds for carrier and/or dial tone.

S11=60 Use 60 milliseconds for tone dialing duration & spacing.

V1     Display result code as words (not numbers).

X1     Use the extended result message (CONNECT XXXX) set.

Q0     Modem returns result codes.

S0=0   Do not answer RING.

If  your application will answer incoming calls, then set the S0 register to

the ring on which to automatically answer. Or, use the ATA command to answer

the RING.

If you send the above codes by using SioPutc (as opposed to typing them from

the keyboard), then follow these guidelines:

(1) Send an initial carriage return before the initialization string.

(2) Pause at least two tics (18 tics to the  second)  after  each  character

sent  as  your  modem needs the time to perform its own internal processing.

Pause a little longer if your modem is  not  accepting  your  initialization

string.

(3)  Pause  one  and a half seconds after sending any initialization command

such as ATZ or AT&F since your modem must do quite a bit of processing.

If you experience any problems in initializing your Hayes modem, you  should

first reset it to factory settings by sending:

AT&F

Your modem  may  require  additional  initialization  other  than  presented

above.  Refer  to  your modem manual for details. It you have a comm program

such as ProComm or Telix that is known to initialize your  modem  correctly,

then you may wish to use the same initialization string.

Refer  to  the  SIMPLE  program (functions ModemSendTo and ModemWaitFor in the

file MODEM_IO.BAS) for an example of sending an initialization string  to  a

Hayes compatible modem.

PCL4PB Users Manual                                               Page 15

5.0 Problems

If you cannot get your application to run properly, first  compile  and  run

the  terminal  emulator program SIMPLE provided on your distribution disk.  If

you are using a null modem cable or a non-programmable modem, be sure to set

the HAYES constant to 0 in the source code (%AT.COMMAND.SET = 0). If you are

using a Hayes compatible modem, set the AT.COMMAND.SET constant  to  1.   If

you  are  using a programmable modem which is not Hayes compatible, then you

must modify the initialization string for your particular modem.

If your application does not run but SIMPLE runs correctly, then you have most

likely  made  a  programming  mistake  in  your  application.   MarshallSoft

Computing  cannot  debug  your  application,  especially over the telephone!

However, consider each of the following when searching for an error in  your

application.

1.  Have you included the file PCL4PB.BI in your application ?

2.   Did  you  link  with  the  correct  PCL4PB  library ?  This is the most

probable cause if your application 'hangs' as soon as it starts and you must

reboot.

3.  Is your receive buffer large enough ? If you are using 1K data blocks in

YMODEM,  then  your  receive  buffer should be at least 1K (2K if baud rates

above 19200 are to be used).

4.  Have you selected too high a baud rate (if you are using a slow PC) ? If

only one COM port is being run, you should be able to run at 38400  baud  on

8088 machines and 115200 on most 286 and all 386 and 486 machines.

5.   Are  you attempting to run another application in the background ?  Try

running without any other programs running in the background (unload all TSR

programs).

6.  If you are running two COM ports simultaneously, are you using  separate

receive buffers ? (you should).

7.   Did SioReset return a zero value ?  If not, then you must call SioReset

again. See SIMPLE.BAS for an example.

8.  Did you send the proper initialization string to your modem ?   Did  you

set DTR and RTS ? (you should).

9.   Do  you  have more than one COM1 port, etc.  For example, if you have a

COM1 port on your motherboard, you cannot add another  COM1  port  or  modem

board that uses COM1 without first disabling the COM1 on the motherboard.

10.   Your  first  comm  port should be COM1.  If you have a second port, it

should be COM2, not COM3 or COM4.

11.  Are you passing the proper segment of the receive (or transmit) buffer?

See SIMPLE.BAS for an example.

PCL4PB Users Manual                                               Page 16

6.0 Serial Communications

6.1 Communications Basics

The heart of serial  communications  is  the  UART  (Universal  Asynchronous

Receiver  Transmitter).   The  IBM PC/XT/AT and compatibles use the INS8250,

INS16450, or the INS16550 UART.  The purpose of the UART is:

(1) To convert bytes from the CPU (Central Processing Unit), into  a  serial

format  by  adding  the  necessary start, stop, and parity bits to each byte

before transmission, and to then transmit each bit at the correct baud rate.

(2) To convert the incoming stream (at a specified baud rate) of serial bits

into bytes by removing the start, stop, and parity bits  before  being  made

available to the CPU.

The  UART  is part of the serial interface circuitry which allows the CPU to

send and receive signals over the RS232 lines. This  can  be  diagrammed  as

follows:

Serial Interface

+-------------------+

|                   |

+-----+  Data Bus  |     +------+      |    RS232 Signals

| CPU +------------+     | UART |      +----------------*

+-----+            |     +------+      |

|                   |

+-------------------+

The INS8250/16450/16550 UART is capable of operating in one  of  two  modes,

"polled" and "interrupt driven".  The serial communications functions in the

BIOS  uses  the  polled method.  In this approach, the CPU is typically in a

loop asking the UART over and over again if it has a byte ready. If it does,

the polling code returns the byte.  But, if the next byte  comes  in  before

the polling code is executing again, then that byte is lost.

In the interrupt driven approach (used by PCL4PB for incoming data), when  a

byte  is  received by the UART, an interrupt is generated and the "Interrupt

Service Routine"  (ISR)  is  executed  immediately,  suspending  temporarily

whatever  else is executing. The ISR then moves the byte to a buffer so that

your application program can later read it. Refer to the  sections  6.6  and

6.7  entitled  "RS232  Signals" and "National INS8250 UART" respectively for

further information on these topics.

PCL4PB Users Manual                                               Page 17

6.2 Standard Port Addresses

There  are  a  few  things to know about how serial communications ports are

used by IBM PC/XT/AT and compatible computers.  The  standard  IBM  PC/XT/AT

configuration values are as follows:

Port    Reg.   IRQ  Vector

COM1    3F8H    4     12

COM2    2F8H    3     11

COM3    3E8H    4     12

COM4    2E8H    3     11

(Refer to your DigiBoard manual for DigiBoard addresses, or your BOCA  board

manual for BOCA port addresses).

PCL4PB  assumes the above values.  If necessary, the UART base address can be

changed by SioUART, and IRQ lines can be re-assigned  by  SioIRQ.   Remember

that  each  port to be used concurrently must have a unique IRQ line.  Refer

to the PCL4PB Reference Manual for specific details.

When installing new  communications  cards,  the  following  guidelines  are

recommended:

(1)  Be  sure to read the documentation for the hardware you are installing.

Pay special attention to UART base addresses and IRQ lines, particularly  if

trying to set up a non-standard configuration.

(2)  If  you  have  a  choice in base addresses and IRQ lines, always choose

standard values as defined above.

(3) The first port should be COM1, the second COM2, etc.  Do NOT  skip  over

any port.

(4)  Use SioUART to zero all unused ports (for example, call SioUART(COM4,0)

if there is no COM4 port installed).

(5) Be carefull not to configure two ports for the  same  address.  This  is

easier to do than you may believe.

(6)  Choose  an  external  modem over an internal one.  It is much easier to

debug problems with an external modem than an internal one.

(7) Select hardware flow control (RTS/CTS) if flow control is  required  and

hardware flow control is not the default.

(8)  Always  test your port as soon as it is installed. Try several programs

that use the communications ports.

PCL4PB Users Manual                                               Page 18

6.3 Running 3 or 4 Ports Concurrently

PCL4PB supports up to 4 serial ports running concurrently (more if you  have

a  DigiBoard  or BOCA board).  One free interrupt for each port is required.

Refer to the next section if you have a DigiBoard or BOCA board.

Interrupts IRQ4 and IRQ3 are dedicated to  the  communications  ports  in  a

standard  IBM  PC/XT/AT configuration.  IRQ4 is shared between COM1 and COM3

while IRQ3 is shared between COM2 and COM4.  This means that you can run two

ports simultaneously provided that they don't share an interrupt.

Suppose that you wish to run 3 ports simultaneously. To begin, you must have

3 serial UARTs installed on your computer.  Assume,  for  purposes  of  this

discussion,  that  COM1  is installed on your motherboard, and that you have

purchased a new 2 port serial communications board.

You should be able to configure the first serial board port as  COM2,  which

uses IRQ3.  Refer to the manual that came with your serial board.

In  order  to  run the third serial port concurrently with the first two, an

unused interrupt must be found.  If your serial card can use only  IRQ3  and

IRQ4,  then there is no way to run a third line since IRQ4 and IRQ3 are used

for COM1 and COM2.

However, many serial cards can use other IRQs, typically IRQ2 through  IRQ5.

Since  IRQ5  is  normally  used  for  a  second  printer  port, it is a good

candidate for COM3.  To use IRQ5 for the third serial port, first  set  your

serial card to use IRQ5 for COM3 (refer to your serial card manual) and then

add the following line to your applications code before calling SioReset:

SioIRQ(COM3,IRQ5);

Don't  forget  to  disable  any device that might use IRQ5, such as a second

printer port or a music card.   Unfortunately,  there  is  no  easy  way  to

determine  that  you have no conflicts until you actually attempt to use the

IRQ.  If there are conflicts, your system will probably hang  and  you  will

have to reboot.

To  run  a  fourth  serial  port,  another  free IRQ must be found.  On some

systems, IRQ7 can be used. To use IRQ7 for the fourth serial port, first set

your serial card to use IRQ7 for COM4 and then add:

SioIRQ(COM4,IRQ7);

One way to determine which IRQs are in use is to  first  boot  your  system,

start  DEBUG  and then type "i21" (after the - prompt).  The Master PIC mask

register byte will be displayed.  IRQ7  corresponds  to  the  leftmost  bit,

while  IRQ0 corresponds to the rightmost bit.  A zero bit indicates that the

corresponding IRQ is in use, while a one bit indicates that the IRQ is free.

To summarize, your serial card must be able to  generate  the  correct  IRQ,

which  is not already being used. Refer to the entry for the SioIRQ function

in the PCL4PB Reference Manual.

PCL4PB Users Manual                                               Page 19

6.4 Using Multiport Cards

The PCL4PB library supports the dumb Digiboard (PC/4 & PC/8)  and  the  dumb

BOCA board (BB1004, BB1008, and BB2016). PCL4PB will also run with most dump

multi-port cards, since they all usually are programmed in the same way. See

Section 3.2.3 "BBS Programs" for a list of multiport card manufacturers.

6.4.1 The DigiBoard

PCL4PB supports the DigiBoard PC/4 and PC/8. In order to use the  DigiBoard,

you  must  configure  PCL4PB  using  the SioPorts(), SioUART(), and SioIRQ()

functions.

Your PCs ports must be partitioned into "standard" PC ports  and  dumb  card

ports.  Remember that standard PC ports cannot share IRQs like the DigiBoard

(or  BOCA  board)  can. If you are using IRQ4 and IRQ3 for standard PC ports

COM1 and COM2, then you cannot use either for DigiBoard ports (try  IRQ5  or

IRQ7).

Suppose  that  COM1 through COM2 are standard PC ports (using IRQ4 and IRQ3)

and you have installed a PC/8 DigiBoard  that  you  wish  to  use  for  COM3

through  COM10 using interrupt line IRQ5.  You choose to use the recommended

DigiBoard UART addresses starting at 0x100:

DEFINT A-Z

RC = SioPorts(8,COM1,&H140,DIGIBOARD)  'COM1 = 1st BOCA port

Address = &H100                        '1st BOCA UART address

FOR Port=COM1 TO COM8                  'look at each port

RC = SioUART(Port,Address)           'set the UART address

Address = Address + 8                'compute next address

RC = SioIRQ(Port,IRQ5)               'set the BOCA IRQ

NEXT I

The DigiBoard uses 0x140 for the status address for odd interrupts and 0x141

for even interrupts.

Digiboard may be contacted at 6400 Flying  Cloud  Drive,  Eden  Prairie,  MN

55344.  Telephone 612-943-9020 or FAX 612-943-5398.

6.4.2 The BOCA Board

PCL4PB  supports  the  dumb  BOCA  board.   As  with the DigiBoard, you must

configure PCL4PB before using the BOCA board.

For example, to configure the BOCA BB2016 to use COM1 to  COM16,  with  base

addresses starting at 0x100 and IRQ5:

DEFINT A-Z

RC = SioPorts(16,COM1,&H107,BOCABOARD) 'COM1 = 1st BOCA port

Address = &H100                        '1st BOCA UART address

FOR Port=COM1 TO COM16                 'look at each port

RC = SioUART(Port,Address)           'set the UART address

Address = Address + 8                'compute next address

RC = SioIRQ(Port,IRQ5)               'set the BOCA IRQ

NEXT I

BOCA  may  be  contacted at BOCA Research, Inc., 6413 Congress Avenue, Suite

130, Boca Raton, FL 33487.  Phone 407-241-8088, FAX 407-997-0918.

PCL4PB Users Manual                                               Page 20

6.5 Transmitter Interrupts

Transmitter interrupts are supported by the library.  Separate libraries are

provided, one with transmitter interrupts enabled  and  one  without.   When

transmitter interrupts are NOT enabled, the following logic occurs everytime

you call SioPutc():

1. Wait for transmit buffer to become empty. The transmit

buffer may not be empty if the previous transmit is not

completed (the UART breaks down the byte & sends 1 bit

at a time).

2. When the transmit buffer is empty, the byte from the

SioPutc() call is loaded into the transmit buffer and

control is returned to the caller.

Note that you can not write to the UART any faster the the UART baud rate.

When transmitter interrupts are enabled, the byte from SioPutc() is put into

a previously prepared (by SioTxQue) transmitter queue. The interrupt service

routine  fetches bytes from this queue as soon as the previous byte has been

sent.

While you can now call SioPutc() faster than the baud rate, bytes are  still

transmitted at the given baud rate.

The   above   sounds   like  transmitter  interrupts  are  the  way  to  go.

Unfortunately, this is often NOT the case.  Most applications  will  perform

better if transmitter interrupts are NOT enabled.

The  reason  is that transmitter interrupts double the amount of code in the

time critical interrupt service routines.  While the library is processing a

transmitter interrupt (which can take a while), incoming bytes  can  not  be

processed.  What this means is that a given machine can run at a higher baud

rate without transmitter interrupts. This problem is compounded when running

multiple ports simultaniously.

However,  there are a few application areas where transmitter interrupts are

preferable.  If your application will be  transmitting  blocks  of  data  at

fairly slow baud rates you might profit from enabling transmitter interrupts

provided  that there is something else for the processor to do (which is NOT

the case in most protocols).

Recall  that  PCL4PB2.LIB  is the PCL4PB library with transmitter interrupts

enabled.

PCL4PB Users Manual                                               Page 21

6.6 RS-232 Signals

RS-232 is the name of the serial data interface  standard  used  to  connect

computers  to modems.  Most IBM compatible computers are built with at least

one serial port and use either DB9 (9 pin) or DB25 (25 pin) connectors.

A summary of these pins and  their  function  follows.   For  more  detailed

information, refer to one of the many books dealing with RS-232 interfacing.

Signal Ground Pin 7 (DB25), Pin 5 (DB9)

The SG line is used  as  the  common  signal  ground,  and  must  always  be

connected.

Transmit Data Pin 2 (DB25), Pin 3 (DB9)

The TX line is used to carry data from the computer to the modem.

Receive Data Pin 3 (DB25), Pin 2 (DB9)

The RX line is used to carry data from the modem to the computer.

Data Terminal Ready Pin 20 (DB25), Pin 4 (DB9)

The  DTR  line is used by the computer to signal the modem that it is ready.

DTR should be set high when talking to a modem.

Data Set Ready Pin 6 (DB25), Pin 6 (DB9)

The DSR line is used by the modem to signal the computer that it is ready.

Request to Send Pin 4 (DB25), Pin 7 (DB9)

The RTS line is used to "turn the line around" in half  duplex  modems,  and

for hardware flow control in most modems that require flow control.

Clear to Send Pin 5 (DB25), Pin 8 (DB9)

The  CTS  line  is used to "turn the line around" in half duplex modems, and

for hardware flow control in most modems that require flow control.

Data Carrier Detect Pin 8 (DB25), Pin 1 (DB9)

The DCD line is used by the modem to signal the computer that a data carrier

signal is present.

Ring Indicator Pin 22 (DB25), Pin 9 (DB9)

The RI line is asserted when a 'ring' occurs.

PCL4PB Users Manual                                               Page 22

6.7 National INS8250, INS16450, and INS16550 UARTs

The  Personal  Communications  Library  is  based  on  the standard National

INS8250, INS16450, and INS16550 UARTs. The 8250 was the original  UART  used

in  the IBM PC, whereas the 16450 is a faster version found on most 286 & up

machines. The 16550 contains a 16 byte FIFO to further reduce communications

overhead. These UARTs consists of 8 register ports as follows:

Offset    R/W   Register

0       R/W   Receiver (read) / Transmitter (write)

1       R/W   Interrupt Enable (read)

2       R     Interrupt Identification

2       W     FIFO control (INS16550 only)

3       R/W   Data Format (Line Control)

4       R/W   RS-232 (Modem) Control

5       R/W   Line Status

6       R/W   RS-232 (Modem) Status

7       R/W   Not used.

For the standard PC ports (not DigiBoard or BOCA ports), the UART  registers

are  based  at 3F8h (COM1), 2F8h (COM2), 3E8h (COM3), and 2E8h (COM4).  COM1

and COM3 share interrupt request line IRQ4 while COM2 and COM4 share request

line IRQ3.  This means that  COM1  and  COM3  can't  be  used  concurrently.

Similarly for COM2 and COM4.

If  you  have a DigiBoard (or BOCA board) installed, you will have 4 or more

additional ports using INS16450 or INS16550 UARTS.   The  default  DigiBoard

and  BOCA  board  ports are located at 100h, 108h, 110h, etc.  Refer to your

DigiBoard (or BOCA board) manual.

Four sources of interrupts  are  possible  with  the  8250  and  16550:  (1)

receiver error or BREAK, (2) receiver data ready, (3) ready to transmit, and

(4)  RS232  input.   These  four  sources  of  interrupts  are summarized as

follows:

Source of Interrupt        Action Required to Clear

Receiver error or BREAK.   Read Line Status register.

Receiver data.             Read data from data register.

Transmitter Buffer Empty.  Write to data register or read IID reg.

RS232 input.               Read Modem Status register.

However, PCL4PB only enables the receiving data interrupt. This  means  that

interrupts can only be caused by incoming data.

If  you are not familiar with the INS8250, several good books are available.

Refer to Sectionb 9.3, "Further Reading" for  recommendations.   Although  a

knowledge of the 8250 is not necessary to use PCL4PB, a general knowledge of

the theory of asynchronous serial communications is recommended.

PCL4PB Users Manual                                               Page 23

6.8 Register Summary

REG 0 : Data Register

Reading  from  the  data  register  fetches  the next input byte, once it is

ready.  Writing to the data register transmits the byte written to  it  over

the serial line.

REG 1 : Interrupt Enable

The  Interrupt Enable register enables each of four types of interrupts when

the appropriate bit is set to a one.

bit 3 : Enable interrupt on RS232 input.

bit 2 : Enable interrupt on receiver error or break.

bit 1 : Enable interrupt on transmitter buffer empty (TBE).

bit 0 : Enable interrupt on received data (RxRDY).

REG 2 : Interrupt Identification (IID)

Reading  the Interrupt Identification (read only) register once an interrupt

has occurred identifies the interrupt as follows:

Bit 2  Bit 1  Bit 0  Priority    Interrupt

0      0      1      none      none

1      1      0      0 (high)  Serialization or break.

1      0      0      1         Received data.

0      1      0      2         Transmitter Buffer Empty.

0      0      0      3 (low)   RS232 Input.

In the INS16650, REG 2 (write only)  is  also  the  FIFO  control  register.

Writing bits 6 & 7 will set the FIFO trigger level (number of bytes received

before an interrupt is generated).

Bit 7  Bit 6   Trigger             Bit 7  Bit 6   Trigger

0      0      1 byte               1      0      8 bytes

0      1      4 bytes              1      1      14 bytes

REG 3 : Line Control

RS232 line parameters are selected by writing to this register.

bit 7   : DLAB = 0

bit 6   : BREAK on(1), off(0).

bits 5-3: Parity None(000),ODD(001),EVEN(011),MARK(101),SPACE(111)

bit 2   : One stop bit(0), two stop bits(1).

bits 1-0: Data bits = 5 (00), 6(01), 7(10), 8(11).

When  the Divisor Latch Access Bit (DLAB) is 1, registers 0 and 1 become the

LS and MS bytes of the Baud Rate Divisor registers.

Baud   Divisor      Baud  Divisor      Baud  Divisor

300    0180        4800   0018       38400   0003

1200    0060        9600   000C       57600   0002

2400    0030       19200   0006      115200   0001

PCL4PB Users Manual                                               Page 24

REG 4 : Modem Control

RTS, DTR, loopback testing, and  General  Purpose  Outputs  #1  and  #2  are

controlled by the Modem Control register as follows:

bit 4 : Enable local loopback.

bit 3 : Enable GP02. Necessary for 8250 interrupts.

bit 2 : Enable GP01.

bit 1 : Set / clear RTS.

bit 0 : Set / clear DTR.

REG 5 : Line Status

Reading  the  Line Status register provides status information as follows (1

for TRUE, 0 for FALSE) :

bit 6 : Transmitter Empty.

bit 5 : Transmitter Buffer Empty (TBE).

bit 4 : BREAK detect.

bit 3 : Framing error.

bit 2 : Parity error.

bit 1 : Overrun error.

bit 0 : Data Ready.

REG 6 : Modem Status

Reading the Modem Status register provides the following status  information

(1 for TRUE, 0 for FALSE) :

bit 7 : DCD status.

bit 6 : RI status.

bit 5 : DSR status.

bit 4 : CTS status.

bit 3 : Delta DCD status.

bit 2 : Delta RI status.

bit 1 : Delta DSR status.

bit 0 : Delta CTS status.

The  delta  bits  (bits 0 through 3) are set whenever one of the status bits

(bits 4 through 7) changes (from 0 to 1 or from 1 to 0) since the last  time

that  the  Modem Status register was read. Reading the Modem Status register

clear the delta bits.

REG 7 : Scratch Register

There is no function associated with register 7.  It does not exist in early

versions of the 8250.

PCL4PB Users Manual                                               Page 25

7.0 Example Programs

Five  example  programs  are include with PCL4BP.  In addition, the Personal

Protocol Library for C (PPL4PB) includes the  terminal  program  TERM  which

features  ASCII,  XMODEM,  YMODEM,  and ZMODEM protocol transfers.  Complete

source code is included in the shareware product for all  of  the  protocols

above  except  ZMODEM.   In  order  to  get  ZMODEM  source,  PPL4PB must be

registered.

7.1 MINIMAL

MINIMAL is the simpliest possible communications program. It reads from  the

serial port & displays on the screen and reads from the keyboard & sends out

over the serial line. COM1 and 9600 baud are hard coded for simplicity.

7.2 SIMPLE

SIMPLE  is  a simple terminal program. It operates like MINIMAL, except that

you specify both a port and a baud rate. For example.

SIMPLE 1 9600

7.3 LOGIN

LOGIN is programmed to dial our support BBS (205-880-9748)  and  log  on  as

GUEST.  Start LOGIN like SIMPLE by providing a COM port and a baud rate. For

example,

LOGIN 1 38400

7.4 DOOR

The  DOOR program is a simple communications program which can "take over" a

serial port.  For example, say you  are  running  PROCOMM,  PCPLUS,  or  any

communications  program  with  a  DOS  gateway.  Select the DOS gateway from

PROCOMM (or whatever) and then type

DOOR 1

at the DOS prompt to take over port COM1.

7.5 SELFTEST

The SELFTEST.C program is designed to test your serial ports  provided  that

you  have two ports which can be connected together with a null modem cable.

SELFTEST can also be used to test your multiport board.

For example, to test PC port COM1 against COM2, type:

SELFTEST PC 1 2

SELFTEST may need to  be  configured  for  non-standard  PC  ports  or  your

multiport board. Refer to the SELFTEST source code for more information.

PCL4PB Users Manual                                               Page 26

8.0 Legal Issues

8.1 Registration

If  you wish to register the PCL4PB library, please send $65 plus $3 S&H ($6

outside of North America) to:

MarshallSoft Computing, Inc.

Post Office Box  4543

Huntsville AL 35815

Multiple copies are available: $50 for 3 to 9, $35 for 10 to 19, and $25 for

20 or more. A site license is also available for $495 (includes  5  sets  of

printed documentation). We pay shipping.

We  accept  American Express (account number, expiration date, exact name on

your card, and complete AmEx billing address required), checks in US dollars

drawn on a US bank, purchase orders (POs) from  recognized  US  schools  and

companies  listed  in  Dun  &  Bradstreet, and COD (street address and phone

number required) within the USA (plus a $3  COD  charge).

You  can  also order PCL4PB from The Public Software Library (PSL) with your

MC, Visa, AmEx, or Discover card by  calling  800-242-4PSL  (from  overseas:

713-524-6394)  or  by  FAX  at 713-524-6398 or by CompuServe at [71355,470].

THESE NUMBERS ARE FOR ORDERING ONLY. The product number for PCL4PB is 11499.

Be  sure  to  specify PowerBASIC. Have your shipping address and credit card

billing address ready.

If you wish to update from an older version of PCL4PB, send $20 plus $3  S&H

($6  outside  of  North  America).   Updates  must  be ordered directly from

MarshallSoft Computing.

The registered package includes:

o  Assembler source code for the library.

o  Laser printed Users and Reference Manuals.

o  Telephone, FAX, and BBS support for one year.

o  Utility program (EXAMPORT) which displays  a  detailed  formatted  report

for any serial port.

Print the file INVOICE.DOC if an invoice is needed. The registered user will

receive the latest version of  PCL4PB  shipped  by  two  day  priority  mail

(packet  airmail  overseas).   A  3.5"  diskette  is provided unless a 5.25"

diskette is requested.

PCL4PB Users Manual                                               Page 27

8.2 License

MarshallSoft Computing, Inc. grants the registered user of PCL4PB the  right

to  use one copy of the PCL4PB library (in object form) on a single computer

in the development of any software product (other  than  libraries  such  as

PCL4PB).   The user may not use the library on more than one computer at the

same time.  The source code for the library (PCL4PB.ASM) is  copyrighted  by

MarshallSoft Computing and may not be released in whole or in part.

Products  developed  using PCL4PB can include the object form of the library

and may be distributed without any royalty.

8.3 Warranty

MARSHALLSOFT COMPUTING, INC.  DISCLAIMS  ALL  WARRANTIES  RELATING  TO  THIS

SOFTWARE,  WHETHER  EXPRESSED  OR  IMPLIED, INCLUDING BUT NOT LIMITED TO ANY

IMPLIED WARRANTIES OF MERCHANTABILITY AND FITNESS FOR A PARTICULAR  PURPOSE,

AND  ALL  SUCH WARRANTIES ARE EXPRESSLY AND SPECIFICALLY DISCLAIMED. NEITHER

MARSHALLSOFT COMPUTING, INC. NOR ANYONE ELSE WHO HAS BEEN  INVOLVED  IN  THE

CREATION,  PRODUCTION,  OR DELIVERY OF THIS SOFTWARE SHALL BE LIABLE FOR ANY

INDIRECT, CONSEQUENTIAL, OR INCIDENTAL DAMAGES ARISING OUT  OF  THE  USE  OR

INABILITY  TO  USE  SUCH  SOFTWARE EVEN IF MARSHALLSOFT COMPUTING, INC.  HAS

BEEN ADVISED OF THE POSSIBILITY OF SUCH DAMAGES OR CLAIMS. IN NO EVENT SHALL

MARSHALLSOFT COMPUTING, INC.'S LIABILITY FOR ANY SUCH  DAMAGES  EVER  EXCEED

THE  PRICE  PAID FOR THE LICENSE TO USE THE SOFTWARE, REGARDLESS OF THE FORM

OF THE CLAIM. THE PERSON USING THE SOFTWARE BEARS ALL RISK AS TO THE QUALITY

AND PERFORMANCE OF THE SOFTWARE.

Some states do not allow  the  exclusion  of  the  limit  of  liability  for

consequential  or  incidental damages, so the above limitation may not apply

to you.

This agreement shall be governed by the laws of the  State  of  Alabama  and

shall  inure  to  the  benefit  of  Marshallsoft  Computing,  Inc.   and any

successors, administrators, heirs and assigns.   Any  action  or  proceeding

brought  by either party against the other arising out of or related to this

agreement shall be brought only in a STATE or  FEDERAL  COURT  of  competent

jurisdiction  located in Madison County, Alabama. The parties hereby consent

to in personam jurisdiction of said courts.

PCL4PB Users Manual                                               Page 28

9.0 Summary

9.1 Revision History

Version 4.2 - 1 Sept 1994

This is the first release of the BASIC language version of our library.  The

assembler language library is identical to that used by C/C++, version  4.2.

Only  the high level code (BASIC) had to be re-coded from the original C/C++

version.

Version 4.3 - 25 April 1995

o  BREAK detection bug fixed.

o  Port definitions extended to COM20.

o  Support for IRQ8 through IRQ15.

o  Line status bits preserved.

[The TERM program moved to PPL4PB]

9.2 Function Summary

Refer to the PCL4PB Reference Manual (PCL4PB.REF) for detailed information  on

the  communications  and  support  functions.  A  one  line  summary of each

function follows:

SioBaud     Sets the baud rate of the selected port.

SioBrkKey   Returns non-zero if the Control-BREAK key was pressed.

SioBrkSig   Asserts, cancels, or detects BREAK signal.

SioCTS      Reads the Clear to Send (CTS) modem status bit.

SioDCD      Reads the Data Carrier Detect (DCD) modem status bit.

SioDelay    Delays one or more tics (18 tics per second).

SioDone     Terminates further serial processing.

SioDSR      Reads the Data Set Ready (DSR) modem status bit.

SioDTR      Set, clear, or read the Data Terminal Ready (DTR) bit.

SioError    Displays error in text.

SioFIFO     Sets the interrupt level for the INS16550.

SioFlow     Enables / disables hardware flow control.

SioGetc     Reads the next character from the serial line.

SioGetDiv   Reads the baud rate divisor registers.

SioInfo     Returns library version number & memory model.

SioIRQ      Assigns an IRQ line to a port.

SioLine     Reads the line status register.

SioLoopBack Performs a UART loopback test.

SioModem    Reads the modem status register.

SioParms    Sets parity, stop bits, and word length.

SioPorts    Sets # ports, 1st DigiBoard / BOCA port & status reg.

SioPutc     Transmit a character over a serial line.

SioRead     Reads any of 7 UART ports.

SioReset    Initialize a serial port for processing.

SioRI       Reads the Ring Indicator (RI) modem status bit.

SioRTS      Sets, clears, or reads the Request to Send (RTS) line.

SioRxBuf    Sets up receive buffer.

SioRxFlush  Flushes (clears) the receive buffer.

SioRxQue    Returns the number of characters in the receive queue.

SioTimer    Returns the number of system clock tics.

SioTxBuf    Sets up transmit buffer.

SioTxFlush  Flushes (clears) the transmit buffer.

SioTxQue    Returns the number of characters in the transmit queue.

SioUART     Sets the UART base address.

SioUnGetc   "Un-gets" (puts back) a specified character.

PCL4PB Users Manual                                               Page 29

9.3 Further Reading

The best way to learn about serial communications is to read a good book  on

the subject. Several good texts are available.  Two that I like are:

(1) C  Programmers's  Guide to Serial Communications by Joe Campbell (SAMS)

(2) Mastering Serial Communications by Peter Gofton (SYBEX).

10.0 Other MarshallSoft Computing Products for PowerBASIC

10.1 The Personal Protocol Library for PowerBASIC

The Personal Protocol Library for PowerBASIC  (PPL4PB)  consists  of  a  DOS

library  which  implements  XMODEM, XMODEM-CRC, XMODEM-1K, XMODEM-G, YMODEM,

YMODEM-G, and  ZMODEM  file  transfer  protocols.   A  script  compiler  and

interpreter   is   also   included   which  is  capable  of  such  tasks  as

automatically logging onto a BBS and downloading a file or retrieving  mail.

Three example script programs are included.

The  protocol  library (PPL4PB) requires the Personal Communications Library

for PowerBASIC (PCL4PB). It is available for $40 plus $3 S&H.

PCL4PB Users Manual                                               Page 30