CDX2APR   Version 1.0

Copyright (C) GraphiCode Inc 1993,1994  All Rights Reserved.

This converter will convert from a CADIX .APT aperture file to a

GraphiCode .APR aperture file.  The converter can be used to convert single

files or groups of files.

usage: CDX2APR <aperture file> [<outfile>]

Example:

To convert the aperture file "ALIST1.APT" for use with GraphiCode

products, type:

CDX2APR ALIST1

This creates the GraphiCode aperture wheel file "ALIST1.APR".

To convert "ALIST1.APT" to "NEWALIST.APR" type:

CDX2APR ALIST1 NEWALIST

To convert several aperture files type:

CDX2APR *.APT

This is the format of the CADIX file supported by this converter:

;                ***************************************

;                *                                     *

;                *     Photo Program Table  SAMPLE     *

;                *            (Ver 3.0.0)              *

;                *                                     *

;                ***************************************   1992.09.03

;

;     [ General Definitions ]

;                                 ;   [] : Default

Warning_Error_Stop = NO           ; { YES | [ NO ] }

Fill_Mode          = F            ; { [ F ] (Fill) | O (Ouline) }

Layers             = 0:255        ; Output Layers ( 0-255 )

Sub_Layers         = 0:255        ; Output Sublayers ( 0-255 )

Origin_Mode        = WINDOW       ; { [ WINDOW ] | DATABASE }

Format             = GERBER       ; { [ GERBER ] | MUTOH }

Window_Unit        = DATABASE     ; { [ DATABASE ] | USER }

Use_Sort           = YES          ; { YES | [ NO ] }

Overlap            = 10            ; ( [ 10 ] micron )

Tolerance          = 0 0          ; ( [ 0 0 ] micron )

Del_Intersection   = NO           ; { YES | [ NO ] }

Fill_Direction     = A            ; { [ A (AUTO) ] | H (HORIZONTAL) | V (VERTICAL) }

;     [ Aperture Definitions ]   ( MAX = 256 )

;

;         APT-No. : Shape :  Diameter  : D-Code : Cellname

Aperture =   1      ROUND        50       D10

Aperture =   2      ROUND        55       D11

Aperture =   3      ROUND        60       D12

Aperture =   4      ROUND        65       D13

Aperture =   5      ROUND        70       D14

Aperture =   6      ROUND        75       D15

Aperture =   7      ROUND        80       D16

Aperture =   8      ROUND        85       D17

Any comments about this converter may be directed to THE SYSOP of the

GraphiCode BBS (206)771-6980.