(User.programs)

Item: 377 by akcs.billb@hpcvbbs.cv.hp.com

Author: [William R. Brassfield]

Subj: Great Circle Distance and Direction Find

Keyw: latitude, longitude, distance, direction

Date: Mon Mar 30 1992

To use this program, press the "GCIRC" menu option, and you will be

prompted for latitude and longitude data, for locations 1 and 2.  Enter

them in either degrees/decimal_degrees, or degrees/minutes/decimal_minutes,

or degrees/minutes/seconds (you must be consistent, however).  After this,

press the "GO" menu option, and you will be prompted for the format of your

latitude/longitude data.  Select and press the appropriate menu item.  The

great circle distance and INITIAL heading will then be displayed.  (Press

the VAR button to return to the normal stack display (or the ON button, but

the VAR button is preferred).  For two locations with the same latitude but

different longitudes, the direction will be either 90 degrees or 270

degrees, ONLY AT THE EQUATOR LATITUDE.  At higher (northern) latitudes, the

direction will be 90- or 270+; at lower (southern) latitudes, the direction

will be 90+ or 270-, depending on whether you want to travel east or west.

This is perfectly normal because only the equator latitude line represents

a "great circle" path.

This program can be quite useful for the pilot planning a cross-country

flight.  It saves a lot of time, especially when the departure and

destination locations are on different sectional charts.

It should be noted that this program does not account for the equatorial

"bulge" of the earth, which will cause some error (but probably not enough

to get worried about).  In this program, the earth is assumed to be a

perfect sphere.

Another note:  In a super-long-distance cross-country flight, a

great-circle path does not have a constant heading!  For example, if you

are on the 45 degree north latitude line, and you want to travel 2000 miles

east (destination also on 45 degree latitude line), then your initial

direction (no-wind heading) will be 79.67 degrees; at mid-point, your

direction will be 90.00 degrees, and as you arrive at your destination,

your direction will be 100.33 degrees.  This does not mean that you will be

turning the plane; in fact, you will be flying as straight a path as

possible for the given curvature of the earth.  Your reference to true

north, however, does change, and you will need to periodically check your

magnetic compass and magnetic variation on your sectional chart to keep

yourself on track.

Enough said about the program and how to use it.  Enjoy!

Oops!  Forgot to mention -- it's a directory called "GCIRCLE".  In it you

will find { GCIRCLE DEGS DMIN DMS LAT1 LON1 LAT2 LON2 HM-> MILES HEAD MESG

LA1 LO1 LA2 LO2 PN GO }

Northern latitudes are positive; southern ones are negative.  Western

longitudes are positive; eastern ones are negative.  All continental U.S.

latitudes and longitudes are, therefore, positive.